3GPP TSG-RAN WG4 Meeting #110	R4-2402450

# Athens, Greece, February 26 – March 1, 2024

Source: 	T-Mobile USA

Title:	TP for TR 38.880: Input for CA_n71A-n77A with 3Tx

Agenda Item:	7.25.2

Document for:	Approval

- Introduction

This contribution provides a text proposal for CA_n71A-n77A 3Tx with 2Tx on n77 for TR 38.880.

# Text Proposal

<< Beginning of Text Proposal >>

## 5.x	CA_n71A-n77A

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n71+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n71-n77 | n71, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.5.3-1 and 5.5.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n71-n77.

Table 5.5.3-1: MSD scenarios for CA_n71-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic Mixing |  |  | No |
| IMD | n71+n77 | n71 | Yes |
| Cross band leakage | n77 UL | n71 DL | Yes |

Table 5.5.3-2: MSD scenarios for CA_n71-n77 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic Mixing |  |  | No |
| IMD | n71+n77 | n71 | No |
| Cross band leakage | n77 UL | n71 DL | Yes |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

The PC2 IMD MSD can be re-used from 2Tx.

| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

The PC2 MSD due to cross band isolation can be re-used from PC2 with 2Tx and therefore is not shown here.

#### 5.x.4.2	REFSENS requirements for total power class 1.5

There is no PC1.5 MSD requirement for CA_n71-n77A because PC1.5 requires 2Tx in the TDD band currently. We propose increasing the PC2 MSD by adding 3dB * IMD order or 3*5=15 dB.

| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 26.4 | FDD | IMD57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

The PC1.5 MSD due to cross band isolation can be re-used from PC1.5 with 2Tx and therefore is not shown here.

<< End of Text Proposal >