3GPP TSG-RAN WG4 Meeting #110	R4-2402464

# Athens, Greece, February 26 – March 1, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n25 with DL CA_n25(2A)

Agenda item:	7.19.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n25 for DL CA_n25(2A).

In other PC2 and PC1.5 MSD analysis, T-Mobile USA has been using calculations that increase the Interference by 3dB, but not the noise. This contribution proposes new MSD values based on this approach for single Tx.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n25(2A)

### 5.x.1	UE maximum output power

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | n253 | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  | 60 | 1 |
|  |  | See n25 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 60 | 4 and 5 |
| NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

For PC2, CA_n25(2a) has self-interference for UL n25. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n71 without TxD

For CA_n25(2A), this is the configuration and MSD for UL n25 with PC3

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) |  | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 |  | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Using that approach, the following is proposed as a the PC2 MSD which would require a new table in 38.101-1:

Table 7.3A.2.2-1a: Intra-band non-contiguous CA with one PC2 uplink configuration for reference sensitivity in FDD bands without Transmit Diversity.

| CA configuration | SCS (PCC/SCC) (kHz) |  | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 |  | 5MHz + 5MHz | Wgap = 55.0 | 105 | 7.3 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [27.6] 8 | FDD |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n25 with TxD

[TBD]