3GPP TSG-RAN WG4 Meeting # 110	R4-2402492

Athens, Greece, 26 Feb  - 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on active downlink TCI state switching delay for unified TCI for sDCI mTRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-2-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Fix the requirements of FR2 when SSB are adjacent for unknown case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the requiements of FR2 when SSB are adjacent for unknown case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of Active downlink TCI state switching delay for unified TCI for sDCI mTRP will be incorrect of the case when SSB are adjacent of FR2 for unknown case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==========================Start of change 1 =============================

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP

8.21.1	Introduction

The requirements in this clause apply for a UE configured with DLorJoint-TCIState configurations for DL channels on a serving cell and UE reports its capability of S-DCI based MTRP. Further the requirements also apply for all the list of serving cells in simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR provided all serving cells in the list are configured with the same serving cell and BWP in unifiedTCI-StateRef-r17. UE shall complete the switch of active downlink TCI state within the delay defined in this clause.

8.21.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

-	The SSB can be associated with the serving cell PCI.

Otherwise, the downlink TCI state is unknown.

8.21.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

When a MAC CE for DL TCI state switch indicates single TCI state switch, requirements specified in clause 8.15.3 apply.

When a MAC CE for DL TCI state switch indicates dual TCI state switch,

- If both the target TCI states are known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ max{TOk1*(Tfirst-SSB1 + AD1*TSSB1  + TSSB-proc), TOk2*(Tfirst-SSB2 + AD2*TSSB2 + TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state; Tfirst-SSB2 is time to first SSB transmission of second TCI states of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk1 = 1 if first target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	TOk2 = 1 if second target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	AD1 = 1 if SSBs are adjacent in FR2 and TSSB1 =TSSB2 ; 0 otherwise

-	AD2 = 1 if SSBs are adjacent in FR2 and TSSB2 = TSSB1 ; 0 otherwise

Among the dual target TCI states indicated for switch, if the one of the  target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot THARQ + 3Nslotsubframe,µ + TL1-RSRP1 + max {TOuk1*(Tfirst-SSB1+ AD1*TSSB1 + TSSB-proc), TOk2*(Tfirst-SSB2 + TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

-	AD1 = 1 if SSBs are adjacent in FR2; 0 otherwise

If the both of the dual target TCI state are unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + max{TL1-RSRP1 +TOuk1*(Tfirst-SSB1+ TSSB-proc), TOk2*(Tfirst-SSB2+ TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP1 = 0 for the first TCI state and T L1-RSRP2 = 0 for the second TCI state in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP1 for the first TCI state and T L1-RSRP2 for the second TCI state are the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk1 for the first TCI state and TOuk2 for the second TCI state  TOuk1 = 1 and TOuk2 = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk1 = 1 and TOuk2 = 1when TCI state switching involves other QCL types only

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD; Tfirst-SSB2 is time to second SSB transmission of first TCI states of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types; Tfirst-SSB2 is time to second SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

For FR2 both of the dual target TCI state are unknown when SSB are adjacent, Longer delay is expected.

==========================End of change 1 =============================