3GPP TSG-RAN4 Meeting #110	 R4-2402497

# Athens, Greece, Feb 26 –March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | TBD | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-5 on DMRS bundling for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-2-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NTN Ka-band, the DMRS bundling requriment needs to be added in Rel-18 according agreed new UE capability [maxDurationDMRS-Bundling-NTN-GSO-r18] and maxDurationDMRS-Bundling-NTN-NGSO-r18. Requirement in 38.101-2 cannot be directly referenced as the original requriement only for TDD bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the requirement with the new UE capaiblity and referencing to the FDD DMRS bundling requriement table in 38.101-1, the test conditions are retained as the same as 38.101-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN DMRS bundling requirements for NTN Ka-band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

### 9.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-2 [x] clause 6.4.2 except caluse 6.4.2.6 shall apply for VSAT UE.

#### 9.4.2.1	Phase continuity requirements for DMRS bundling

For bands that UE indicates the support of DMRS bundling, when the UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 in clause 6.4.2.5 in TS 38.101-1 [5] for the measurement conditions defined in Table 6.4.2.6-2 in clause 6.4.2.6 in TS 38.101-2 [x], within a measurement time window limited by the UE capability of maximum duration for NGSO when IE [maxDurationDMRS-Bundling-NTN-NGSO-r18] is present and for GSO when IE [maxDurationDMRS-Bundling-NTN-GSO-r18] is present, and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.8 in TS 38.101-2. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms. The additional test condition for doppler and delay condition shall be set according to A.4.

The above requirements are applicable when all the following conditions are met within the measurement time window.

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	There is no change in UE EIRP level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions.

-	No uplink beam switching occurs.

NOTE: Phase continuity requirements for DMRS bundling is defined only within FR2-1 in this release of the specification.

------------------------------------------------------------- END CHANGE ------------------------------------------------------