3GPP TSG-RAN4 Meeting #110	 R4-2402500

# Athens, Greece, Feb 26 –March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | TBD | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.176-2 for mIAB RF conformance testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Perf |  |  |  |  |  | Date: |  |  | 2024-2-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | new RF requriements defined for mIAB in 38.174 and test cases needs to be updated accordlingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Tx dyanmic range and output power test for mIAB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | mIAB conformancing test is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.4.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.176-1/2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

### 6.3.2	Minimum requirement

The minimum requirement for IAB-DU type 1-O and IAB-MT type 1-O is in TS 38.174 [2], clause 6.3.2.

The minimum requirement for IAB type 2-O is in TS 38.174 [2], clause 6.3.3.

The minimum requirement for mIAB type 1-O and type 2-O is in TS 38.174 [2], clause 9.3.1B.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.4.2.1.5	Test requirements

6.4.2.1.5.1	IAB type 1-O

For IAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.1-1:

Table 6.4.2.1.5.1-1: IAB type 1-0 Output power dynamics test requirements.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement (Note 1) |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) -1.2 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) -1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.8 < ΔP ≤ 10 log(Maximum RB) + 15.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |

For mIAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.1-2, Table 6.4.2.1.5.1-3 and Table 6.4.2.1.5.1-4 :

Table 6.4.2.1.5.1-2: mIAB type 1-O Output power dynamics test requirements for band n41

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Local area | ≤40MHz | -10 log(Maximum RB)- 45- ([6.5]+ 0.7)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 0.7) |
|  | 40MHz < BW ≤ 100MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1) |

Table 6.4.2.1.5.1-3: mIAB type 1-O Output power dynamics test requirements for band n77 and n78

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Local area | ≤40MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1) |
|  | 40MHz < BW ≤ 100MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1.6)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1.6) |

Table 6.4.2.1.5.1-4: mIAB type 1-O Output power dynamics test requirements for band n79

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Local area | BW≤20MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1.3)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1.3) |
|  | 20MHz<BW≤40MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1.5)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1.5) |
|  | 40MHz < BW ≤ 100MHz | -10 log(Maximum RB)- 45- ([6.5]+ 1.6)< ΔP<-10 log(Maximum RB)- 45+ ([6.5]+ 1.6) |

6.4.2.1.5.2	IAB type 2-O

For IAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.2-1:

Table 6.4.2.1.5.2-1: IAB type 2-0 Output power dynamics test requirements.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) -1.2 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) -1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.8 < ΔP ≤ 10 log(Maximum RB) + 15.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |

For mIAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.2-2:

Table 6.4.2.1.5.2-1: IAB type 2-O Output power dynamics test requirements.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Local area | ≤40MHz | -10 log(Maximum RB)- 30- ([6.5]+ 0.7)< ΔP<-10 log(Maximum RB)- 30+ ([6.5]+ 0.7) |
|  | 40MHz < BW ≤ 100MHz | -10 log(Maximum RB)- 30- ([6.5]+ 1.0)< ΔP<-10 log(Maximum RB)- 30+ ([6.5]+1.0) |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------