1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for NR repeater type 1-C. These have been derived from, and are consistent with the conducted requirements for repeater type 1-C in NR repeater specification defined in TS 38.106 [2].

A repeater type 1-C only has conducted requirements so it requires compliance to this specification only.

Arepeater type 2-O has only radiated requirements so it requires compliance to TS 38.115-2 [3] only.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.106: "NR; Repeater Radio Transmission and Reception"

[3]	3GPP TS 38.115-2: "NR; Repeater conformance testing, Part 2: Radiated conformance testing"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[6]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[7]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing, Part 1: Conducted conformance testing"

[8]	3GPP TS 38.211: "NR; Physical channels and modulation"

[9]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[11]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”

[12]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters"

[13]	Federal Communications Commission: "Title 47 of the Code of Federal Regulations (CFR) "

[14]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[15]	3GPP TR 25.942: "RF system scenarios"

[16]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[17]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[18]	IEC 60 721: "Classification of environmental conditions"

[19]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[20]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[21]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Antenna connector: connector at the conducted interface of the repeater type 1-C

Fractional bandwidth: fractional bandwidth FBW is defined as

gap between passbands: frequency gap between two consecutive passbands that belong to the same operating band, where the RF requirements in the gap are based on co-existence for un-coordinated operation

Inter-passband gap: The frequency gap between two supported consecutive passbands that belong to different operating bands.

Maximum passband output power: mean power level measured per passband at the antenna connector, during the transmitter ON state in a specified reference condition.

multi-band connector: Antenna Connector for a Multi-band repeater.

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Nominal channel bandwidth: Bandwidth calculated as min(100MHz, BWpassband) in FR1 or min(400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

Non-contiguous spectrum: spectrum consisting of two or more passbands separated by inter-passband gap(s).

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

passband edge: Frequency at the edge of the passband

Repeater type 1-C: Repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

Requirement set: one of the NR requirements set as defined for NR repeater

single-band connector: Repeater type 1-C antenna connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the repeater.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction.

Transmitter ON state: Time period during which the repeater is transmitting downlink or uplink signals in the corresponding direction.

Transmitter transient period: Time period during which the repeater is changing from the OFF state to the ON state or vice versa.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWNominal	Nominal channel bandwidth

BWPassband	Passband bandwidth

f	Separation between the passband edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper passband edge

Foffset,low	Frequency offset from FC,low to the lower passband edge

f_offset	Separation between the passband edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the operating band

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

PEM,n54,ind	Declared emission level for Band n54 in the band 1518-1559 MHz; ind = a, b, c, d, e, fPmax,p,AC	Maximum passband output power measured per antenna connector

Prated,in	Rated pass band input power to the repeater for the test per antenna connector

Prated,p,AC	Rated passband output power per antenna connector

Prated,t,AC	Rated total output power declared per antenna connectorWgap	Inter passband Bandwidth gap size

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

BW	Bandwidth

CACLR	Cumulative ACLR

CP-OFDM	Cyclic Prefix-OFDM

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

UL	Uplink

WA	Wide Area

4	General conducted test conditions and declarations

4.1	Measurement uncertainties and test requirements

4.1.1	General

4.1.2	Acceptable uncertainty of Test System

4.1.2.1	General

4.1.2.2	Conducted characteristics measurements

Table 4.1.2.2-1: Maximum Test System uncertainty for conducted characteristics tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Repeater output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |  |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 4.2 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |  |
| 6.5.2 ACLR/ CACLR | BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.5.3 Operating band unwanted emission | 1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ 4.2 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.5.4 Spurious emissions | In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB 4 GHz < f  19 GHz ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.6 EVM | 1% signal analyser 2% stimulus signal |  |
| 6.7 Input intermodulation | ±1,2 dB | Formula: RSS CW1 level error, 2 x CW2 level error, and measurement error (using all errors = ±0,5 dB) |
| 6.8 Output intermodulation | For operating band unwanted emission: 2,1 dB The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. For spurious emission: In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB; 4GHz < f 19 GHz ±4,0 dB. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required in 6.5.4 and 6.5.5. | Formula: RSS 2x Interference signal level error and operating band unwanted emission measurement level error. (1 dB interference signal level error is assumed). |
| 6.9 ACRR | 0,7 dB |  |
| 6.10.1 Transmit ON/OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.10.1 Transmit OFF power | N/A |  |

Table 4.1.2.2-2: Maximum Test System uncertainty for conducted characteristics tests for NCR

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Repeater output power |  |  |
| 6.3 Frequency stability |  |  |
| 6.4 Out of band gain |  |  |
| 6.5.2 ACLR/ CACLR |  |  |
| 6.5.3 Operating band unwanted emission |  |  |
| 6.5.4 Spurious emissions |  |  |
| 6.6 EVM |  |  |
| 6.7 Input intermodulation |  |  |
| 6.8 Output intermodulation |  |  |
| 6.9 ACRR |  |  |
| 6.10.1 Transmit ON/OFF power |  |  |
| 6.10.1 Transmit OFF power | N/A |  |

4.1.3	Interpretation of measurement results

4.2	Conducted requirement reference points

For repeater type 1-C, the requirements are applied at the repeater antenna connector (BS-side connector or UE-side connector) for downlink or uplink for the configuration in normal operating conditions.

Figure 4.2-1: Repeater type 1-C downlink and uplink interface

4.2A	Network controlled Repeater type 1-C

For NCR type 1-C, the NCR-Fwd RF requirements are applied at the NCR antenna connector (BS-side connector or UE-side connector) for downlink or uplink for the configuration in normal operating conditions.

For NCR type 1-C, the NCR-MT RF requirements are applied at the NCR antenna connector (BS-side connector) for the configuration in normal operating conditions.

Figure 4.2A-1: Network controlled Repeater type 1-C downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate antenna connectors.

4.2B	Network controlled Repeater type 1-H

For NCR type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

For NCR type 1-H, the NCR-Fwd conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector or UE-side TAB connector) for downlink or uplink for the configuration in normal operating conditions.

For NCR type 1-H, the NCR-MT conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector) for the configuration in normal operating conditions.

Figure 4.2B-1: Network controlled Repeater type 1-H downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate TAB connectors.

4.3	Repeater classes

4.3.1	Repeater class for downlink

4.3.2	Repeater class for uplink

4.3A	Network controlled repeater classes

4.3A.1	Network controlled repeater class for downlink

The requirements in this specification apply to downlink Wide Area NCR, downlink Medium Range NCR and downlink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For NCR type 1-C, type 1-H, NCR downlink classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell scenarios with a NCR to UE minimum distance along the ground equal to 35 m.

-	Medium Range NCR are characterised by requirements derived from Micro Cell scenarios with a NCR to UE minimum distance along the ground equal to 5 m.

-	Local Area NCR are characterised by requirements derived from Pico Cell scenarios with a NCR to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios.

-	Note: The requirements in this specification for LA NCR type 1-C apply to NCR type 1-C with declared output power less than or equal to LA rated output power limits as in table 6.2.1-1.

4.3A.2	Network controlled repeater class for uplink

The requirements in this specification apply to uplink Wide Area NCR and uplink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR with and without connectors.

For NCR type 1-C, type 1-H, NCR uplink classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area NCR are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

4.4	Regional requirements

4.5	Repeater configurations

4.5.1	General configurations

For repeater type 1-C, the requirements are applied at the repeater antenna connector (BS-side connector or UE-side connector) for downlink or uplink for the configuration in normal operating conditions.

Figure 4.5.1-1: Repeater type 1-C test ports

For Network controlled Repeater type 1-C, the requirements are applied at the NCR antenna connector (BS-side connector or UE-side connector) for downlink or uplink for the configuration in normal operating conditions.

Figure 4.5.1-2: Network controlled Repeater type 1-C downlink and uplink interface

For Network controlled Repeater type 1-H, the requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector or UE-side TAB connector) for downlink or uplink for the configuration in normal operating conditions.

Figure 4.5.1-3: Network controlled Repeater type 1-H downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate TAB connectors.

4.5.2	Transmission with multiple BS-side antenna connectors

4.5.3	Transmission with multiple UE-side antenna connectors

4.5.4	Duplexers

4.5.5	Power supply options

4.5.6	Ancillary RF amplifiers

4.5.7	Combining of repeaters

4.6	Manufacturer declarations

The following repeater declarations listed in table 4.6-1, when applicable to the repeater under test, are required to be provided by the manufacturer for the conducted requirements testing of the repeater type 1-C. Declarations can be made independently for UL and DL.

Table 4.6-1: Manufacturer declarations for repeater type 1-C conducted test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.1 | Repeater class | Repeater class of the repeater, declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. |
| D.2 | Operating bands and passband frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the repeater and passband frequency range(s) within the operating band(s) that the repeater can operate in. Declarations shall be made per antenna connector. |
| D.3 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [4]. |
| D.4 | Additional operating band unwanted emissions | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 2, Note 3). |
| D.5 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. |
| D.6 | Co-location with other base stations, repeaters and IABs | The manufacturer shall declare whether the repeater under test is intended to operate co-located with Base Stations, repeaters and IABs of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. |
| D.7 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. |
| D.8 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D.12) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. |
| D.9 | Rated output power per passband (Prated,p,AC) | Conducted rated output power per passband, per single band connector or multi-band connector. Declared per supported passband, per antenna connector. (Note 1) |
| D.10 | Rated total output power (Prated,t,AC) | Conducted total rated output power. Declared per supported operating band, per antenna connector. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1) |
| D.11 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) |
| D.12 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the repeater. Declared per antenna connector. |
| D.13 | Equivalent connectors | List of antenna connectors which have been declared equivalent. Equivalent connectors imply that the antenna connector are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector are identical and the transmitter unit and/or receiver unit driving the antenna connector are of identical design. |
| D.14 | Connecting network loss range for repeater testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for repeater type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) |
| D.15 | Long delay repeater | Declared only if the repeater internal delay between the input and output for this repeater does not fit within the TDD transient time. The repeater is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of repeaters internal delay is declared using this declaration. |
| D.16 | Input signal power level for maximum output power | Declaration of input signal power level required to reach maximum output power. Declared per passband. |
| D.17 | Repeater radiating direction | Declaration on whether the repeater is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the repeater radiates. |
| NOTE 1: If a repeater is capable of 256QAM operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, and the other declaration is applicable when not configured for 256QAM operation. If a repeater is not capable of 256QAM operation, only one declaration can be made. NOTE 2: If repeater is declared to support Band n20 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 3: If repeater BS is declared to support Band n24 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. |  |  |

Table 4.6-2: Manufacturer declarations for NCR type 1-C and type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | NCR-Fwd | NCR-MT |
| D.1 | Repeater class |  |  | Repeater class of the repeater, declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. |  |  |  |  |  |
| D.2 | Operating bands and passband frequency ranges |  |  | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the repeater and passband frequency range(s) within the operating band(s) that the repeater can operate in. Declarations shall be made per antenna connector. |  |  |  |  |  |
| D.3 | Spurious emission category |  |  | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [4]. |  |  |  |  |  |
| D.4 | Additional operating band unwanted emissions |  |  | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 2, Note 3). |  |  |  |  |  |
| D.5 | Co-existence with other systems |  |  | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. |  |  |  |  |  |
| D.6 | Co-location with other base stations, repeaters and IABs |  |  | The manufacturer shall declare whether the repeater under test is intended to operate co-located with Base Stations, repeaters and IABs of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. |  |  |  |  |  |
| D.7 | Single band connector or multi-band connector |  |  | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. |  |  |  |  |  |
| D.8 | Other band combination multi-band restrictions |  |  | Declare any other limitations under simultaneous operation in the declared band combinations (D.12) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. |  |  |  |  |  |
| D.9 | Rated output power per passband (Prated,p,AC) |  |  | Conducted rated output power per passband, per single band connector or multi-band connector. Declared per supported passband, per antenna connector. (Note 1) |  |  |  |  |  |
| D.10 | Rated total output power (Prated,t,AC) |  |  | Conducted total rated output power. Declared per supported operating band, per antenna connector. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1) |  |  |  |  |  |
| D.11 | Rated multi-band total output power, Prated,MB,TABC |  |  | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) |  |  |  |  |  |
| D.12 | Operating band combination support |  |  | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the repeater. Declared per antenna connector. |  |  |  |  |  |
| D.13 | Equivalent connectors |  |  | List of antenna connectors which have been declared equivalent. Equivalent connectors imply that the antenna connector are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector are identical and the transmitter unit and/or receiver unit driving the antenna connector are of identical design. |  |  |  |  |  |
| D.14 | Connecting network loss range for repeater testing with ancillary RF amplifiers |  |  | Declaration of the range of connecting network losses (in dB) for repeater type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) |  |  |  |  |  |
| D.15 | Long delay repeater |  |  | Declared only if the repeater internal delay between the input and output for this repeater does not fit within the TDD transient time. The repeater is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of repeaters internal delay is declared using this declaration. |  |  |  |  |  |
| D.16 | Input signal power level for maximum output power |  |  | Declaration of input signal power level required to reach maximum output power. Declared per passband. |  |  |  |  |  |
| D.17 | Repeater radiating direction |  |  | Declaration on whether the repeater is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the repeater radiates. |  |  |  |  |  |
| NOTE 1: If a repeater is capable of 256QAM operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, and the other declaration is applicable when not configured for 256QAM operation. If a repeater is not capable of 256QAM operation, only one declaration can be made. NOTE 2: If repeater is declared to support Band n20 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 3: If repeater BS is declared to support Band n24 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. |  |  |  |  |

4.7	Test configurations

4.7.1	General

4.7.2	Test signal used to build Test Configurations

4.7.3	RTC1: Contiguous spectrum operation

4.7.5	RTC2: Non-contiguous spectrum operation

4.7.6	RTC3: Multi-band test configuration for full carrier allocation

4.7.7	RTC4: Multi-band test configuration with high PSD per carrier

4.7A	Test configurations for NCR

4.7A.1	General

4.7A.2	Test signal used to build Test Configurations

4.7A.3	NCRTC1: Contiguous spectrum operation

4.7A.4	NCRTC2: Non-contiguous spectrum operation

4.7A.5	NCRTC3: Multi-band test configuration for full carrier allocation

4.7A.6	NCRTC4: Multi-band test configuration with high PSD per carrier

4.8	Applicability of requirements

4.8.1	General

4.8.2	Applicability of test configurations for single-band operation

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for a repeater capable of single carrier, and/or multi-carrier operation in both contiguous and non-contiguous spectrum in single band.

For a repeater declared to support a single passband within a single band (D.2), the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For a repeater declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for all passbands, the test configurations in the third column of table 4.8.3-1 shall be used for testing.

For a repeater declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for all passbands, the test configurations in the fourth column of table 4.8.3-1 shall be used for testing.

Table 4.8.3-1: Test configurations for a repeater capable of single or multiple passbands in a single band

| Test case | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| --- | --- | --- | --- |
| Repeater output power | RTC1 | RTC1 | RTC1, RTC2 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD repeater) | RTC1 | RTC1 | RTC1, RTC2 |
| Error Vector Magnitude | RTC1 | RTC1 | RTC1, RTC2 |
| Adjacent Channel Leakage power Ratio (ACLR) | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | RTC2 | RTC2 |
| Operating band unwanted emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Transmitter spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Output intermodulation | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Input intermodulation | N/A | N/A | N/A |
| Adjacent Channel Rejection Ratio (ACRR) | RTC1 | RTC2 | RTC1, RTC2 |
| Receiver spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |

Table 4.8.3-2: Test configurations for a NCR capable of single or multiple passbands in a single band

| Test case | Single passband NCR | Multiple passband capable NCR with identical parameters per passband | Multiple passband capable NCR with different parameters per passband |
| --- | --- | --- | --- |
| NCR output power |  |  |  |
| Frequency stability |  |  |  |
| Out of band gain |  |  |  |
| Transmit ON/OFF power (only applied for NR TDD repeater) |  |  |  |
| Error Vector Magnitude |  |  |  |
| Adjacent Channel Leakage power Ratio (ACLR) |  |  |  |
| Cumulative ACLR requirement in non-contiguous spectrum |  |  |  |
| Operating band unwanted emissions |  |  |  |
| Transmitter spurious emissions |  |  |  |
| Output intermodulation |  |  |  |
| Input intermodulation |  |  |  |
| Adjacent Channel Rejection Ratio (ACRR) |  |  |  |
| Receiver spurious emissions |  |  |  |

4.8.4	Applicability of test configurations for multi-band operation

For a repeater declared to be capable of multi-band operation, the test configuration in table 4.8.4-1 and/or table 4.8.3-1 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-1 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.3-1 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-1 shall be used.

Table 4.8.4-1: Test configuration for a BS capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| Repeater output power | RTC1/2 (Note 1), RTC3 | RTC1/2 (Note 1), RTC3 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD BS) | RTC3 | RTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude (Note 8) | RTC1/2 (Note 1), RTC3 | RTC1/2 (Note 1), RTC3 |
| Adjacent Channel Leakage power Ratio (ACLR) | RTC1/2 (Note 1), RTC4 (Note 2) | RTC1/2 (Note 1, 3), RTC4 (Note 2, 3) |
| Cumulative ACLR requirement in non-contiguous spectrum | RTC2 (Note 1), RTC4 (Note 2) | RTC2 (Note 1, 3) |
| Operating band unwanted emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Transmitter spurious emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Output intermodulation | RTC1/2 (Note 1) | RTC1/2 (Note 1, 3) |
| Input Intermodulation | N/A | N/A |
| Adjacent Channel Rejection Ratio | RTC1/2 (Note 1), RTC4 (Note 2) | RTC1/2 (Note 1, 3), RTC4 (Note 2, 3) |
| Receiver spurious emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Note 1: RTC1 and/or RTC2 shall be applied in each supported operating band. Note 2: RTC4 may be applied for Inter RF Bandwidth gap only. Note 3: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |

Table 4.8.4-1: Test configuration for a NCR capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| NCR output power |  |  |
| Frequency stability |  |  |
| Out of band gain |  |  |
| Transmit ON/OFF power (only applied for NR TDD BS) |  |  |
| Frequency error |  |  |
| Error Vector Magnitude (Note 8) |  |  |
| Adjacent Channel Leakage power Ratio (ACLR) |  |  |
| Cumulative ACLR requirement in non-contiguous spectrum |  |  |
| Operating band unwanted emissions |  |  |
| Transmitter spurious emissions |  |  |
| Output intermodulation |  |  |
| Input Intermodulation |  |  |
| Adjacent Channel Rejection Ratio |  |  |
| Receiver spurious emissions |  |  |
| Note 1: RTC1 and/or RTC2 shall be applied in each supported operating band. Note 2: RTC4 may be applied for Inter RF Bandwidth gap only. Note 3: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |

4.9	RF channels and test models

4.9.1	RF channels

4.9.2	Test models

4.9.2.1	General

4.9.2.2	FR1 test models for repeater type 1-C for DL

4.9.2.3		FR1 test models for repeater type 1-C for UL

4.9A	RF channels and test models for NCR

4.9A.1	RF channels

4.9A.2	Test models

4.9A.2.1	General

4.9A.2.2	FR1 test models for NCR for DL

4.9A.2.3		FR1 test models for NCR for UL

4.10	Requirements for contiguous and non-contiguous spectrum

4.11	Requirements for repeater capable of multi-band operation

4.12	Format and interpretation of tests

5	Operating bands and channel arrangement

For the NR operation in NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.106 [2], clause 5 and its relevant clauses.

6	Conducted characteristics

6.1	General

6.2	Repeater output power

6.2.1	Definition and applicability

6.2.2	Minimum requirement for RF repeater

6.2.2A	Minimum requirement for NCR

6.2.3	Test purpose

6.2.4	Method of test

6.2.4.1	Initial conditions

6.2.4.2	Procedure for RF repeater

6.2.4.2A	Procedure for NCR

6.2.5	Test requirement for RF repeater

6.2.5A	Test requirement for NCR

6.3	Frequency stability

6.3.1	Definition and applicability

6.3.2	Minimum Requirement for RF repeater

6.3.2A	Minimum Requirement for NCR

6.3.3	Test purpose

6.3.4	Method of test

6.3.5	Test Requirements for RF repeater

6.3.5A	Test Requirements for NCR

6.4	Out of band gain

6.4.1	Definition and applicability

6.4.2	Minimum Requirement for RF repeater

6.4.2A	Minimum Requirement for NCR

6.4.3	Test purpose

6.4.4	Method of test

6.4.4.1	Initial conditions

6.4.4.2	Procedure for RF repeater

6.4.4.2A	Procedure for NCR

6.4.5	Test Requirements for RF repeater

6.4.5A	Test Requirements for NCR

6.5	Unwanted emissions

6.5.1	General

6.5.2	Adjacent Channel Leakage Power Ratio (ACLR)

6.5.2.1	Definition and applicability

6.5.2.2	Minimum requirement for RF repeater

6.5.2.2A	Minimum requirement for NCR

6.5.2.3	Test purpose

6.5.2.4	Method of test

6.5.2.4.1	Initial conditions

6.5.2.4.2	Procedure for RF repeater

6.5.2.4.2A	Procedure for NCR

6.5.2.5	Test requirements for RF repeater

6.5.2.5	Test requirements for NCR

6.5.3	Operating band unwanted emissions

6.5.3.1	Definition and applicability

6.5.3.2	Minimum requirement for RF repeater

6.5.3.2A	Minimum requirement for NCR

6.5.3.3	Test purpose

6.5.3.4	Method of test

6.5.3.4.1	Initial conditions

6.5.3.4.2	Procedure for RF repeater

6.5.3.4.2A	Procedure for NCR

6.5.3.4	Test requirements for RF repeater

6.5.3.4A	Test requirements for NCR

6.5.4	Transmitter spurious emissions

6.5.4.1	Definition and applicability

6.5.4.2	Minimum requirement for RF repeater

6.5.4.2A	Minimum requirement for NCR

6.5.4.3	Test purpose

This test measures conducted spurious emissions while the transmitter is in operation.

6.5.4.4	Method of test

6.5.4.4.1	Initial conditions

6.5.4.4.2	Procedure for RF repeater

6.5.4.4.2A	Procedure for NCR

6.5.4.5	Test requirements for RF repeater

6.5.4.5A	Test requirements for NCR

6.5.5	Receiver spurious emissions

6.5.5.1	Definition and applicability

6.5.5.2	Minimum requirement for RF repeater

6.5.5.2A	Minimum requirement for NCR

6.5.5.3	Test purpose

6.5.5.4	Method of test

6.5.5.4.1	Initial conditions

6.5.5.4.2	Procedure for RF repeater

6.5.5.4.2A	Procedure for NCR

6.5.5.5	Test requirements for RF repeater

.

6.5.5.5A	Test requirements for NCR

6.6	Repeater Error Vector Magnitude

6.6.1	Downlink repeater error vector magnitude

6.6.1.1	General

6.6.1.2	Minimum requirements for RF repeater

6.6.1.2A	Minimum requirements for NCR

6.6.1.3	Test purpose

6.6.1.4	Method of test

6.6.1.4.1	Initial conditions

6.6.1.4.2	Procedure for RF repeater

6.6.1.4.2A	Procedure for NCR

6.6.1.5	Test requirement for RF repeater

6.6.1.5A	Test requirement for NCR

6.6.2	Uplink repeater error vector magnitude

6.6.2.1	General

6.6.2.2	Minimum requirement for RF repeater

6.6.2.2A	Minimum requirement for NCR

6.6.2.3	Test purpose

6.6.2.3	Method of test

6.6.2.3.1	Initial conditions

6.6.2.3.2	Procedure for RF repeater

6.6.2.3.2A	Procedure for NCR

6.6.2.4	Test requirement for RF repeater

6.6.2.4	Test requirement for RF repeater

6.7	Input intermodulation

6.7.1	Definition and applicability

6.7.1.1	General

6.7.1.2	Minimum requirements for RF repeater

6.7.1.2A	Minimum requirements for NCR

6.7.1.3	Test purpose

6.7.1.4	Method of test

6.7.1.4.1	Initial conditions

6.7.1.4.2	Procedure for RF repeater

6.7.1.4.2A	Procedure for NCR

6.7.1.5	Test requirements for RF repeater

6.7.1.5A	Test requirements for NCR

6.8	Output intermodulation

6.8.1	Definition and applicability

6.8.1.1	General

6.8.1.2	Minimum requirements for RF repeater

6.8.1.2A	Minimum requirements for NCR

6.8.1.3	Test purpose

6.8.1.4	Method of test

6.8.1.4.1	Initial conditions

6.8.1.4.2	Procedure for RF repeater

6.8.1.4.2A	Procedure for NCR

6.8.1.5	Test requirements for RF repeater

6.8.1.5A	Test requirements for NCR

6.9	Adjacent Channel Rejection Ratio (ACRR)

6.9.1	Definitions and applicability

6.9.2	Co-existence with UTRA, E-UTRA and NR

6.9.2.1	Minimum requirements for RF repeater

6.9.2.1A	Minimum requirements for NCR

6.9.2.2	Test purpose

6.9.2.3	Method of test

6.9.2.3.1	Initial conditions

6.9.2.3.2	Procedure for RF repeater

6.9.2.3.2A	Procedure for NCR

6.9.2.3.3	Test Requirements for RF repeater

6.9.2.3.3A	Test Requirements for NCR

6.10	Transmit ON/OFF power

6.10.1	Transmitter OFF power

6.10.1.1	Definition and applicability

6.10.1.2	Minimum requirement for RF repeater

6.10.1.2A	Minimum requirement for NCR

6.10.1.3	Test purpose

6.10.1.4	Method of test

6.10.1.5	Test requirements for RF repeater

6.10.1.5A	Test requirements for NCR

6.10.2	Transmitter transient period

6.10.2.1	Definition and applicability

6.10.2.2	Minimum requirement for RF repeater

6.10.2.2A	Minimum requirement for NCR

6.10.2.3	Test purpose

6.10.2.4	Method of test

6.10.2.4.1	Initial conditions

6.10.2.4.2	Procedure for RF repeater

6.10.2.4.2A	Procedure for NCR

#### 6.10.2.5	Test requirements for RF repeater

#### 6.10.2.5A	Test requirements for NCR

6.11	Output power dynamics

6.11.1	Definition and applicability

6.11.2	Minimum requirement

6.11.3	Test purpose

6.11.4	Method of test

6.11.5	Test requirements

6.12	Transmit signal quality

6.12.1	Definition and applicability

6.12.2	Minimum requirement

6.12.3	Test purpose

6.12.4	Method of test

6.12.5	Test requirements

6.13	Transmit intermodulation

6.13.1	Definition and applicability

6.13.2	Minimum requirement

6.13.3	Test purpose

6.13.4	Method of test

6.13.5	Test requirements

6.14	Conducted reference sensitivity

6.14.1	Definition and applicability

6.14.2	Minimum requirement

6.14.3	Test purpose

6.14.4	Method of test

6.14.5	Test requirements

6.15	Conducted maximum input level

6.15.1	Definition and applicability

6.15.2	Minimum requirement

6.15.3	Test purpose

6.15.4	Method of test

6.15.5	Test requirements

6.16	Conducted adjacent channel selectivity

6.16.1	Definition and applicability

6.16.2	Minimum requirement

6.16.3	Test purpose

6.16.4	Method of test

6.16.5	Test requirements

6.17	Conducted blocking characteristics

6.17.1	Definition and applicability

6.17.2	Minimum requirement

6.17.3	Test purpose

6.17.4	Method of test

6.17.5	Test requirements

6.18	Conducted spurious response

6.18.1	Definition and applicability

6.18.2	Minimum requirement

6.18.3	Test purpose

6.18.4	Method of test

6.18.5	Test requirements

6.19	Conducted intermodulation characteristics

6.19.1	Definition and applicability

6.19.2	Minimum requirement

6.19.3	Test purpose

6.19.4	Method of test

6.19.5	Test requirements

6.20	Conducted spurious emissions

6.20.1	Definition and applicability

6.20.2	Minimum requirement

6.20.3	Test purpose

6.20.4	Method of test

6.20.5	Test requirements