3GPP TSG-RAN WG4 Meeting#110                             R4-2402516

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.20.5

Source: 	ZTE Corporation

Title: 	Discussion on NCR-MT RRM performance requirement

Document for:	Approval

Introduction

Until last RAN#102 meeting, RAN4 has completed the core requirement for NCR-MT RRM requirement and it’s supposed to start the discussion on RRM performance requirement in this meeting. RAN4 need to discuss at least the following issues:

- Spec structure and corresponding work split among companies;

- Test case and Test configurations.

In this contribution, we share our initial views on RRM performance requirement for NCR-MT and spec structure.

Discussion

## 2.1. Test cases and test configurations

Similar as IAB-MT RRM requirement defined in TS38.174, RAN4 has defined the following RRM requirements for NCR-MT in TS 38.106.

- RRC_CONNECTED state mobility for NCR-MTs

- RRC re-establishment

- Random Access

- Timing

- NCR-MT transmit timing

- NCR-MT timing advance

- Cell phase synchronization accuracy

- Signalling Characteristics for NCR-MTs

- Radio Link Monitoring

- Link Recovery procedure

Since most of NCR-MT RRM requirement are reusing from IAB-MT except for some additional specific SMTC configuration for IAB-MT, therefore we would like to propose to reuse test cases of IAB-MT as baseline for NCR-MT RRM test case.

Regarding the test case for random access requirement, the corresponding test case in TS 38.133 are defined based on the absolute and relative power tolerance requirement in TS 38.101-1/2, however for NCR-MT, there are no absolute/relative power tolerance requirement for Wide area NCR-MT and only relative power tolerance requirement defined for Local area NCR-MT, therefore we propose not to define the test case for NCR-MT due to missing requirements for conducting the test.

Proposal 1: to reuse the IAB-MT RRM test case as baseline for NCR-MT RRM test case.

| NCR-MT RRM core requirements |  | Test cases |
| --- | --- | --- |
| RRC Connected mode mobility control | RRC Re-establishment |  |
| Timing | NCR-MT transmit timing |  |
|  | NCR-MT timing advance |  |
| Signalling Characteristics | Radio Link Monitoring |  |
|  | Beam Failure Detection and Link Recovery procedures |  |

From the test configuration perspective, since NCR-MT is supposed to be supported in any of FR1 and FR2-1 bands, however IAB-MT is only supposed to be supported in n41/77/78/79 and FR2-1 TDD bands. In other words, some FDD configuration in FR1 also need to be considered for NCR-MT.

Proposal 2: to add the test configuration of FDD operation on top of IAB-MT test configuration for NCR-MT;

## 2.2. Spec structure and work split

Similar as IAB specification, the RRM performance requirement for NCR-MT is proposed to keep in the Annex of 38.106, the spec structure of NCR-MT RRM performance is proposed as following:

Annex G (normative): NCR-MT RRM Testing

G.1	NCR-MT RRM test configurations

G.1.1	Reference measurement channels

G.1.2	OFDMA channel noise generator (OCNG)

G.1.3	Antenna configurations

G.1.4	BWP configurations

G.1.5	SSB Configurations

G.1.6	SMTC Configurations

G.1.7	CSI-RS configurations

G.1.8	Angle of Arrival (AoA) for FR2 RRM test cases

G.1.9	TCI State Configuration

G.1.10	Configurations of CSI-RS for tracking

G.2	NCR-MT RRM test cases

G.2.1	RRC_CONNECTED state mobility for NCR-MTs

G.2.2	Timing

G.2.2.1	Transmit timing

G.2.2.1.1	NR NCR-MT Transmit Timing Test for FR1

G.2.2.1.2	NR NCR-MT Transmit Timing Test for FR2

G.2.3	Signalling Characteristics for NCR MTs

G.2.3.1	Radio link Monitoring

G.2.3.2	Beam Failure Detection and Link Recovery Procedure

Annex H (normative):
Conditions for NCR-MT RRM requirements applicability for operating bands

Conclusions

In this contribution, we want to share some initial views on NCR-MT RRM performance requirements and proposals are made as following:

Proposal 1: to reuse the IAB-MT RRM test case as baseline for NCR-MT RRM test case.

| NCR-MT RRM core requirements |  | Test cases |
| --- | --- | --- |
| RRC Connected mode mobility control | RRC Re-establishment |  |
| Timing | NCR-MT transmit timing |  |
|  | NCR-MT timing advance |  |
| Signalling Characteristics | Radio Link Monitoring |  |
|  | Beam Failure Detection and Link Recovery procedures |  |

Proposal 2: to add the test configuration of FDD bands on top of IAB-MT test configuration for NCR-MT;

Proposal 3: to consider the spec structure of NCR-MT RRM performance requirement as proposed in section 2.2 and the corresponding work split.

| NCR-MT RRM core requirements |  | Work split |
| --- | --- | --- |
| Test configurations |  |  |
| RRC Connected mode mobility control | RRC re-establishment |  |
|  | Random Access | N/A |
| Timing | NCR-MT transmit timing |  |
|  | IAB-MT timing advance |  |
| Signalling Characteristics | Radio Link Monitoring |  |
|  | Beam Failure Detection and Link Recovery procedures |  |
| Conditions for NCR-MT RRM requirements applicability for operating bands |  |  |

References

- R4-2321643 Big CR to TS 38.106 on RRM core requirements for NR network-controlled repeaters, Approved.

- R4-2015511, Discussion on test cases for IAB, Huawei, Noted