3GPP TSG-RAN WG4 Meeting#110                              R4-2402521

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.18.5.1

Source: 	ZTE Corporation

Title: 	Further discussion on UE RF Tx requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In the previous RAN4 meetings, there were lots of discussions on VSAT RF requirements, however there are still lots of open issues left for further discussions. In the contribution, we provided some further detailed analysis for the remaining issues.

Discussion

## 2.1. The minimum peak EIRP requirement and minimum output power

From our understanding, the minimum EIRP requirement should be decided from at least the following factors to be considered: link budget between SAN and VSAT, system performance and also realistic implementation assumption.

Basic principle:

- The minimum peak EIRP of NTN VSAT will at least support the low MCS at the SAN receiver side. Low MCS in the following calculation is based on QPSK with coding rate as 1/3, then corresponding SNR for 95% throughput should be around -1dB;

- For the minimum output power, it should be calculated based on the closest distance with 90 degree elevation angle; For the minimum peak EIRP is assumed as lowest supported elevation angle.

- Both minimum peak EIRP and minimum output power should be defined in the BW agnostic way similar as current specification.

EIRPmin−PathlossFreeSpce−Atmosphereloss+SANAntennaGain=−174dBm/Hz+10log10BW+NF+SNR+IM

The minimum EIRP could be derived as following:

EIRPmin=−174dBm/Hz+10log10BW+NF+SNR+IM−SANAntennaGain+PathlossFreeSpce+Atmosphereloss

Where:

- NF is 3.5dB for GEO and LEO at the Ka-band;

- BW is assumed as 100MHz since 100MHz as mandatory channel bandwidth, FFS for other bandwidth

- SNR is assumed as -1dB which is applicable for QPSK with 1/3 coding rate;

- IM is the implementation margin for low modulation order;

- SAN_AntennaGain= 58.5-[3/0]dB;  where 58.5dB is for GEO and 38.5dB is for LEO and 3dB margin is considered for VSAT is located at the beamprint edge which is corresponding to 3dB beamwidth of radiation beam. If VSAT is located at the center of beamprint, 0dB margin could be considered;

- Pathloss_FreeSpace=32.44+20*log10(D2)+20*log10(f_MHz);  Note: if consider the pathloss captured in TR 38.811 clause 6.6, then LOS probability and shadow fading varying with elevation angle should be considered as well especially for the low elevation angle.

- f_ML is assumed as 27GHz for UL direction;

- D2 is slant distance between VSAT and SAN and could be derived as following:

Where

- h is assumed as 35786km for GEO 12000km/600km for LEO1200KM/LEO600KM

- B is assumed as 6378km for earth radius

- r is assumed as h+B;

- Theta is elevation angle, this could be 30 or 90 degree.

- D2 is D2=(B∗sinθ)2+r2−B2−B∗sinθ

In order to better visualize the required transmitter antenna size of NTN UE, we made the following estimation for it.

Antenna_size_min=10^((EIRP_min-Gain_element-Tx_power_perPa)/20)

Where

- EIRP_mini is derived as above;

- G_element is assumed as 5dB;

- Tx_power_perPa is assumed between 5dBm and 7dbm per the discussion for FR2 BS SEM discussion in the past. Companies could refer to the following contribution R4-1807526/6929/0283.

Proposal 1: to consider the basic principle as described in section 2.1 to specify the minimum peak EIRP and minimum output power for VSAT.

- 2.2. SEM

Agreement:

Option 1:

- Proposal 1:

- for the out-of-band emission requirement, propose to use the ITU-R SM.1541-6, Annex 5 OoB domain emission limits for space services (earth and space stations) as starting point and further discuss any modification if necessary (e.g. to keep consistency with other regional out-of-band emission requirement). [ZTE,R4-2320332]

Option 2:

Table 9.5.2.2-1: General NR spectrum emission mask for FR2-NTN.

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
|  0-5 | -5 | -5 | -5 | -5 | 1 MHz |
|  5-10 | -13 | -5 | -5 | -5 | 1 MHz |
|  10-20 | -13 | -13 | -5 | -5 | 1 MHz |
|  20-40 | -13 | -13 | -13 | -5 | 1 MHz |
|  40-80 | -13 | -13 | -13 | -13 | 1 MHz |
|  80-100 | -13 | -13 | -13 | -13 | 1 MHz |
|  100-160 |  | -13 | -13 | -13 | 1 MHz |
|  160-200 |  | -13 | -13 | -13 | 1 MHz |
|  200-400 |  |  | -13 | -13 | 1 MHz |
|  400-800 |  |  |  | -13 | 1 MHz |
| ± 800-1600 |  |  |  |  | 1 MHz |
| ± 1600-3200 |  |  |  |  | 1 MHz |
| ± 3200-4000 |  |  |  |  | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |

Option 3:  other options are not precluded.

During the last RAN4 meeting, we agreed VSAT ACLR requirement as [14]dB for both fixed VSAT and mobile VAST which is the same as Rel-17 GSO SAN ACLR requirement. In the current ITU-R SM.1541-6, the regulatory requirement is the same for earth station and space station. For the Ka-band, since its uplink spectrum is higher than 15GHz, then its reference bandwidth should be updated from 4kHz to 1MHz.

Proposal 2: propose to specify the VSAT SEM requirement as following:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWUE | 0.5 MHz f_offset < 2× BWUE + 0.5 MHz | maxSE limit, Prated,UE – 10log10(BWUE) −40×log10 f_offset−0.5BWUE×2+1dBm | 1 MHz |
| NOTE 1: BWUE is in the unit of MHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for earth services. |  |  |  |

- 2.3. Transmitter spurious emission requirement

The maximum input power of NTN VSAT will support the NTN VSAT with capability of decoding highest modulation order at the supported elevation angle 90 closest to satellite, then corresponding SNR of DL 64QAM with 948/1024 coding rate for 95% throughput should be around 20dB;

Agreement:

Option 1:

- The following regulatory requirement should be taken into account.

- for the general transmitter spurious emission requirement, propose to follow the ITU-R SM.329 recommendation.[ZTE,R4-2320332]

TABLE 2 [ITU-R SM.329]

| Space services (mobile earth stations)(3), (4) | 43  10 log P, or 60 dBc, whichever is less stringent |
| --- | --- |
| Space services (fixed earth stations)(3), (4) | 43  10 log P, or 60 dBc, whichever is less stringent |

| Service category in accordance with RR Article 1 or equipment type(1), (2) | Maximum permitted spurious domain emission power in the relevant reference bandwidth (see recommends 4.1) (dBm) with P, PEP or X (W) |
| --- | --- |
| All services except those services quoted below | –13 dBm if P 500 W 10 log P – 40 if P 500 W |
| Space services (mobile earth stations)(3), (4) | –13 dBm if P 50 W 10 log P – 30 if P 50 W |
| Space services (fixed earth stations)(3), (4) | –13 dBm if P 50 W 10 log P – 30 if P 50 W |
| Space services space stations(3), (5), (6) | –13 dBm if P 50 W 10 log P – 30 if P 50 W |

- CEPT ERC 74.01

- FCC 25.202(f)(4)

Proposal 3: propose to specify the VSAT spurious emission requirement as following:

| Spurious frequency range | Prated,c,sys (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,c,sys – 60dB |  |  |

- 2.4. Transmitter dynamic range requirement

Agreement: The following requirements need be specified.

- The minimum output power

- The OFF output power

- Table 6.3.2-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | -35 | -35 | -35 | -35 |
|  | 47.52 MHz | 95.04 MHz | 190.08 MHz | 380.16 MHz |

- The transition period requirement

- Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR2

- Power control requirement,

6.3.4	Power control

6.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

6.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 ('Pmin') and the maximum output power as specified in sub-clause 6.2.1 as minimum peak EIRP ('Pmax'). The intermediate power point 'Pint' is defined in table 6.3.4.2-2

Table 6.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

Conclusions

In this contribution, we want to share some further views on VSAT RF requirements for NTN in Ka-band.and proposals are made as following:

Proposal 1: to consider the basic principle as described in section 2.1 to specify the minimum peak EIRP and minimum output power for VSAT.

Proposal 2: propose to specify the VSAT SEM requirement as following:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWUE | 0.5 MHz f_offset < 2× BWUE + 0.5 MHz | maxSE limit, Prated,UE – 10log10(BWUE) −40×log10 f_offset−0.5BWUE×2+1dBm | 1 MHz |
| NOTE 1: BWUE is in the unit of MHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for earth services. |  |  |  |

Proposal 3: propose to specify the VSAT spurious emission requirement as following:

| Spurious frequency range | Prated,c,sys (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,c,sys – 60dB |  |  |

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- R4-2220573	WF on NTN UE RF requirements, ZTE, approved.

- R4-2220119, Topic summary for [105][139] NR_NTN_enh_UERF, ZTE

- https://www.etsi.org/deliver/etsi_en/301400_301499/301459/02.01.01_60/en_301459v020101p.pdf

- RF Spectrum FCC 47CFR25.138, 47CFR25.202,

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.202

- https://www.digisat.org/enterprise-75cm-professional-ka-band-terminal

- https://www.satsig.net/eut2beac.htm

- R4-2303533,WF on NTN UE RF requirements for Ka band, ZTE, Approved.

- R4-2306624, WF on NTN UE RF requirements in Ka-band. ZTE, approved.

- https://www.eutelsat.pl/files/contributed/satellites/pdf/esog120.pdf