3GPP TSG-RAN WG4 Meeting #110	R4-2402721

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results

Agenda item:	8.27.2

Document for:	Information

Introduction

In [1], the Way Forward (WF) of the discussions on Enhancement of NR Dynamic Spectrum Sharing (eDSS) during RAN4#109 is documented. It was agreed in RAN4#109 that interested companies will bring preliminary simulation for evaluation in RAN4#110. The simulation assumptions were also agreed in RAN4#109 and tabulated in [1].

Simulation Results

In this contribution, we provide our simulation results based on the agreements in [1], which defined 8 simulation test cases. The corresponding simulation assumptions [1] are given in the Appendix below for a direct reference.

The simulation results of PDCCH eDSS based on the given simulation assumptions are tabulated in Table 1 below:

Table 1. Simulation Results of PDCCH eDSS

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 1 | 5.0 |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | 1 | -2.3 |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | 1 | -1.4 |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | 1 | -6.1 |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | 1 | 4.6 |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | 1 | -2.2 |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | 1 | -1.1 |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | 1 | -6.0 |

Our Observations and Proposals related to the simulation results and to other open issues can be found in [2].

Conclusion

In this contribution, we provide our preliminary simulation results on PDCCH of eDSS. More on our Observations and Proposals related to the simulation results and to other open issues can be found in [2].

- References

- R4-2321139, WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, 3GPP RAN4#109, Ericsson, Chicago, USA, November 13 ‒ 17, 2023

- R4-2402720, Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements, 3GPP RAN4#110, Nokia, Nokia Shanghai Bell, Athens, Greece, February 26 ‒ March 01, 2024

- Appendix

## Simulation Assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |