3GPP TSG-RAN WG4 Meeting #110	R4-2402734

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Core Requirements for RedCap Positioning

Agenda item:	8.14.2.4

Document for:	Discussion

Introduction

The specification of positioning for RedCap devices is one of the objectives in the Rel-18 WID on expanded and improved positioning [1]. The WID depicts the RAN4 work related to this accuracy improvement technology.

In RAN4#109 [2], general aspects/scenarios for RedCap positioning were discussed.

Issue 2-1-1: PRS measurements with FH without gaps Agreements: - PRS measurements with FH without gaps is not supported and therefore no RAN4 requirements are needed. Issue 2-1-2: Applicable RRC states for PRS measurements without FH Agreements: - RAN4 is already defining requirements of RedCap positioning without FH in RRC_IDLE and RRC_INACTIVE. In RRC idle the requirements are for RSTD, PRS-RSRP, PRS-RSRPP. Issue 2-1-3: Applicable RRC states for PRS measurements with FH Agreements: - Uphold the previous agreement in WF in R4-2314353 at RAN4#108

In RAN4#109 [2], PRS measurements for RedCap without FH were discussed.

Issue 2-2-1: CSSF for PRS measurement requirements in RRC connected state without FH Agreements: - Update clause 9.1A.5.2 to update CSSF for PRS measurement requirements in RRC connected state without FH - Ericsson volunteers to provide the draft CR: - CR#20A: CSSF for PRS measurement requirements in RRC connected state for RedCap

In RAN4#109 [2], PRS measurements for RedCap with FH were discussed.

Issue 2-3-1: Side conditions for PRS measurements with FH Agreements: - The side conditions for positioning measurements without FH are reused for positioning measurements with FH. Issue 2-3-3: Nsample for reduced number of samples in PRS measurement requirements with FH Agreements: - When UE is configured to perform positioning measurements with FH then Nsample = 2 is considered in measurement period requirement for reduced number of sample case. Issue 2-3-4: Measurement sample definition under FH Agreements: - Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap. Issue 2-3-5: Number of Rx beam sweeps for defining PRS measurement requirements with FH Agreements: - The Rx beam sweeping factor (NRxBeam) definition in Rel-17 is reused for PRS measurements with FH. Issue 2-3-6: Number of hops within a single MG occasion Agreements: - Number of hops within a single MG occasion is defined as a requirement. The details are FFS: Issue 2-3-7: Type of PRS measurement requirements with FH Agreements: - RAN4 to define requirements for positioning measurement based on multiple hops. Issue 2-3-10: Applicability conditions for PRS measurement requirements with FH Agreements: - Requirements for PRS measurements with FH are applicable subject to explicit request by the LMF to perform measurements with FH.

This contribution investigates impacts to RRM core requirements for RedCap positioning.

Discussion

The remaining issues and impacts to RRM core requirements for RedCap positioning on DL and UL are discussed in this section.

PRS measurement for RedCap with FH

- Proposals - Option 1: HW - The number of hops within a single MG occasion Nhop is defined as Nhop=minN-1*MK+1,Nhop,max where - N is the number of PRS repetitions within the MG occasion - M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap) - K is the number of slots per hop and K=maxK',M - K'=1 if the RF switching time is ≤ 7 symbols and K'=2 otherwise - Nhop,max is the maximum number of hops derived from the configured PRS BW BWPRS - Option 2: QC - Nhops=Nhopsslot∙NrepPRS if MrepPRS=1 or Nhopsslot≥1, otherwise Nhops=1+NrepPRS-11MrepPRS∙Nhopsslot - NrepPRS is the number of PRS inter-slot repetitions within a single MG instance, excluding the gap retuning times - MrepPRS is the stride of PRS inter-slot repetitions - Nhopsslot is the number of frequency hops per slot as a function of number of PRS symbols, PRS comb size, and retuning time between hops. FFS how to determine Nhopsslot from these parameters and whether to limit its maximum value.

It is discussed that the number of hops within a single MG occasion is defined as a requirement. For the definition of the number of hops, the upper bound of the number of hops in an MG occasion is determined based on the configured PRS BW. In the definition of Option1, it needs a further discussion on whether the unit of K and M should be a slot, and also the method of defining the value of K’ remains unclear. Therefore, the number of hops should be defined as a function of factors such as PRS symbols, PRS comb size, switching time, total PRS BW size, and maximum BW size supported by UE.

The upper bound of the number of hops in an MG occasion is determined based on the configured PRS BW and/or the maximum size of PRS BW.

The number of hops should be defined as a function of factors such as PRS symbols, PRS comb size, switching time, total PRS BW size, and maximum BW size supported by UE.

Conclusion

The scope of RRM impacts for RedCap positioning as part of the Rel-18 WI on Expanded and Improved NR positioning is investigated in this contribution.

The following proposals are made.

- The upper bound of the number of hops in an MG occasion is determined based on the configured PRS BW and/or the maximum size of PRS BW.

The number of hops should be defined as a function of factors such as PRS symbols, PRS comb size, switching time, total PRS BW size, and maximum BW size supported by UE.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation, RAN#99

- R4-2317386, “WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation,” Ericsson, RAN4#108bis.