3GPP TSG-RAN4 Meeting #110	R4-2402767

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.113 | CR | 0137 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR to TS 37.113 on update of FRC used in performance criteria |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 6.1.1: update of fixed reference channel models used for E-UTRA. - 6.1.5, 6.2: NR with NB-IoT is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS are not up to date. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.5, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

## 6.1	Performance criteria for continuous phenomena for BS

### 6.1.1	E-UTRA performance criteria

The throughput in table 6.1.1-1 is stated relative to the maximum throughput of the FRC. The maximum throughput for an FRC is equal to the payload size * the number of uplink subframes per second.

The BS Uplink and Downlink paths shall each meet the performance criteria defined in table 6.1.1-1 during the test. If the Uplink and Downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in table 6.1.1-1. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.1-1: E-UTRA BS Performance criteria for continuous phenomena for BS

| E-UTRA channel bandwidth (MHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- |
| 1.4 | FRC A1-1 in Annex A.1 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 3 | FRC A1-2 in Annex A.1 in TS 36.104 [2] |  |
| 3 | FRC A1-6 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation |  |
| 5 | FRC A1-3 in Annex A.1 in TS 36.104 [2] |  |
| 5 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation |  |
| 10 | FRC A1-3 in Annex A.1 in TS 36.104 [2] (Note 3) |  |
| 10 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation |  |
| 15 | FRC A1-3 in Annex A.1 in TS 36.104 [2] (Note 3) |  |
| 15 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation |  |
| 20 | FRC A1-3 in Annex A.1 in TS 36.104 [2] (Note 3) |  |
| 20 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation |  |

NOTE 1:	The performance criteria, "Throughput > 95 % / No loss of service", applies also if a bearer with another characteristics is used in the test.

NOTE 2:	The performance criteria, "Throughput > 90 % / No loss of service", applies instead if the Uplink and Downlink paths are evaluated as a one loop.

NOTE 3:	This is the information data rate of a single instance of the bearer mapped to 25 resource blocks. The performance criteria shall be met for each consecutive application of a single instance of the bearer mapped to disjoint frequency ranges with a width of 25 resource blocks each.

### 6.1.2	UTRA performance criteria

The BS Uplink and Downlink paths shall each meet the performance criteria defined in table 6.1.2-1 during the test. If the Uplink and Downlink paths are evaluated as a one loop, then the criterion is two times the value shown in table 6.1.2-1. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.2-1: UTRA BS performance criteria for continuous phenomena for BS

| Bearer information data rate | Performance criteria |
| --- | --- |
| 12.2 kbps | BLER < 10-2 No loss of service |
| 64 kbps |  |
| 144 kbps |  |
| 384 kbps |  |

NOTE:	The performance criteria, "BLER < 10-2 / No loss of service", applies also if a bearer with another characteristics is used in the test.

### 6.1.3	GSM/EDGE performance criteria

#### 6.1.3.1	GSM/EDGE downlink

The BER of the downlink shall be assessed during the test according to one of the test methods of Annex A.1.

If the test method of Annex A.1.1 is used, the measured BER of the class 2 bits of TCH/FS shall not exceed 1,6 % during the test.

NOTE:	This BER is the upper limit in TS 45.008 [32] for RXQUAL = 3.

If the test method of Annex A.1.2 is used, the value of RXQUAL shall not exceed 3 during the test.

At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data, and the communication link shall have been maintained.

#### 6.1.3.2	GSM/EDGE uplink

The BER of the uplink shall be assessed during the test according to one of the test methods of Annex A.2.

If the test method of Annex A.2.1 is used, the value of RXQUAL shall not exceed 3 during the test.

If the test method of Annex A.2.2 is used, the measured BER of the class 2 bits of TCH/FS shall not exceed 1,6 % during the test.

NOTE:	This BER is the upper limit in TS 45.008 [32] for RXQUAL = 3.

For a base station, the RXQUAL of the uplink shall not exceed three (3) measured during the test sequence.

At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data, and the communication link shall have been maintained.

### 6.1.4	NB-IoT performance criteria

The throughput in table 6.1.4-1 is stated relative to the maximum throughput of the FRC. The Maximum throughput for an FRC equals the Payload size / (Number of Resource Unit * time to send one Resource Unit).

The BS Uplink and Downlink paths shall each meet the performance criteria defined in table 6.1.4-1 during the test. If the Uplink and Downlink paths are evaluated as a one loop, then the criterion is two times the throughput reduction shown in table 6.1.4-1. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.4-1: NB-IoT BS Performance criteria for continuous phenomena for BS

| NB-IoT Sub-carrier spacing [kHz] | Reference measurement channel | Performance criteria (Note 1, Note 2) |
| --- | --- | --- |
| 15 | FRC A14-1 in Annex A.14 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 3.75 | FRC A14-2 in Annex A.14 in TS 36.104 [2] |  |

NOTE 1:	The performance criteria, "Throughput > 95 % / No loss of service", applies also if a bearer with another characteristics is used in the test.

NOTE 2:	The performance criteria, "Throughput > 90 % / No loss of service", applies instead if the Uplink and Downlink paths are evaluated as a one loop.

### 6.1.5	NR performance criteria

The throughput in table 6.1.5-1 is stated relative to the maximum throughput of the FRC. The maximum throughput for an FRC is equal to the [payload size * the number of uplink subframes per second].

The BS Uplink and Downlink paths shall each meet the performance criteria defined in table 6.1.5-1 during the test. If the Uplink and Downlink paths are evaluated as a one loop, then the criterion is two times the throughput reduction shown in table 6.1.5-1. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.5-1: NR BS performance criteria for continuous phenomena for BS

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate (as in TS 38.104 [35], annex A.1) (Note 3) | Performance criteria (Note1, Note 2) |
| --- | --- | --- | --- |
| 3 | 15 |  |  | G-FR1-A1-7 | Throughput > 95 %, No loss of service |  |
|  |  | G-FR1-A1-21 for NR with NB-IoT in-band operation |  |
| 5, 10, 15 | 15 | G-FR1-A1-1 |  |
|  |  | G-FR1-A1-10 for NR with NB-IoT in-band operation |  |
| 10, 15 | 30 | G-FR1-A1-2 |  |
| 10, 15 | 60 | G-FR1-A1-3 |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 |  |
|  |  | G-FR1-A1-11 for NR with NB-IoT in-band operation |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. NOTE 3: These reference measurement channels are not applied for band n46, n96 and n102. |  |  |  |

Table 6.1.5-1a: FR1 performance criteria for continuous phenomena for band n46

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate (as in TS 38.104 [35], annex A.1) (Note 3) | Performance criteria (Note1, Note 2) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-13 |  |
|  | 60 | G-FR1-A1-3 |  |
| 20 | 15 | G-FR1-A1-14 |  |
|  | 30 | G-FR1-A1-15 |  |
|  | 60 | G-FR1-A1-6 |  |
| 40 | 15 | G-FR1-A1-16 |  |
|  | 30 | G-FR1-A1-17 |  |
|  | 60 | G-FR1-A1-6 |  |
| 60 | 30 | G-FR1-A1-18 |  |
|  | 60 | G-FR1-A1-6 |  |
| 80 | 30 | G-FR1-A1-19 |  |
|  | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1.5-1b: FR1 performance criteria for continuous phenomena for band n96 and n102

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate (as in TS 38.104 [35], annex A.1) (Note 3) | Performance criteria (Note1, Note 2) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-15 |  |
|  | 60 | G-FR1-A1-6 |  |
| 40 | 15 | G-FR1-A1-16 |  |
|  | 30 | G-FR1-A1-17 |  |
|  | 60 | G-FR1-A1-6 |  |
| 60 | 30 | G-FR1-A1-18 |  |
|  | 60 | G-FR1-A1-6 |  |
| 80 | 30 | G-FR1-A1-19 |  |
|  | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

## 6.2	Performance criteria for transient phenomena for BS

At the conclusion of each exposure the EUT shall operate with no user noticeable loss of the communication link.

At the conclusion of the total test comprising the series of individual exposures the EUT shall operate as intended with no loss of user control functions or stored data, as declared by the manufacturer, and the communication link shall have been maintained.

The below characteristics should be chosen based on manufacture declaration and are defined as:

-	The E-UTRA channel bandwidth and bearer information data rate are defined in table 6.1.1-1.

-	The UTRA bearer information data rate are defined in table 6.1.2-1.

-	The GSM/EDGE the test BER assessment are defined in subclause 6.1.3.1 for downlink and subclasue 6.1.3.2 for uplink, while the criteria is defined in subclause 6.2.

-	The NB-IoT sub-carrier spacing and reference measurement channel are defined in table 6.1.4-1.

-	The NR channel bandwidth, sub-carrier spacing and bearer information data rate are defined in table 6.1.5-1, table 6.1.5-1a and table 6.1.5-1b.

## 6.3	Performance criteria for continuous phenomena for Ancillary equipment

---------- end of changes----------