3GPP TSG-RAN4 Meeting #110	R4-2402854

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Title: 	Way forward on [110][306] NR_NTN_enh_Part2

Agenda Item:	8.18.9

Source: 	Ericsson

Document for:	Approval

# For agreement

## SAN RF

### SAN type 2-O out-of-band blocking requirement

Agreement:

Requirement:

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.0029 | CW |
| 12750 to FUL,low – [1500] | EISREFSENS + 6 dB | 0.0029 | CW |
| FUL,high + [1500] to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.0029 | CW |

### SAN type 2-O in-channel selectivity requirement

Agreement:

Specify ICS requirement based on ICS value of 14 dBc and considering the IoT level is up to 5dBc

## SAN Conformance

### MU for 27.5-30 GHz for SAN type 2-O OTA receiver tests

Agreement:

Reused FR2 TN MUs (24.25-29.5 GHz) for SAN type 2-O receivers tests in 27.5-30.0 GHz.

### MU for 17.3-20.2 GHz for SAN type 2-O OTA transmitter tests

Agreement:

Consider FR2 TN MUs (24.25-29.5 GHz) for SAN type 2-O transmitter tests in 17.3-20.2 GHz.

Values are kept in [], encouraging additional inputs from TE vendors.

### Manufacturer declarations

Agreement:

Reused manufacturer declarations for SAN type 1-O except OSDD-related manufacturer  declaration

Add EIS REFSENS for FR2 (EISREFSENS_50M)

### Test signal

Agreement:

Use the following test signal when testing SAN type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth declared per operating band (D.7) shall be used. |  |  |

### Test models

Agreement:

Specify following test models:

| Test models for FR2 in TS38.141-2 | Applicability for NTN SAN | Test model for FR2-NTN |
| --- | --- | --- |
| NR-FR2-TM1.1 | Applicable but need to be updated for FDD NTN FR2 | NR-SAN-FR2-TM1.1 |
| NR-FR2-TM2 | Applicable, applicability for 64QAM pending on the further discussion. | NR-SAN-FR2-TM2 |
| NR-FR2-TM3.1 | Applicable | NR-SAN-FR2-TM3.1 |

### Test configurations

Agreement:

Reuse test configuration NRTC1 (TS 38.181) for SAN type 2-O, no other TC is needed.

### RF channels to be tested

Agreement:

RF channels for single carrier and multi-carrier operation for transmitter requirements for SAN type 1-O can be reused for SAN type 2-O.

| Requirements |  | single carrier | Multi-carrier |
| --- | --- | --- | --- |
| 9.2 | Radiated transmit power | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.3 | OTA SAN output power | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.4 | OTA total power dynamic range | M | - |
| 9.6.2 | OTA Frequency Error | Same as EVM | Same as EVM |
| 9.6.3 | OTA Modulation quality | B, T | BRFBW, TRFBW |
| 9.7.2 | OTA Occupied BW | M | - |
| 9.7.3 | OTA ACLR | B, T | BRFBW, TRFBW |
| 9.7.4 | OTA Out-of-band emissions | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.7.5 | OTA Transmitter spurious emissions | B for spurious frequencies below the band, T for frequencies above the band | BRFBW for spurious frequencies below the band, TRFBW for frequencies above the band |

# Work split to draft TS 38.181

| Subclause number | Subclause title | Volunteered company |
| --- | --- | --- |
| 4.1 | Measurement uncertainties and test requirements | Ericsson |
| 4.2 | Requirement reference points |  |
| 4.5 | SAN configurations |  |
| 4.6 | Manufacturer declarations | All |
| 4.7 | Test configurations | Ericsson |
| 4.8 | Applicability of requirements | CATT |
| 4.9 | RF channels and test models | CATT |
| 9.2 | Radiated transmit power | Ericsson |
| 9.3 | OTA SAN output power | THALES |
| 9.4 | OTA output power dynamics | Ericsson |
| 9.6 | OTA transmitted signal quality | THALES |
| 9.7.1 / 9.7.2 / 9.7.3 | General / OTA occupied bandwidth / ACLR | THALES |
| 9.7.4 / 9.7.5 | OTA out-of-band emissions / OTA transmitter spurious emissions | THALES |
| 10.1 / 10.2 | General / OTA sensitivity | CATT |
| 10.3 | OTA reference sensitivity level | CATT |
| 10.4 / 10.5 | OTA dynamic range / OTA in-band selectivity and blocking | ZTE |
| 10.6 | OTA out-of-band blocking | ZTE |
| 10.9 | OTA in-channel selectivity | ZTE |