3GPP TSG-RAN WG4 Meeting # 110 	R4-2402883

Athens, Greece, Feb 26th – March 1st, 2024

Agenda Item: 8.5.1.2

Source: MediaTek Inc.

Title: Discussion on case 2 requirements (NCSG and concurrent MG)

Document for: Discussion

# Introduction

In existing Rel-15/16 NR, two measurement gaps have been identified, which are per-UE and per-FR measurement gap. Later in Rel-17 NR, three measurement gap enhancement have been considered, which are: (i) pre-configured MG pattern(s) per configured BWP (fast MG configuration), (ii) multiple concurrent and independent MG patterns, and (iii) network controlled small gap (NCSG). Now in Rel-18, further work objective to enhance the existing measurement gap is agreed on, which is ‘Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG’, given in the revised work item description (WID) [1] as below:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Define requirements only for the following two cases for UE configured with: - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: This WID does not include any inter-working with MUSIM gaps

In the previous RAN4 109 meeting, the issues are captured in the way forward (WF) [2]. The analysis and discussion on the issues from the WF are provided in the next section.

# Discussion

In this contribution, we discuss the issues of: (i) UE capabilities, and (ii) other Rel-17 rules.

## Discussion on UE capabilities for Concurrent gaps with NCSG and NCSG + NCSG

The open issues of UE capabilities are given as:

Issue [Case 2] Whether to consider an additional capability for NCSG + NCSG in an FR Moderator: Agreement from previous meeting that a new capability for Concurrent gaps with NCSG in an FR will be defined as a new UE capability in Rel-18. This issue is whether to consider an additional UE capability for NCSG + NCSG. - Way forward - Option 1: No, without additional UE capability. - Option 2: Yes, with additional UE capability - Option 3: Whether to consider an additional capability for NCSG + NCSG in an FR can be discussed once RRM requirements for NCSG + NCSG become stable

Furthermore, on issue of UE capability for NCSG + NCSG: In the meeting RAN4#105, RAN4 agreed to consider the scenario of NCSG + NCSG in an FR. However, it was also mentioned during the discussion that the necessity to introduce this scenario is not high. This is because NCSG can be seen as a supplement to traditional MG and it is expected to have gap as common case. Besides, it is common in RAN4 to introduce a UE capability when there is a feature with tight requirements that normal UE cannot achieve or a feature that is not typical for all NR UE. Therefore, RAN4 should support this with UE capability (i.e. it is left to the UE implementation). Furthermore, the use of NCSG + NCSG scenario is different than the scenario of NCSG + type-2 MG, hence, RAN4 should define two separate capabilities to indicate the support of these two scenarios.

- RAN4 shall support two separate UE capabilities for the scenarios of (i) NCSG + NCSG in an FR, and (ii) Type-2 MG + NCSG in an FR.

## Discussion on other Rel-17 rules to be revisited

The agreements from previous meetings are given as:

### Previous agreements on potential changes for NCSG upon SCell activation: < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

The open issues are captured as:

Issue 4-2-1: [Rel-17] Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? (Clarify Rel-17 understanding) < Agreement from online session> Continue discussion Rel-17 Issue 4-2-1, 4-2-2 in maintenance part. - Option 1: - No, - The deactivated SCell MO(s) are measured within NCSG if the UE reports ‘intraFreq-needForNCSG’ on the band(s) where the deactivated SCell MO(s) located in. - Otherwise, the deactivated SCell MO(s) are measured outside of MG with interruption. - Option 2: - The Rel-17 UE behavior is that when the SMTC of deactivated SCell is fully or partially overlapped with NCSG, the deactivated SCell is measured via NCSG regardless the UE capability report of intraFreq-needForNCSG. - Option 3: - In Rel-17, if the UE supports NCSG (ncsg-MeasGapNR-Patterns-r17 or ncsg-MeasGapPatterns-r17) and the network configures an NCSG supported by the UE: - A deactivated SCell is measured within NCSG if at least some of the SCell’s SMTC overlaps with NCSG occasions; otherwise, the deactivated SCell is measured outside of NCSG. - An activated SCell is measured within NCSG only if either the SCell’s SSB is outside the active DL BWP or the SCell’s SMTC fully overlaps with NCSG, and the UE signaled that the SCell can be measured with NCSG via needForGapNCSG-InfoNR; otherwise, the activated SCell is measured outside of NCSG, if possible. Issue 4-2-2: [Rel-17] Whether a new UE capability is needed for the support of NCSG for deactivated SCell? < Agreement from online session> - Continue discussion Rel-17 Issue 4-2-1, 4-2-2 in maintenance part. - Option 1: - No - Option 2: - A new indication shall be introduced enable support of NCSG for deactivated SCell only.

Issue 4-2-1/4-2-2:

Given that this issue is currently handled in the maintenance phase of Rel-17 the issues 4-3-1/4-3-2/4-3-3 shall be postponed until the Rel-17 understanding is clarified.

- RAN4 shall wait for the outcome of the issue on whether all deactivated Scell will be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG from Rel-17 maintenance on Rel-17 MGE.

# Summary

In this contribution, discussion on NCSG and concurrent MG is provided, and we have the following proposals:

Proposal 1: RAN4 shall support two separate UE capabilities for the scenarios of (i) NCSG + NCSG in an FR, and (ii) Type-2 MG + NCSG in an FR.

Proposal 2: RAN4 shall wait for the outcome of the issue on whether all deactivated Scell will be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG from Rel-17 maintenance on Rel-17 MGE.

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2321640, “WF for NR_MGE-2_part1”, MediaTek Inc, Chicago, USA, Nov 2023.