3GPP TSG-RAN WG4 Meeting #110	R4-2403062

Athens, Greece, 26th Feb 2024 ‒ 1st Mar 2024

Agenda item:			8.23.5

Source:	Ericsson

Title:	Ad-hoc meeting minutes for [110][325] NR_redcap_enh_demod

Document for:	Information

# Topic #2: UE demodulation and CSI reporting requirements (8.23.4.1)

## Open issues summary

### Sub-topic 2-1	General

Issue 2-1-1: Scope of UE demodulation performance part

- Proposals (Nokia)

  - Consider the impact to UE demodulation performance requirements and CSI reporting requirements for both eRedCap support levels

    - Support level 1: Rel-18 eRedCap device does not support Rel-17 RedCap

    - Support level 2: Rel-18 eRedCap device also supports Rel-17 RedCap

- Recommended WF

  - Moderator: According to TS38.306 V18.0.0, an eRedCap UE shall set this field (supportOfERedCap-r18) to supported but shall not indicate support of supportOfRedCap-r17.

  - Moderator first wants to confirm if there is any UEs corresponding to ‘level 2’

Discussion:

Rel-18 eRedCap UE demodulation performance part focus only on Rel-18 eRedCap UE (Level 1)?

Apple: it is reasonable to consider Rel-18 standalone eRedCap UE.

Huawei: Same view as Apple.

Tentative agreement:

Rel-18 eRedCap UE demodulation performance part focus only on Rel-18 eRedCap UE. Rel-18 eRedCap UE demodulation requirements are specified for UE indicating support of supportOfERedCap-r18.

### Sub-topic 2-2	PDSCH demodulation requirements

Issue 2-2-1: Whether to define PDSCH demodulation requirements

- Proposals

  - Option 1 (Apple): Discuss whether there are any need to set additional PDSCH requirements for Rel-18 eRedCap UE.

  - Option 2 (Huawei, Ericsson, Nokia, Qualcomm): Define new PDSCH demodulation requirements for Rel-18 eRedCap UE in FR1.

- Recommended WF

  - Most companies propose to define new PDSCH demodulation requirements targeting Rel-18 eRedCap UEs in FR1.

  - The moderator recommends defining new PDSCH demodulation requirements for Rel-18 eRedCap UEs in FR1

Apple: Define PDSCH demodulation requirements with the scope that demodulation requirements should not exceed Rel-17 RedCap scope.

Qualcomm: If the existing RedCap requirements does not exceed 10Mbps, it should be applicable for Rel-18 eRedCap UE.

Tentative agreement:

Define new PDSCH demodulation requirements for Rel-18 eRedCap UEs in FR1.

Issue 2-2-2:  Supported UE capability

- Observation:

  - Rel18 eRedCap UEs (supportOfERedCap-r18) may support the peak rate of 10Mbps:

    - With reduced BW (not supporting eRedCapNotReducedBB-BW-r18, FG 48-1) and,

    - Without reduced BW (supporting eRedCapNotReducedBB-BW-r18, FG 48-2)

- Proposals

  - Option 1 (Qualcomm, Huawei, Nokia): Define PDSCH targeting UEs both not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1) and eRedCap UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2).

  - Option 2 (Ericsson): Define PSDCH targeting UE only not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1).

- Recommended WF

  - Discuss whether Option 1 is agreeable.

Apple: OK to define both the capabilities, but need to consider the total number of test cases is not too much.

Tentative agreement:

Define PDSCH targeting UEs:

- not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1)

- supporting eRedCapNotReducedBB-BW-r18 (FG 48-2).

Issue 2-2-3:  Test setup

- Proposals (Huawei, Ericsson, Nokia, Qualcomm)

  - Specify PDSCH demodulation requirements for both 1Rx and 2Rx configurations, and for TDD, FDD and HD-FDD configurations.

- Recommended WF

  - Agree with the proposal.

Tentative agreement:

Specify PDSCH demodulation requirements for both 1Rx and 2Rx configurations, and for TDD, FDD and HD-FDD configurations.

Issue 2-2-4:  PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

- Test case proposals for FDD/HD-FDD 1Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | Ericsson Huawei Nokia Qualcomm |
| 16QAM 0.48, | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | Ericsson Huawei Qualcomm |
| 64QAM | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 | Ericsson Qualcomm |
|  | 3/4 | Rank 1 | 20 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 15 | TBD | TDLA30-10 | Nokia |

- Test case proposals for TDD 1Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | Ericsson Huawei Nokia Qualcomm |
| 16QAM | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | Ericsson Huawei Qualcomm |
| 64QAM | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 | Ericsson Qualcomm |
|  | 3/4 | Rank 1 | 10 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 7 | TBD | TDLA30-10 | Nokia |

- Test case proposals for FDD/HD-FDD 2Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | Ericsson Huawei Nokia Qualcomm |
| 16QAM | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | Ericsson Huawei Qualcomm |
| 64QAM | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 | Ericsson |
|  | 0.50 | Rank 2? | 25 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 2? | 20 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 15 | TBD | TDLA30-10 | Nokia |

- Test case proposals for TDD 2Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | Ericsson Huawei Nokia Qualcomm |
| 16QAM | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | Ericsson Huawei Qualcomm |
| 64QAM | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 | Ericsson |
|  | 0.50 | Rank 2? | 12 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 2? | 10 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 7 | TBD | TDLA30-10 | Nokia |

- Recommended WF

  - Define at least QPSK 1/3 rank 1 and 16QAM 0.47 rank 1 by reusing Rel-17 RedCap PDSCH demodulation requirement test setup.

  - Discuss whether to define other MCS cases: 64QAM and 256QAM. If defined, what is code rate, PRB size and rank.

Issue 2-2-5:  Test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

- Test case proposals for FDD/HD-FDD 1Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | Huawei Qualcomm |
| 16QAM | TBD | Rank 1 | 52 | TBD | TDLC300-100 | Qualcomm |
| 64QAM | TBD | Rank 1 | 52 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 1 | 20 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 15 | TBD | TDLA30-10 | Nokia |

- Test case proposals for TDD 1Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | Huawei Qualcomm |
| 16QAM | TBD | Rank 1 | 51 | TBD | TDLC300-100 | Qualcomm |
| 64QAM | TBD | Rank 1 | 51 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 1 | 10 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 7 | TBD | TDLA30-10 | Nokia |

- Test case proposals for FDD/HD-FDD 2Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | Huawei Qualcomm |
| 16QAM | TBD | Rank 1 | 52 | TBD | TDLC300-100 | Qualcomm |
| 64QAM | TBD | Rank 2 | 52 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 2? | 20 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 15 | TBD | TDLA30-10 | Nokia |

- Test case proposals for TDD 2Rx

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | Proposed companies |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | Huawei Qualcomm |
| 16QAM | TBD | Rank 1 | 51 | TBD | TDLC300-100 | Qualcomm |
| 64QAM | TBD | Rank 2 | 51 | TBD | TDLA30-10 | Qualcomm |
|  | 3/4 | Rank 2? | 10 | TBD | TDLA30-10 | Nokia |
| 256QAM | 4/5 | Rank 1 | 7 | TBD | TDLA30-10 | Nokia |

- Recommended WF

  - Define at least QPSK 1/3 rank 1 (Same requirements as UE not supporting eRedCapNotReducedBB-BW-r18).

  - Discuss whether to define other MCS cases: 16QAM, 64QAM and 256QAM. If defined, what is code rate, PRB size and rank.

### Sub-topic 2-3	SDR requirements

Issue 2-3-1: Applicability of SDR requirements for eRedCap UEs.

- Proposals (Ericsson, Huawei, Nokia)

  - Reuse the existing SDR test setup for eRedCap

    - (Nokia) Existing PRB bundling type and bundling size parameters can be applicable for eRedCap, i.e., PRBs are allocated in contiguous manner.

- Recommended WF

  - SDR requirements are applicable for eRedCap.

  - Existing PRB bundling type and bundling size parameters can be applicable for eRedCap, i.e., PRBs are allocated in contiguous manner.

Tentative agreement:

SDR requirements are applicable for eRedCap.

Existing PRB bundling type and bundling size parameters can be applicable for eRedCap, i.e., PRBs are allocated in contiguous manner.

Issue 2-3-2: How to capture the applicability.

- Proposals (Huawei)

  - Update TS 38.101-4 5.5.1 as follows:

5.5.1 FR1 single carrier requirements The requirements in this clause are applicable to the FR1 single carrier case. The requirements and procedure defined in Clause 5.5A.1 apply using operating band instead of CA configuration, and bandwidth instead of bandwidth combination. For RedCap and eRedCap, the requirements and procedure are defined in Clause 5.5A.1 except that the MIMO layers are configured to 2 for UE supporting 2 MIMO layers and 1 for UE supporting 1 MIMO layers for all operating band. Antenna configuration is 1x1 for UE supporting 1 layer and 2x2 for UE supporting 2 layers. For eRedCap with reduced peak date rate and reduced baseband bandwidth in FR1, only 25 PRBs for 15kHz SCS and 12RBs for 30kHz SCS are allocated for PDSCH transmission. For RedCap UE with HD-FDD mode, the additional test parameters are specified in Table 5.5.1-1.

- Recommended WF

  - Discuss the proposed TP.

### Sub-topic 2-4	PDCCH/PBCH demodulation requirements

Issue 2-4-1: Whether to define PDCCH demodulation requirements.

- Proposals (Apple, Ericsson, Nokia)

  - Not define new PDCCH demodulation requirements

    - PDCCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

- Recommended WF

  - Moderator expects this is agreeable without discussion:

    - Not define new PDCCH demodulation requirements

      - PDCCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

Tentative agreement:

Not define new PDCCH demodulation requirements

- PDCCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

Issue 2-4-2: Whether to define PBCH demodulation requirements.

- Proposals (Apple, Ericsson, Nokia)

  - Not define new PBCH demodulation requirements

    - PBCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

- Recommended WF

  - Moderator expects this is agreeable without discussion:

    - Not define new PBCH demodulation requirements

      - PBCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

Tentative agreement:

Not define new PBCH demodulation requirements

- PBCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

### Sub-topic 2-5	Applicability of UE demodulation requirements

Issue 2-5-1: Test applicability

- Proposals

  - Option 1 (Ericsson): Introduce applicability of demodulation requirements for eRedCap UE as follows in TS 38.101-5 5.1.1.

  - Option 2 (Nokia): Discuss whether the verification of QPSK performance requirements in the supported duplex mode could be set as minimum PDSCH requirements for eRedCap support level 2 in addition to verifying Rel-17 RedCap UE demodulation requirements.

- Recommended WF

  - Depending on the conclusion of Sub-topics 2-1, 2-2, 2-3, and 2-4.

### Sub-topic 2-6	CSI reporting requirements

Issue 2-6-1: Whether to define new CSI reporting requirements.

- Observation (Ericsson): CSI reporting requirements (CQI/PMI/RI reporting tests) defined for Rel-17 RedCap UE cannot be reused for Rel-18 eRedCap UE because RMC used for Rel-17 RedCap UE CSI reporting test is based on PDSCH with 10MHz for FDD/HD-FDD and 20MHz for TDD.

- Proposals

  - Option 1 (Apple, Huawei): Not to define new CSI reporting requirements.

  - Option 2: Define new CSI reporting requirements.

    - Option 2a (Ericsson): CQI and PMI

    - Option 2b (Nokia), CQI, PMI and RI.

- Recommended WF

  - Discuss whether to define new CSI porting requirements for Rel-18 eRedCap UEs.

Apple: Don’t want to increase the number of test cases from Rel-17. What is the intention of Option 2?

Huawei: Since CSI-RS allocation does not change, no need new requirements.

Qualcomm: Same view as Apple. UE BW does not change. It does not affect to CSI measurement.

Huawei: First we should discuss whether to test CSI reporting or not. If we agree to test, discuss how to test.

Tentative agreement:

Reuse the same test scenario of Rel-17 RedCap CSI reporting requirements for Rel-18 eRedCap UE.

- FRC/RMC should adapt to Rel-18 eRedCap UE capabilities (e.g., 10Mbps, baseband BW)

### Sub-topic 2-7 CQI reporting requirements (depending on Sub-topic 3-1)

Issue 2-7-1:  CQI reporting test in static condition

- Proposals (Ericsson, Nokia)

  - Define CQI reporting test in static condition for 1Rx/2Rx with TDD/FDD/HD-FDD.

    - Option 1 (Ericsson): Rank 1, lower SNR test point only

- Recommended WF

  - Depending on Sub-topic 2-6.

Issue 2-7-2:  CQI reporting test in fading condition

- Proposals (Ericsson, Nokia)

  - Define CQI reporting test in fading condition for 1Rx/2Rx with TDD/FDD/HD-FDD.

    - Option 1 (Ericsson): Rank 1, lower SNR test point only

- Recommended WF

  - Depending on Sub-topic 2.6.

Issue 2-7-3:  CQI-TBS table

- Proposals

  - Define new CQI-TBS mapping table for eRedCap CQI reporting tests.

    - Option 1 (Ericson): 15PRBs for SCS=15kHz and 7PRBs for SCS=30kHz, rank 1 only

    - Option 2 (Nokia): 25PRBs for SCS=15kHz and 12PRBs for SCS=30kHz

- Recommended WF

  - Depending on Sub-topic 2-6.

|  |  |  |  | Rank 1 Option 1 |  | Rank 1 Option 2 |  | Rank 2 Option 1 |  | Rank 2 Option 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS Scheme |  |  |  | TBS.1-3x | TBS.1-5x | TBS.1-3y | TBS.1-5y | TBS.1-4x | TBS.1-6x | TBS.1-4y | TBS.1-6y |
| MCS table |  |  |  | 64QAM |  | 64QAM |  | 64QAM |  | 64QAM |  |
| Number of allocated PDSCH resource blocks |  |  |  | 25 | 12 | 15 | 7 | 25 | 12 | 7 | 3 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Available RE-s |  |  |  | 3000 | 1440 | 1800 | 840 | 3000 | 1440 | 840 | 360 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  | Information Bit Payload per Slot |  | Information Bit Payload per Slot |  | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 704 | 336 | 432 | 192 | 1416 | 672 | 408 | 168 |
| 2 | 0.2344 | 0 |  | 704 | 336 | 432 | 192 | 1416 | 672 | 408 | 168 |
| 3 | 0.3770 | 2 |  | 1128 | 552 | 672 | 320 | 2280 | 1128 | 640 | 272 |
| 4 | 0.6016 | 4 |  | 1800 | 888 | 1128 | 504 | 3624 | 1736 | 1032 | 432 |
| 5 | 0.8770 | 6 |  | 2664 | 1256 | 1608 | 736 | 5248 | 2536 | 1480 | 640 |
| 6 | 1.1758 | 8 |  | 3624 | 1736 | 2152 | 984 | 7040 | 3368 | 2024 | 848 |
| 7 | 1.4766 | 11 | 16QAM | 4352 | 2152 | 2664 | 1256 | 8968 | 4224 | 2472 | 1064 |
| 8 | 1.9141 | 13 |  | 5760 | 2792 | 3496 | 1608 | 8968 | 4224 | 3240 | 1416 |
| 9 | 2.4063 | 15 |  | 7160 | 3496 | 4352 | 2024 | 8968 | 4224 | 4032 | 1736 |
| 10 | 2.7305 | 18 | 64QAM | 8192 | 3904 | 4864 | 2280 | 8968 | 4224 | 4608 | 2024 |
| 11 | 3.3223 | 20 |  | 9992 | 4726 | 6016 | 2792 | 8968 | 4224 | 5504 | 2408 |
| 12 | 3.9023 | 22 |  | 9992 | 4726 | 7040 | 3368 | 8968 | 4224 | 6528 | 2792 |
| 13 | 4.5234 | 24 |  | 9992 | 4726 | 8064 | 3824 | 8968 | 4224 | 7552 | 3240 |
| 14 | 5.1152 | 26 |  | 9992 | 4726 | 9224 | 4224 | 8968 | 4224 | 8456 | 3752 |
| 15 | 5.5547 | 28 |  | 9992 | 4726 | 9992 | 4608 | 8968 | 4224 | 9224 | 3968 |

### Sub-topic 2-8	PMI reporting requirements (depending on Sub-topic 3-1)

Issue 2-8-1:  PMI reporting tests

- Proposals (Ericsson, Nokia)

  - Define PMI reporting test for 1Rx/2Rx with TDD/FDD/HD-FDD.

    - Option 1 (Ericsson):

      - For PMI reporting test, the test metric is given as a throughput ratio of follow PMI and random PMI at 90% of peak rate with follow PMI.

      - New FRC should be defined with 25PRBs for SCS=15kHz and 12PRBs for SCS=30kHz

- Recommended WF

  - Depending on Sub-topic 2-6.

### Sub-topic 2-9	RI reporting requirements (depending on Sub-topic 3-1)

Issue 2-9-1: Test applicability

- Proposals

  - Option 1 (Nokia): Define RI reporting requirements for 2Rx with TDD/FDD/HD-FDD

  - Option 2 (Ericsson): Not to define RI reporting requirements.

- Recommended WF

  - Depending on Sub-topic 2-6.

### Sub-topic 2-10	Applicability of UE CSI reporting requirements

Issue 2-10-1: Test applicability

- Proposals

  - Option 1 (Ericsson): Introduce applicability of CSI reporting requirements for eRedCap UE as follows in TS 38.101-5 6.1.1.

  - Option 2 (Nokia): Decide for eRedCap support level 2 whether to specify only a minimum set of requirements for CSI reporting or the same set of requirements as for eRedCap support level 1.

- Recommended WF

  - Depending on the conclusion of Sub-topics 2-6, 2-7, 2-8, and 2-9.

  - Option 2 also depends on the conclusion on Sub-topic 2-1.