3GPP TSG RAN WG4 Meeting #110	R4-2403257

Athens, Greece, 26 February – 01 March, 2024

Agenda item:			8.3.5

Source:	Moderator (vivo)

Title:		Ad-hoc minutes for NR FR2 multi-Rx WI

Document for:	Approval

- Introduction

In this meeting, there are two email threads dedicated for FR2 multi-Rx chain DL reception, i.e., [110][205] FR2_multiRx_part1 and [110][209] FR2_multiRx_part2. Topic summaries for the two email threads [1, 2] summarized open issues in this meeting for the WI.

Since performance part is prioritized in this meeting, the ad-hoc session is mainly to handle some issues for performance part.

- Discussion

- Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

- Sub-topic 2-1: Test cases design

Sub-topic description:

Open issues and candidate options before f2f meeting:

#### Issue 2-4: Test case(s) for fast beam sweeping

- Proposals

  - Option 1a: (MTK)

    - Test case 1: Group-based beam reporting (GBBR) and fast beam sweeping

  - Option 1b: (CATT)

    - SSB based L1-RSRP measurements with GBBR can be considered to verify the enhancements of fast beam sweeping on L1 measurements.

  - Option 2a: (vivo)

    - SSB-based RLM measurement delay

    - SSB-based BFD measurement delay

    - SSB-based TRP specific CBD measurement delay

    - L1-RSRP with GBBR measurement delay

  - Option 2b: (OPPO)

    - TC1: SSB based RLM Out-of-sync Test with faster beam sweeping for FR2 PCell in non-DRX mode

    - TC2: SSB based BFD and LR Test with faster beam sweeping for FR2 PCell in non-DRX mode

    - TC3: SSB based L1-RSRP measurement with faster beam sweeping when DRX is not used

  - Option 2c: (ZTE)

    - Introduce test cases to verify the fast beam sweeping, the candidate test case including the SSB based GBBR L1-RSRP measurement, non-GBBR L1-RSRP measurement, RLM, BFD and CBD.

  - Option 2d: (Huawei)

    - RLM: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE capable of fast beam sweeping

    - BFD and Link Recovery: Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for UE capable of fast beam sweeping

    - TRP specific BFD and Link Recovery: TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for UE capable of fast beam sweeping

    - Non-GBBR L1-RSRP:	SSB based L1-RSRP measurement when DRX is not used for UE capable of fast beam sweeping

  - Option 2e: (Samsung)

    - RAN4 to introduce one test case to verify the enhancement of faster beam sweeping for RLM and BFD.

    - RAN4 to introduce a new test case for SSB based L1 measurement requirements for FR2 multi-Rx UE supported [faster beam switching capability] with groupBasedBeamReporting-r17 configured in Rel-18.

  - Option 2f: (Nokia)

    - For R18 multi-Rx reception, introduce one test case to verify the enhancement of faster beam sweeping on each type of SSB based L1 measurements.

  - Option 2g: (QC)

    - Define test cases for a fast Rx beam sweeping capable UE such that the test cases do not require any side conditions defined for multi-Rx based mTRP support. If the test cases are defined based on the framework of RLM/BFD, the RLM/BFD resources shouldn’t be configured as group-based L1-RSRP measurement resources.

  - Option 3: (CMCC, Ericsson)

    - Define test case(s) for fast beam sweeping.

  - Option 4: (Apple)

    - FFS test cases for fast beam sweeping

- Recommended WF

  - Company to discuss if option 1x is agreeable firstly.

  - If not, then discuss how to select TCs from option 2x.

Discussion:

Q1: Is it enough to define only one test case for fast beam sweeping?

TC 1: Group-based beam reporting (GBBR) and fast beam sweeping

Nokia: How fast beam sweeping is tested with GBBR?

MTK: GBBR is tested firstly. Fast beam sweeping can be tested afterwards with L1-RSRP non-GBBR.

SS: Whether two beam pairs are needed in the test?

Nokia: Choose one for fast beam sweeping with L1-RSRP non-GBBR.

ZTE: We donnot need to combine fast bream sweeping and GBBR.

SS: Choose another test for either RLM or BFD.

QC: we need to consider AoA configuraiton in the tests. Legacy test configuration should be reused for testing multi-Rx requirements. RLM tests is more complicated than BFD tests. The UE passes multi-Rx test should not be tested with [same] legacy test.

Agreements:

Option 1:

TC1: Define one test case for fast beam sweeping with non-GBBR L1-RSRP measurement with non-DRX.

TC2: Define one test case for fast beam sweeping with [RLM] with non-DRX.

Note: We may further discuss the conditions of how fast beam sweeping are applicable in the test configuration.

FFS AoA setup.

Option 2:

Introduce new tests for fast beam sweeping by revising all relevant legacy tests. The UE passes multi-Rx test should not be tested with [same] legacy test.

Q2: If not, what additional test case(s) should be introduced?

#### Issue 2-5: Test case(s) for scheduling restriction

- Proposals

  - Option 1a: (MTK)

    - Test case 3: Scheduling restriction. To consider either RLM or L1-RSRP scheduling restriction for test case.

  - Option 1b: (CATT)

    - Scheduling restriction relaxation for CSI-RS based L1-RSRP measurements with/without GBBR can be considered as the test case to verify scheduling restriction enhancement (relaxation).

  - Option 1c: (Huawei, Nokia)

    - For R18 multi-Rx reception, it is suggested to introduce one test case to verify the enhancement of scheduling restriction relaxation on CSI-RS based L1 measurements.

  - Option 2a: (vivo)

    - Scheduling restriction for CSI-RS based RLM

    - Scheduling restriction for CSI-RS based BFD

    - Scheduling restriction for CSI-RS based L1-RSRP with GBBR

    - Scheduling restriction for CSI-RS based RLM

  - Option 2b: (Samsung)

    - Scheduling restriction for CSI-RS based RLM

    - Scheduling restriction for CSI-RS based L1-RSRP

    - Scheduling restriction for TRP specific beam failure detection on FR2

  - Option 2c: (Ericsson)

    - Scheduling availability test cases are specified for:

      - RLM,

      - L1-RSRP,

      - BFD (cell-specific, TRP-specific),

      - CBD (cell-specific, TRP-specific).

  - Option 3: (CMCC, ZTE, Apple)

    - Define test case (s) for scheduling restriction.

  - Option 4: (OPPO)

    - RAN4 to discuss whether to introduce test case to verify scheduling restriction relaxation on CSI-RS based L1 measurement for R18 multi-Rx reception

- Recommended WF

  - Company to discuss if option 1x is agreeable firstly.

  - If not, then discuss how to select TCs from option 2x.

Discussion:

Q1: Is it enough to define only one test case for scheduling restriction?

Candidates for TC 2:

Option A: Test for scheduling restriction with CSI-RS based RLM

Option B: Test for scheduling restriction with CSI-RS based L1-RSRP with GBBR

Option C: Test for scheduling restriction with CSI-RS based L1-RSRP without GBBR.

Agreement:

TC 1:

Combine GBBR with scheduling restriction, i.e., GBBR is always configured in the test.

Scheduling restriction relaxation during the L1-RSRP non-GBBR is tested in the follow-up L1-RSRP non-GBBR measurement.

Only L1-RSRP GBBR measurement delay requirements are tested in this test, i.e., no accuracy is tested in this test.

Measurement restriction requirements are also tested in this test.

Q2: If not, what additional test case(s) should be introduced?

#### Issue 2-6: Test case(s) for measurement restriction

- Proposals

  - Option 1: (MTK)

    - Do not introduce test cases for measurement restriction for multi-RX

  - Option 2: (Apple, OPPO)

    - FFS test cases for measurement restriction.

  - Option 3: (CMCC, Nokia, ZTE)

    - Define test case (s) for measurement restriction.

  - Option 3a: (vivo)

    - Simultaneous measurement of CSI-RS based RLM and CSI-RS based L1-RSRP with GBBR

    - Simultaneous measurement of CSI-RS based BFD and CSI-RS based L1-RSRP without GBBR

    - TRP specific CSI-RS based BFD measurement delay

  - Option 3b: (Ericsson)

    - RLM

    - L1-RSRP

    - BFD (cell-specific, TRP-specific)

    - CBD (cell-specific, TRP-specific)

- Recommended WF

  - Company to discuss if option 1 is agreeable firstly.

  - If not, then discuss how to select TCs from option 3x.

Discussion:

Q1: Is it okay NOT to define test case for measurement restriction?

Apple: we don’t need to have measurement restriction requirements test.

Nokia: measurement restriction requirements can be tested together in scheduling restriction test.

QC: measurement restriction requirements can be tested together scheduling restriction.

MTK: there may be signal setup issues.

Q2: If not, what test case(s) should be introduced?

#### Issue 2-7: Test case(s) for dual TCI state switching

- Proposals

  - Option 1: (CATT)

    - For MAC-CE based dual PDCCH TCI state switch in mDCI scenario, no test cases are defined.

    - For MAC-CE based dual PDCCH TCI state switch in sDCI and RRC based TCI state switching in mDCI scenarios, the legacy delay requirements for single TCI state switching can be reused.

    - For DCI based dual TCI state switching for mDCI and sDCI, new test cases need to be defined.

    - For active TCI state list update, new test cases need to be defined.

  - Option 2: (vivo)

    - TC1: MAC-CE based TCI state switch for s-DCI PDCCH reception

    - TC2: DCI based TCI state switch for s-DCI PDSCH reception

    - TC3: DCI based TCI state switch for m-DCI PDSCH reception

    - TC4: RRC based TCI state switch for PDCCH reception

    - TC5: Active TCI state list update for s-DCI

  - Option 3: (Nokia)

    - For RRC-based TCI state switch in m-DCI, and MAC-CE based TCI state switch in s-DCI, it is sufficient that the UE passes the legacy test case.

    - Define MAC-CE based dual TCI state switch test case for m-DCI for overlapping TCI state switches from two TRPs.

    - Define MAC-CE based dual TCI state switch test case for PDCCH repetition.

    - Define a combined test case for dual active TCI state list update and DCI-based TCI state switch for s-DCI and m-DCI.

  - Option 4: (MTK)

    - Test case 2: Group-based beam reporting and TCI state switching

      - The test scenario is from single TCI (TCI state 0) to dual TCI (TCI states 1 and 2).

        - To avoid duplicated test scope, we don’t consider the scenarios of dual TCI to dual TCI or dual TCI to single TCI.

  - Option 5: (Huawei)

    - TC1:	sDCI MAC-CE based active TCI state switching

      - [RS1] to [RS2, RS3] for PDCCH repetition

    - TC2:	sDCI DCI based active TCI state switching

      - [RS1] to [RS2, RS3]

    - TC3:	sDCI DCI based active TCI state switching

      - [RS1, RS2] to [RS1]

    - TC4:	mDCI DCI based active TCI state switching

      - [RS1, RS2] to [RS1, RS3]

  - Option 6: (Ericsson)

    - For dual active TCI state switching delay, at least the following test cases are specified:

      - DCI-based based dual active TCI state switching with sDCI,

      - DCI-based based dual active TCI state switching with mDCI,

      - MAC-CE based dual active TCI state switching with sDCI,

      - RRC-based dual active TCI state switching.

      - For active TCI state list update, RAN4 prioritizes test cases with sDCI.

- Recommended WF

  - Company to discuss following test cases in a case-by-case manner. Combination of test cases, if possible, are encouraged.

    - TC1: MAC-CE based TCI state switch for s-DCI PDCCH repetition

    - TC2: DCI based TCI state switch for s-DCI scheduled PDSCH reception

    - TC3: DCI based TCI state switch for m-DCI scheduled PDSCH reception

    - TC4: RRC based TCI state switch for PDCCH reception

    - TC5: Active TCI state list update for s-DCI

    - TC6: MAC-CE based TCI state switch for m-DCI PDCCH reception

Discussion:

Q1: Any views on following tests with the test configuration of [RS1] to [RS2, RS3]?

TC1: MAC-CE based TCI state switch for s-DCI PDCCH repetition

TC2: DCI based TCI state switch for s-DCI scheduled PDSCH reception

TC3: DCI based TCI state switch for m-DCI scheduled PDSCH reception

TC4: RRC based TCI state switch for PDCCH reception

TC5: Active TCI state list update for s-DCI

TC6: MAC-CE based TCI state switch for m-DCI PDCCH reception

#### Issue 2-8: Test case(s) for group-based beam reporting

- Proposals

  - Option 1: (MTK)

    - Group-based beam reporting (GBBR) is tested in test cases for fast beam sweeping/TCI state switching.

  - Option 2: (Nokia)

    - Define a test case for group-based beam reporting using 4 probes, where the UE has to report two beam pairs from two different RS sets.

  - Option 3: (Ericsson)

    - For L1-RSRP, measurement delay and measurement accuracy tests are specified for GBBR

- Recommended WF

  - Further discuss

Discussion:

Q1: Any questions about option 1?

#### Issue 2-9: Test case(s) for group-based beam reporting accuracy requirements

- Proposals

  - Option 1: (Ericsson)

    - For L1-RSRP, measurement delay and measurement accuracy tests are specified for GBBR

- Recommended WF

  - Further discuss

Discussion:

Q1: Any questions about option 1?

#### Issue 2-10: List of test case(s) for multi-Rx in Rel-18

- Proposals

  - Option 1:

    - To be determined by conclusion of issues 2

- Recommended WF

  - Further discuss.

#### Issue 2-1: Whether to consider 4-layer MIMO in RRM test cases

- Proposals

  - Option 1: (Apple, Huawei, ZTE)

    - 4-layer MIMO is not verified in RRM test cases

  - Option 2: (Qualcomm)

    - Do not discuss the number of MIMO layers per TRP for RRM test cases

- Recommended WF

  - Agree on option 1, or close the issue without any conclusion.

#### Issue 2-2: AoA selection in RRM test cases

- Proposals

  - Option 1: (Huawei, Apple)

    - The AoAs for test cases shall be selected from the set that meet corresponding RF requirements. The selection of AoA offset shall wait for further RF conclusion.

  - Option 2: (Samsung)

    - The AoAs for RRM test cases do not need to be selected from the set that meet corresponding RF requirements, and are not subject to the RF requirement

    - For 2 AoAs selection for RRM test cases, if EIS requirement need to be considered, both EIS1 and EIS 2 should satisfy the spatial side condition; Or 95% throughput should be satisfied

  - Option 3: (Qualcomm)

    - Do not discuss the method of AoA selection for RRM test cases

- Recommended WF

  - Further discuss.

#### Issue 2-2a: Whether and how to define new 2AoA setup for multi-Rx

- Proposals

  - Option 1: RAN4 to discuss the necessity of defining a new 2AoA setup for multi-Rx chain DL reception (Samsung)

    - Setup X: 2 AoAs for multi-Rx chain DL reception

      - Setup Xa: 2 AoAs, both AoAs are in Rx beam peak directions.

        - It is possible for HST multi-Rx supported PC6.

      - Setup Xb: 2 AoAs, both AoAs are in non Rx beam peak directions.

      - Setup Xc: 2 AoAs,

        - Setup Xc-1: 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction

        - Setup Xc-2: 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction

- Recommended WF

  - Further discuss.

#### Issue 2-3: Number of probes in RRM test cases

- Proposals

  - Option 1a: (vivo)

    - RRM tests for verifying dual TCI states switch delay requirements are defined with at most 3 probes.

  - Option 1b: (CATT)

    - At least 3 probes are needed in the tests.

  - Option 1c: (Huawei)

    - RAN4 don't define test cases for dual TCI state from dual TCI to dual TCI ( [RS1, RS2] to [RS3, RS4]) where 4 active probes are needed, since the performance can be verified by Single TCI to dual TCI( [RS1] to [RS2, RS3]).

  - Option 1d: (Apple)

    - The baseline to verify UE performance of dual TCI state switching is from one TCI state to two TCI states.

  - Option 1e: (Ericsson)

    - For dual active TCI state switching, RAN4 will specify test cases at least for the switching from single to dual TCI state.

    - Deprioritize test cases for dual-to-dual active TCI state switching.

  - Option 2: (Nokia)

    - Four probes should be used for L1-RSRP group-based beam reporting test cases and well as for TCI state switch delay test cases.

    - Define a test case for group-based beam reporting using 4 probes, where the UE has to report two beam pairs from two different RS sets.

  - Option 3: (Qualcomm)

    - Do not discuss the number of probes to be used in RRM test cases

- Recommended WF

  - Further discuss.

Online Agreement on Monday, 26th Feb.:

- For TCI state switching test, further discuss and down-select from the following options:

  - Define test at least for Single TCI to dual TCI ([RS1] to [RS2, RS3])

  - Further discuss dual-to-dual active TCI state switching if the testability is confirmed.

    - Option a:

T1: Two TCI [RS1, RS3], with non-overlapping PDSCH

T2: Two TCI [RS1, RS2], with RS1 and RS2 are a beam pair

    - FFS Option b (feasibility to be further confirmed):

T1: Two TCI [RS1, RS3] (source), with RS1 and RS3 are a beam pair

T2: Two TCI [RS2, RS4] (target), with RS2 and RS4 are a beam pair

The offset of beam pair in T2 is not the same of the offset in T1.

- Sub-topic 4-2: Accuracy requirements

Sub-topic description:

Open issues and candidate options before f2f meeting:

#### Issue 2-10: Accuracy requirements for multi-Rx in Rel-18

- Proposals

  - Option 1a: (CATT, CMCC)

    - The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting.

  - Option 1b: (Ericsson, vivo, ZTE)

    - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1.

    - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

  - Option 1c: (Samsung)

    - Under multi-Rx chain DL reception scenario, the legacy accuracy requirements for L1-RSRP measurement should be satisfied for both Rx chains.

    - In this sense, no new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation

  - Option 2: (Qualcomm)

    - Do not define group-based L1-RSRP measurement accuracy requirements unless the requirements are applicable only when there are negligible mutual interferences across different pair of Tx-Rx beams.

- Recommended WF

  - Further discuss.

- Topic #1: Core part maintenance

- Sub-topic 1-1: General aspects

#### Issue 1-2: UE capability for multiRx-FR2-Preference-r18

- Proposals

  - Option 1: (MTK)

    - Add a feature group (30-3) to indicate whether the UE supports providing multi-Rx operation preference for FR2, as already captured in RAN2.

- Recommended WF

  - If option 1 is agreeable?

#### Issue 1-3: Power class for multi-Rx UE capability

- Proposals

  - Option 1: (Agreement in the last meeting)

    - All power classes except PC6

  - Option 2: (Huawei)

    - Only power class 3

- Recommended WF

  - Further discuss.

#### Issue 1-4: Prerequisite features for FG 30-1

- Proposals

  - Option 1: (vivo)

    - 16-2c, 23-5-1

  - Option 2: (ZTE)

    - 16-2c, 23-5-1, 16-2a or 16-2b-1

  - Option 3:

    - 16-2c, 23-5-1, at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3

- Recommended WF

  - If option 1 is agreeable?

#### Issue 1-8: Applicable scenarios of Rel-18 multi-Rx feature

- Proposals

  - Option 1a: (Nokia)

    - Rel-18 multi-Rx related requirements are applicable to PCell, PSCell, and SCell.

  - Option 1b: (QC)

    - Rel-18 multi-Rx related requirements are applicable to PCell, PSCell, and SCell as long as the cell is the only serving cell in the concerned FR2 band

- Recommended WF

  - Further discuss

#### Issue 1-9: Whether and how to capture multi-Rx operation is activated

- Proposals

  - Option 1: (Nokia)

    - No need to capture in RAN4 RRM requirements whether Multi Rx is activated.

  - Option 2: (CATT)

    - The UE is activated with multi-Rx operation when the following conditions are met:

      - The simultaneousReceptionDiffTypeD-r16 is configured as ‘supported’ if separate PDSCHs are transmitted from different TRPs, or

      - UE is requested by gNB to report its preference for multi-Rx operation in FR2 via RRC signalling and the multiRx-PreferenceFR2 is NOT included in the UEAssistanceInformation, and

      - The CSI-RS/SSB resources are partially or fully overlapped with the PDSCH or other CSI-RS/SSB resources in time domain for simultaneous reception, and

      - The TCI states for CSI-RS/SSB resources or PDSCH reception are configured with different QCL Type-D.

    - The requirements that UE is activated with multi-Rx operation can be captured in Clause 3.6 in TS 38.133 as applicability requirements for multi-Rx requirements.

  - Option 3: (Apple)

    - A UE is in multi-RX operation or activated with multi-RX operation if the following conditions are met:

      - The UE reports it support capabilities 16-2c(simultaneousReceptionDiffTypeD-r16) and 23-5-1 (mTRP-GroupBasedL1-RSRP-r17), and

      - UE has most recently indicated to the network its preference of multi-RX operation while in RRC_CONNECTED mode, or

      - UE has not indicated it prefers single-RX operation while in RRC_CONNECTED mode.

- Recommended WF

  - Further discuss.

- References

- R4-2400741	Topic summary for [110][205] FR2_multiRx_part1, Moderator (vivo)

- R4-2400742	Topic summary for [110][206] FR2_multiRx_part2, Moderator (Ericsson)