3GPP TSG-RAN WG4 Meeting # 110												R4-24xxxxx

Athens, GR, 26 February – 01 March, 2024

Agenda item:			8.14.4

Source:	Moderator (Ericsson)

Title:	Ad hoc #2 minutes for NR Rel-18 Positioning Enhancements WI

Document for:	Approval

# Introduction

The document contains discussion related to the positioning RRM core requirements across the following 3 threads:

- Topic # 1: core [110][217] NR_pos_enh2_part1 (was not discussed on Wednesday/Feb29)

- Topic # 2: core [110][218] NR_pos_enh2_part2 (Wednesday/Feb28)

- Topic # 3: core [110][219] NR_pos_enh2_part3 (Wednesday/Feb28)

- Topic # 7: core daft CRs for all threads (Wednesday/Feb28)

- Topic #9: draft LS out (Wednesday/Feb28)

The document contains discussion related to the positioning RRM performance across the following 3 threads:

- Topic # 4: perf [110][217] NR_pos_enh2_part1 (RedCap: Wednesday/Feb28; BW aggregation: was not discussed on Wednesday/Feb28)

- Topic # 5: perf [110][218] NR_pos_enh2_part2 (partly discussed on Wednesday/Feb28)

- Topic # 6: perf [110][219] NR_pos_enh2_part3 (was not discussed on Wednesday/Feb29)

- Topic # 8: perf draft CRs for all threads (Wednesday/Feb28)

# Topic #1: core [110][217] NR_pos_enh2_part1

## RedCap Positioning (Agenda 8.14.2.4)

Issues prioritized during the AH: -

## PRS/SRS bandwidth aggregation (Agenda 8.14.2.5)

Issues prioritized during the AH: -

# Topic #2: core [110][218] NR_pos_enh2_part2

## Sidelink Positioning (Agenda 8.14.2.2)

Issues prioritized during the AH: -

## Carrier Phase Positioning (Agenda 8.14.2.6)

Issues recommended for discussion by moderator: 2-1-1 Q2 and Q3 (Q1 was discussed in the Tuesday ad hoc), 2-1-2, 2-1-3

Issues prioritized during the AH: 2-1-1 (Q2 and Q3), 2-1-2

Issue 2-1-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD:

RAN4#109 agreements: Agreements: - When LMF requests the UE to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) for a PFL, and UE supports FG 41-2-3, the Twindow needs to be considered in the measurement period: - For periodic time window, - Adopt the following updates to the existing measurement period requirements for the PFL - Tavailable is defined as LCM(Tprs, MGRP, Twindow), where Twindow is the maximum periodicity of the indicated time window(s) - When calculating Lprs and Tprs, only the PRS resources in the indicated resources sets and overlapped with both the MG and the indicated time window(s) are considered. - For one-shot time window case, - Lavailable_PRS,i only counts PRS resource instances within the indicated time window(s). - the start of the measurement period is the start of the time window. RAN4#110 agreements (ad hoc 1 on Tuesday/Feb 27): - Define CPP measurement requirements with multiple PFLs used for legacy measurements and CPP measurements done on a single PFL

Proposals

Clarifications on the requirements with multiple PFLs:

- Option 1a: (Huawei, Ericsson)

  - Define CPP measurement requirements with multiple PFLs

- Option 1b: (Qualcomm)

  - When DL RSCPD is reported with RSTD and DL RSCPD is measured in PFL j with periodic time windows, the measurement period is given by

    - TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

  - FFS how to adapt the requirement for aperiodic time window

  - When DL RSCP is reported with UE Rx-Tx and DL RSCP is measured in PFL j with periodic time windows, the measurement period is given by

    - TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

  - FFS how to adapt the requirement for aperiodic time window

Clarifications on the support of indicated time window for CPP measurement:

- Option 2a: (Huawei)

  - When LMF does not request the UE to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) for a PFL, or UE does not support FG 41-2-3, existing RSTD/UE Rx-Tx measurement period requirements apply.

- Option 2b: (Nokia)

  - RAN4 to inquire from RAN1 the condition(s) for applying the two-time window configuration at UE side for CPP.

  - RAN4 to investigate the RRM impact to separate / joint measurement reporting from the two-time-windows configuration for DL CPP in terms of latency.

  - RAN4 to investigate the RRM impact to measurement period definition from the two-time-windows configuration for DL CPP in terms of CSSF adjustments.

- Option 2c: (CATT)

  - Do not define the requirements for one shot time window.

Clarifications on the support of indicated time window for legacy measurement:

- Option 3a: (Qualcomm)

  - When DL RSCPD is reported with RSTD,

    - If the UE supports FG 41-2-8 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

    - If the UE does not support FG 41-2-8 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

    - If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

  - When DL RSCP is reported with UE Rx-Tx,

    - If the UE supports FG 41-2-9 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

    - If the UE does not support FG 41-2-9 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

    - If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

- Option 3b: (Huawei)

  - When LMF does not request the UE to perform legacy measurements inside the time window only, R17 measurement period requirements should apply. UE is not required to report CP measurement if the resource set(s) associated to the window have occasions outside the time window.

- Recommended WF:

Discuss in the meeting:

  - Q1: Clarifications on the requirements with multiple PFLs:

  - Q2: Clarifications on the support of indicated time window for CPP measurement:

  - Q3: Clarifications on the support of indicated time window for legacy measurement:

Discussion:

Q2: When LMF does not configure measurement time window(s) for a PFL or UE does not support FG 41-2-3 (Measurement on indicated DL PRS resource sets within the indicated time window(s) for UE based and UE assisted):

- For a single PFL: existing requirements without time window apply.

- FFS: When multiple PFLs are configured for legacy measurements.

Q3:

FFS: For UE configured for DL RSCPD with RSTD, when [LMF does not request the UE to perform legacy measurements in the measurement time window or] UE does not support FG 41-2-8 (Support to perform legacy measurements inside the indicated time window only for DL TDoA):

- For a single PFL: existing requirements ignoring time window apply for legacy measurements, [FFS requirements for RSCPD].

- FFS: When multiple PFLs are configured for legacy measurements.

FFS: For UE configured for DL RSCP with UE Rx-Tx, when [LMF does not request the UE to perform legacy measurements in the measurement time window or] UE does not support FG 41-2-9 (Support to perform legacy measurements inside the indicated time window only for multi-RTT):

- For a single PFL: existing requirements ignoring time window apply for legacy measurements, [FFS requirement for RSCP].

- FFS: When multiple PFLs are configured for legacy measurements.

Issue 2-1-2: Clarification on the measurement reporting requirements

Proposals

- Proposal 1: (Nokia)

  - RAN4 to adjust the measurement reporting requirements for CPP such that up to 2 RSCPD/DL RSCP measurements in case of reduced latency and else up to 4 RSCPD/DL RSCP measurements are reported along with 1 RSTD/UE Rx-Tx time difference measurement to LMF.

Recommended WF:

- Discuss the proposal.

Discussion:

- RAN4 to adjust the measurement reporting requirements for CPP such that up to 2 RSCPD/DL RSCP measurements in case of reduced latency (case 1) and else up to 4 RSCPD/DL RSCP measurements (case 2) are reported along with 1 RSTD/UE Rx-Tx time difference measurement to LMF.

FFS: Side conditions for each case

Number of samples: 1 for accuracy requirements, upon configuration for measurement period requirements.

Issue 2-1-3: The impact of carrier frequency offset

Background:

Agreements:

- No consensus in RAN4 on whether or not to specify UE behavior or requirements related to measurement of carrier frequency offset in Rel-18 core part. RAN4 can close the WI core part without further agreement on this issue.

- FFS whether and how to account for carrier frequency offset in the accuracy requirements in performance part.

Proposals

- Proposal 1: (Lenovo)

  - Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

  - Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

  - Define the same common reference time for the UE and the PRU.

  - The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements.

  - Above proposals are equally applicable to remove the errors due to carrier frequency offsets when the UE and PRU take measurements in different slots.

- Proposal 2: (Nokia)

  - RAN4 to specify measures for mitigating the impact due to carrier frequency offsets of TRP, UE and PRU.

  - RAN4 to specify a common reference time for RSCP and RSCPD measurement, respectively, between UE and PRU for same TRP as well as for UE for different TRPs in TS 38.133. The reference time can be FFS, e.g. start of slot where the PRS is transmitted, or start of the measurement gap occasion or start of the configured time window for the CP measurement.

  - RAN4 to send an LS to RAN1 informing them that the carrier frequency offset compensation may require a specified UE behavior which will impact RAN1 specs.

Recommended WF:

- Discuss the proposals.

Discussion:

# Topic #3: core [110][219] NR_pos_enh2_part3

## LPHAP use case (Agenda 8.14.2.3)

Issues recommended for discussion by moderator: 1-1-1

Issues prioritized during AH: 1-1-1

Issue 1-1-1: Start of PRS measurement

- Background

  - Based on agreement in RAN4#108 (R4-2314462), there are two cases for PRS measurement with RAN eDRX > 10.24s

    - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity. In this case, UE starts positioning measurement within PTW.

    - Case 2: eDRX cycle is longer than configured PRS measurement reporting periodicity. In this case, positioning measurement start is not limited to PTW.

  - Open issue from RAN4#109 (R4-2321525)

    - For Case 1, whether positioning measurement start is limited to PTW when PRS resource are not within PTW

- Proposals

  - Option 1 (CATT, OPPO, HW, E///, Nokia):

    - For Case 1, PRS measurement start is not limited to PTW when the PRS resource indicated in the assistance data is not within PTW

  - Option 2 (QC):

    - For Case 1, the PRS measurement period with RAN eDRX > 10.24s does not apply if there are no PRS resource instances within the PTWs

- Recommended WF

  - Discuss the options

Discussion:

For Case 1, PRS measurement start is not limited to PTW when the PRS resource indicated in the assistance data is not within PTW.

# Topic #4: perf [110][217] NR_pos_enh2_part1

## RedCap Positioning (Agenda 8.14.3.3)

Issues recommended for discussion by moderator: 5-1-1, 5-1-2, 5-1-3

Issues prioritized during the AH: 5-1-1, 5-1-2, 5-1-3

Issue 5-1-1: Side conditions and channel models for RSTD and UE Rx-Tx measurements for 1Rx UE without Rx FH

- Proposals

  - Option 1: CATT, CMCC, QC, E///

    - The brackets in the side conditions for 1Rx without FH under fading propagation condition can be removed, i.e., the side conditions should be:

      - RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, -10, -10) dB.

      - UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB.

- Recommended WF

  - Discuss above option(s).

Discussion:

The brackets in the side conditions for 1Rx without FH under fading propagation condition can be removed, i.e., the side conditions should be:

RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values of -6 dB for the reference cell and -10 dB for the target cell and is based on 4 samples.

UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values of -3 dB and -10 dB and is based on 4 samples.

Issue 5-1-2: Side conditions and channel models for RSRP measurements for 1Rx UE without Rx FH

- Proposals

  - Option 1: CATT, CMCC

    - PRS-RSRP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB.

  - Option 2: QC, E///

    - RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

- Recommended WF

  - Discuss above option(s).

Discussion:

PRS-RSRP accuracy requirements for 1 Rx RedCap UE without Rx FH:

Side conditions: -3 dB, -10 dB,

4 samples,

Generic requirements, not limited to specific propagation conditions.

Issue 5-1-3: Side conditions and channel models for RSRPP measurements for 1Rx UE without Rx FH

- Proposals

  - Option 1: CATT, CMCC

    - PRS-RSRPP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB.

  - Option 2: QC, E///

    - RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17 with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

- Recommended WF

  - Discuss above option(s).

Discussion:

PRS-RSRPP accuracy requirements for 1 Rx RedCap UE without Rx FH:

Side conditions: -3 dB, -10 dB,

2-tap channel model from Rel-17 with Nsamples = 4.

## PRS/SRS bandwidth aggregation (Agenda 8.14.3.4)

Issues recommended for discussion by moderator: 6-2-3, 6-2-4

Issues prioritized during the AH:

Issue 6-2-3: Bandwidth configuration to define accuracy requirements for positioning measurements based on bandwidth aggregation

- Proposals

  - Option 1: CATT

    - Some of the possible maximum aggregated bandwidth in the latest RAN1 UE features list can be chosen to define accuracy requirements.

  - Option 2: HW

    - For PRS CA, RAN4 to use the following per PFL BW as starting point to define accuracy requirements for RSTD and UE Rx-Tx.

      - 15kHz SCS: 104 (20M)

      - 30kHz SCS: 132 (50M), 272 (100M)

      - 60kHz SCS FR1: 64 (50M), 132 (100M)

      - 60kHz SCS FR2: 64 (50M), 132 (100M)

      - 120kHz SCS: 64 (100M), 128 (200M)

- Recommended WF

  - Discuss the option(s) above.

Possible WF: Option 2?

Discussion:

Issue 6-2-4: Separate accuracy requirement for positioning measurements based on bandwidth aggregation depending on the number of PFLs

- Proposals

  - Option 1: CATT

    - No need to differentiate the number of aggregated PFLs when defining accuracy requirements, i.e., one accuracy requirement table can be shared for both 2 aggregated PFLs and 3 aggregated PFLs.

  - Option 2: CMCC, HW

    - PRS/SRS BW aggregation, it is proposed to define accuracy requirements for both 2 PFLs case and 3 PFLs case

- Recommended WF

  - Discuss the option(s) above.

Possible WF:

Requirements shall cover 2 and 3 PFLs.

The same accuracy requirements apply for 2 PFLs and 3 PFLs

Discussion:

# Topic #5: perf [110][218] NR_pos_enh2_part2

## Sidelink Positioning (Agenda 8.14.3.1)

Issues recommended for discussion by moderator: 1-2-1, 1-2-2, 1-2-3, 1-2-4, 1-2-7, 1-2-8

Issues prioritized during the AH: 1-2-9

Issue 1-2-1: Measurement types to be defined accuracy requirements

Proposals:

- Option 1: (CATT, Qualcomm)

  - Do not define accuracy requirements for SL PRS based RTOA and AOA/ZOA measurement in Rel-18.

- Option 2: (CMCC, Ericsson)

  - Define accuracy requirements for SL-PRS based RSTD, SL-PRS based UE Rx-Tx time difference, SL-PRS based RSRP and SL-PRS based RSRPP. FFS for SL-PRS based AoA/ZOA, SL-PRS based RTOA.

- Option 3: (vivo)

  - RAN4 to define accuracy requirements for SL RTOA measurement. The requirements and side conditions for SL Rx-Tx measurement are reused.

  - RAN4 not to define accuracy requirements for SL AOA/ZOA measurement.

  - RAN4 only to define relative SL RSRP and SL RSRPP accuracy requirements.

  - Define absolute accuracy requirement for SL PRS-RSSI measurement and the existing intra-frequency SL RSSI measurement accuracy requirement for FR1 can be reused.

- Option 4: (Huawei)

  - RAN4 to define accuracy requirements for SL RSTD, SL Rx-Tx, SL PRS-RSRP, SL PRS-RSRPP. RAN4 not to define accuracy requirements for SL RTOA. FFS whether RAN4 to define accuracy requirements for SL AoA/ZoA.

Recommended WF (moderator):

- Define accuracy requirements for SL-PRS based RSTD, SL-PRS based UE Rx-Tx time difference, SL-PRS based RSRP and SL-PRS based RSRPP.

- Further discuss the following Proposals:

  - Do not define accuracy requirements for SL PRS based RTOA and AOA/ZOA measurement in Rel-18.

  - RAN4 only to define relative SL RSRP and SL RSRPP accuracy requirements.

  - Define absolute accuracy requirement for SL PRS-RSSI measurement and the existing intra-frequency SL RSSI measurement accuracy requirement for FR1 can be reused.

Discussion:

Issue 1-2-2: Assumptions to define SL PRS measurement accuracy requirements

Proposals:

- Proposal 1: (Qualcomm)

  - Define measurement accuracy requirements independent of SL PRS comb size.

  - Define one set of measurement accuracy requirements applicable to fully staggered and partially staggered (half comb) SL PRS.

  - For measurement accuracy requirements with multiple samples (Nsample = 4), do not assume coherent combining between samples.

  - Define SL positioning measurement accuracy requirements for measurements derived from SL PRS transmissions/receptions on a single carrier and single numerology.

- Proposal 2: (Huawei)

  - Accuracy requirements for SL PRS measurements are defined based on

    - Channel: as their Uu counterparts

    - Es/Iot: -6dB for target UE

    - Nsample as defined in core requirements

Recommended WF:

- Discuss in the meeting.

Discussion:

Issue 1-2-3: SINR side condition

Proposals:

- For SL RSTD,

  - Option 1: (CATT)

    - [-3, -6]dB for reference UE and target UE.

  - Option 2: (Qualcomm)

    - [0, -3]dB for reference UE and target UE.

  - Option 3: (vivo, Ericsson)

    - [0, -6]dB for reference UE and target UE.

  - Option 4: (Huawei)

    - -6dB for target UE

- For SL Rx-Tx/SL PRS RSRP/RSRRPP,

  - Option 1: (CATT, Huawei, Ericsson)

    - [-6]dB.

  - Option 2: (Qualcomm)

    - Two sets of side conditions 0 dB and -3 dB.

  - Option 3: (vivo)

    - Two sets of side conditions 0 dB and -6 dB.

- For SL AoA/ SL RTOA,

  - Option 1: (Ericsson)

    - The requirements apply down to -6 dB, regardless of whether accuracy requirements are specified or not.

Recommended WF:

Discuss in the meeting.

Discussion:

Issue 1-2-4: SL RSTD measurement accuracy requirements

Proposals:

- Proposal 1: (CATT)

  - Take table 1 as the structure for SL RSTD accuracy requirements.

- Table 1: SL RSTD accuracy in FR1

| Accuracy | SL PRS Ês/Iot | SL PRS SCS | SL PRS bandwidth | Nsample |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | RB |  |  |
| [TBD] | (Ês/Iot)ref ≥TBD (Ês/Iot)i ≥TBD | 15 | ≥ 24 | 4 |  |
|  |  |  | ≥ 48 | 1 |  |
| [TBD] |  | 30 | ≥ 24 | 4 |  |
|  |  |  | ≥ 48 | 1 |  |
| [TBD] |  | 60 | ≥ 24 | 4 |  |
|  |  |  | ≥ 48 | 1 |  |

- Proposal 2: (Qualcomm)

  - Define the SL RSTD absolute measurement accuracy requirement as ±(X+Y+Z) Tc, where

    - X is the simulated measurement accuracy for a given propagation condition and number of measurement samples,

    - Y is the frequency/clock drift margin,

    - Z is the RF calibration margin.

  - Define the RF calibration margin for SL RSTD measurements in FR1 using the following structure:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | Z1 |
| ≥ 48 | ≥ 24 | N/A | Z2 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z3 |

- Proposal 3: (Huawei)

  - Accuracy requirements for SL PRS measurements are defined for following RB numbers

    - 15kHz SCS: 48, 96

    - 30kHz SCS: 24, 48, 96

    - 60kHz SCS: 24, 48

- Proposal 4: (Ericsson)

  - Two sets of accuracy requirements are to be defined for:

    - 1 sample for SL-PRS BW>48 PRBs,

    - 4 samples for 24 PRBs ≤SL-PRS BW≤48 PRBs.

Recommended WF:

- Discuss in the meeting.

Possible WF:

Accuracy requirements are to be defined for both 1 sample and 4 samples, for the respective BWs.

Discussion:

Issue 1-2-7: Test case list

Proposals:

- Proposal 1: (CATT, CMCC, Qualcomm, vivo, Huawei, Ericsson)

  - Define measurement delay test cases for SL RSTD, SL RTOA, SL Rx-Tx and SL-AoA/ZoA

- Proposal 2: (CATT, CMCC, vivo, Ericsson)

  - Define measurement delay test cases for SL RSRP and SL RSRPP.

- Proposal 3: (Qualcomm)

  - Define measurement accuracy test cases for SL RSTD, SL Rx-Tx, SL PRS-RSRP (reported with SL RSTD or Rx-Tx) and SL PRS-RSRPP (reported with SL RSTD or Rx-Tx).

- Proposal 4: (Huawei)

  - RAN4 to define the following TCs for SL positioning.

    - Accuracy TCs: SL RSTD + PRS-RSRP, SL Rx-Tx + PRS-RSRP

Recommended WF:

- Define measurement delay test cases for SL RSTD, SL RTOA, SL Rx-Tx and SL-AoA/ZoA.

- Define measurement accuracy test cases for SL RSTD, SL Rx-Tx, SL PRS-RSRP (reported with SL RSTD or Rx-Tx).

- Further discuss whether to define delay test cases for SL RSRP and SL RSRPP.

- Further discuss whether to define accuracy test cases for SL RSRP and SL RSRPP.

Discussion:

Issue 1-2-8: Test configurations

Proposals:

- Proposal 1: (Qualcomm)

  - Define SL positioning test cases using only AWGN and 2-tap channel (for SL PRS RSRPP) propagation conditions.

- Proposal 2: (Qualcomm)

  - Define SL positioning test cases for a single carrier and single resource pool.

- Proposal 3: (Qualcomm)

  - Define SL PRS configurations for SL positioning test cases that apply to both shared and dedicated resource pools

    - No TDM of SL PRS from different UEs in the same slot

    - No comb-based multiplexing of SL PRS from different UEs in the same slot

    - No FDM of SL PRS from different UEs in the same slot

- Proposal 4: (Huawei)

  - RAN4 to consider the following scope for the test.

    - Resource pool: shared and dedicated

    - Coverage: in coverage and gNB as sync reference

- Proposal 5: (Huawei)

  - RAN4 to consider the following SL PRS related parameters for the test.

    - (symbol num, comb size): (4, 4) and (2, 4)

    - BW: 48 RB for delay test

    - Es/Iot: 3dB (to ensure 100% PSCCH decoding)

- Proposal 6: (Huawei)

  - RAN4 to consider the following SL PRS transmission pattern for the delay test.

    - TX UE1: slot n

    - TX UE2: slot n + 1 and slot n + 100ms

- Proposal 7: (Qualcomm)

  - Do not define SL positioning test cases with additional path reporting.

  - Do not define SL positioning test cases with LoS/NLoS reporting.

  - Do not define SL positioning test cases with Tx/Rx ARP-ID reporting.

- Proposal 8: (vivo)

  - Define test cases for one SL UE and dual SL UEs.

- Proposal 9: (Ericsson)

  - For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements cover both:

    - SL-PRS BW>48 PRBs, and

    - 24 PRBs ≤SL-PRS BW≤48 PRBs.

Recommended WF:

- Discuss in the meeting.

Discussion:

Issue 1-2-9: Specification structure

Proposals:

- Proposal 1: (Ericsson)

  - A new separate section is created for SL positioning measurement performance in clause 10, e.g.:

    - 10.4A NR Sidelink Measurements for Positioning.

Recommended WF:

- Discuss in the meeting.

Discussion:

Based on the discussion of draft CR in R4-2402803: 10.4A is to be used for NR SL measurement accuracy and measurement report mapping

## Carrier Phase Positioning (Agenda 8.14.3.5)

Issues recommended for discussion by moderator: 2-2-1, 2-2-2, 2-2-3, 2-2-4, 2-2-7, 2-2-8

Issues prioritized during the AH:

Issue 2-2-1: Channel model for accuracy requirements

Proposals

- Proposal 1: (CATT, Huawei, Ericsson, QC)

  - Define accuracy requirements for DL RSCPD measurement and relative DL RSCP under AWGN channel.

- Proposal 2: (CATT, Ericsson)

  - Define accuracy requirements for DL RSCPD measurement and relative DL RSCP for Two-Tap channel.

- Proposal 3: (CATT, Nokia)

  - Define accuracy requirements for DL RSCP and RSCPD for Two-Tap channel based on the 	simulation results.

  - For the companion measurements, i.e. UE Rx-Tx time difference and RSTD, the existing 	accuracy requirements for fading channel are reused.

Recommended WF:

- Define accuracy requirements for DL RSCPD measurement and relative DL RSCP under AWGN channel.

- Further discuss whether to define accuracy requirements for DL RSCPD measurement and relative DL RSCP under Two-Tap channel.

Discussion:

Ok with the recommended WF?

Issue 2-2-2: Side condition

Proposals

- For DL RSCPD measurement:

  - Option 1: (CATT)

    - Two sets of side conditions: [-6, -13]dB and [-3, -6]dB

  - Option 2: (Huawei)

    - [-3, -6]dB

  - Option 3: (Ericsson, Nokia)

    - [-6, -13] dB if RSTD measurement is done over 4 samples

- For relative DL RSCP measurement:

  - Option 1: (CATT)

    - Two sets of side conditions: [-6, -13]dB and [-3, -6]dB

  - Option 2: (Huawei)

    - [-3, -6]dB

  - Option 3: (Nokia)

    - Two sets of side conditions: [-3, -13]dB and [0, -6]dB

Recommended WF:

- Discuss in the meeting.

Discussion:

Issue 2-2-3: DL RSCPD absolute accuracy requirements

Proposals

- Proposal 1: (CATT)

  - Define DL RSCPD accuracy requirements based on the following structure:

Table 1: DL RSCPD absolute accuracy in FR1 for AWGN channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 |
| --- | --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |  |
| [TBD] | (Ês/Iot)ref ≥-6dB (Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 |
| [TBD] |  |  | ≥ 52 | ≥ 1 |
| [TBD] |  |  | ≥ 104 | ≥ 1 |
| [TBD] |  | 30 | ≥ 24 | ≥ 4 |
| [TBD] |  |  | ≥ 48 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |
| [TBD] |  | 60 | ≥ 24 | ≥ 4 |
| [TBD] |  |  | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 15 | ≥ 52 | ≥ 1 |
| [TBD] |  |  | ≥ 104 | ≥ 1 |
| [TBD] |  | 30 | ≥ 48 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |
| [TBD] |  | 60 | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |

Table 2: DL RSCPD absolute accuracy in FR2 for AWGN channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 |
| --- | --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |  |
| [TBD] | (Ês/Iot)ref ≥-6dB (Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 |
| [TBD] |  |  | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |
| [TBD] |  | 120 | ≥ 32 | ≥ 4 |
| [TBD] |  |  | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 128 | ≥ 1 |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 60 | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 132 | ≥ 1 |
| [TBD] |  | 120 | ≥ 64 | ≥ 1 |
| [TBD] |  |  | ≥ 128 | ≥ 1 |

- Proposal 2: (CATT, CMCC, Huawei, Ericsson, Nokia)

  - Accuracy requirements for DL RSCPD and relative DL RSCP are defined using same RB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements.

Recommended WF:

- Accuracy requirements for DL RSCPD and relative DL RSCP are defined using same RB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements.

Discussion:

Related to issue 2-2-2?

Issue 2-2-4: DL RSCP relative accuracy requirements

Proposals

- Proposal 1: (CATT)

  - Reuse DL RSCPD accuracy requirements.

Recommended WF:

- Discuss the Proposal.

Discussion:

Related to issue 2-2-2 and 2-2-3?

Issue 2-2-6: Report mapping

Proposals

- Proposal 1: (Ericsson)

  - DL-RSCPD measurement report mapping table is defined as:

| Reported value | Measured quantity value (DL-RSCPD) | Unit |
| --- | --- | --- |
| DL-RSCPD_0000 | -180 ≤ DL-RSCPD < -179.9 | Degree |
| DL-RSCPD_0001 | -179.9 ≤ DL-RSCPD < -179.8 | Degree |
| DL-RSCPD_0002 | -179.8 ≤ DL-RSCPD < -179.7 | Degree |
| … | … | … |
| DL-RSCPD_1798 | -0.2 ≤ DL-RSCPD < -0.1 | Degree |
| DL-RSCPD_1799 | -0.1 ≤ DL-RSCPD < 0 | Degree |
| DL-RSCPD_1800 | 0 ≤ DL-RSCPD < 0.1 | Degree |
| DL-RSCPD_1801 | 0.1 ≤ DL-RSCPD < 0.2 | Degree |
| DL-RSCPD_1802 | 0.2 ≤ DL-RSCPD < 0.3 | Degree |
| … | … | … |
| DL-RSCPD_3598 | 179.8 ≤ DL-RSCPD < 179.9 | Degree |
| DL-RSCPD_3599 | 179.9 ≤ DL-RSCPD < 180 | Degree |

  - DL-RSCP measurement report mapping table is defined as:

| Reported value | Measured quantity value (DL-RSCP) | Unit |
| --- | --- | --- |
| DL-RSCP_0000 | 0 ≤ DL-RSCP < 0.1 | degree |
| DL-RSCP_0001 | 0.1 ≤ DL-RSCP < 0.2 | degree |
| DL-RSCP_0002 | 0.2 ≤ DL-RSCP < 0.3 | degree |
| … | … | … |
| DL-RSCP_3598 | 359.8 ≤ DL-RSCP < 359.9 | degree |
| DL-RSCP_3599 | 359.9 ≤ DL-RSCP < 360 | degree |

  - UL-RSCP measurement report mapping table is defined as:

| Reported value | Measured quantity value (UL-RSCP) | Unit |
| --- | --- | --- |
| UL-RSCP_0000 | 0 ≤ UL-RSCP < 0.1 | degree |
| UL-RSCP_0001 | 0.1 ≤ UL-RSCP < 0.2 | degree |
| UL-RSCP_0002 | 0.2 ≤ UL-RSCP < 0.3 | degree |
| … | … | … |
| UL-RSCP_3598 | 359.8 ≤ UL-RSCP < 359.9 | degree |
| UL-RSCP_3599 | 359.9 ≤ UL-RSCP < 360 | degree |

Recommended WF:

- Agree Proposal 1 which is based on the previous agreements.

Discussion:

Proposal 1 agreeable?

Issue 2-2-7: Test case list

Proposals

- Proposal 1: (CATT, Huawei)

  - RAN4 to define TCs for CPP based on following combinations

    - RSTD with RSCPD, UE Rx-Tx with RSCP

    - RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE

    - FR1, FR2

    - Delay, accuracy

- Proposal 2: (Ericsson)

  - RAN4 to discuss if the test cases to validate UE capability to meet the core requirement for carrier phase measurement needs to be defined.

  - RAN4 to define test cases to validate UE capability to meet the performance requirement defined for carrier phase measurement.

Recommended WF:

- Discuss the test case list based on the following table:

Table 1: Test list for CPP measurement

| Set | PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| 0-1 | General: Configuration for the indicated time window |  |
| PRS measurement delay tests |  |  |
| 1-1 | DL RSCP reporting delay test case in FR1 in RRC_CONNECTED |  |
| 1-2 | DL RSCPD reporting delay test case in FR1 in RRC_CONNECTED |  |
| 1-3 | DL RSCP reporting delay test case in FR2 in RRC_CONNECTED |  |
| 1-4 | DL RSCPD reporting delay test case in FR2 in RRC_CONNECTED |  |
| 1-5 | DL RSCP reporting delay test case in FR1 in RRC_INACTIVE |  |
| 1-6 | DL RSCPD reporting delay test case in FR1 in RRC_INACTIVE |  |
| 1-7 | DL RSCP reporting delay test case in FR2 in RRC_INACTIVE |  |
| 1-8 | DL RSCPD reporting delay test case in FR2 in RRC_INACTIVE |  |
| PRS measurement accuracy tests |  |  |
| 2-1 | DL RSCPD absolute accuracy test case in FR1 in RRC_CONNECTED |  |
| 2-2 | DL RSCP relative accuracy test case in FR1 in RRC_CONNECTED |  |
| 2-3 | DL RSCPD absolute accuracy test case in FR2 in RRC_CONNECTED |  |
| 2-4 | DL RSCP relative accuracy test case in FR2 in RRC_CONNECTED |  |
| 2-5 | DL RSCPD absolute accuracy test case in FR1 in RRC_INACTIVE |  |
| 2-6 | DL RSCP relative accuracy test case in FR1 in RRC_INACTIVE |  |
| 2-7 | DL RSCPD absolute accuracy test case in FR2 in RRC_INACTIVE |  |
| 2-8 | DL RSCP relative accuracy test case in FR2 in RRC_INACTIVE |  |

Discussion:

Issue 2-2-8: Test configurations

Proposals

- Proposal 1: (CATT)

  - Define DL RSCP and DL RSCPD delay test cases in the existing UE Rx-Tx and RSTD clauses.

- Proposal 2: (Huawei)

  - RAN4 to define the tests for CPP with periodic time window

    - Periodicity and offset: 2 times of PRS resource periodicity

    - Offset: same as PRS resource offset

    - Duration: covering all PRS resources from all TRPs

Recommended WF:

- Discuss in the meeting.

Discussion:

# Topic #6: perf [110][219] NR_pos_enh2_part3

## LPHAP use case (Agenda 8.14.3.2)

Issues recommended for discussion by moderator: 1-2-2, 1-2-3, 1-2-4.

Issues prioritized during AH:

Issue 1-2-1: Accuray requirements

- Proposals

  - Option 1 (CATT, CMCC, HW, E///):

    - Existing accuracy requirements are applicable for PRS measurement in INACTIVE with eDRX, and PRS measurement in IDLE.

  - Option 2 (Nokia):

    - RAN4 to discuss whether to reuse the measurement accuracy requirements in Rel-17 or whether to define a new table based on measurement with two samples.

- Recommended WF

  - Suggests to agree on option 1 based on majority view.

Discussion:

Option 1 agreeable?

Issue 1-2-2: RRM testing for PRS measurement

##### Question 1: whether to define test cases for measurement accuracy

- Proposals

  - Option 1 (CATT):

    - No.

  - Option 2 (HW):

    - Yes, for PRS measurement in RRC_IDLE

- Recommended WF

  - Discuss the options

Discussion:

##### Question 2: for which measurement types to define RRC test cases

- Proposals

  - Option 1 (CATT):

    - Define test cases for RSTD, PRS RSRP and PRS RSRPP measurement in RRC_IDLE state and for UE Rx-Tx time difference in RRC_INACTIVE state.

- Recommended WF

  - Discuss option 1

Discussion:

Measurements delay TCs for PRS RSTD/RSRP/RSRPP in RRC_IDLE and UE Rx-Tx in RRC_INACTIVE?

##### Question 3: what eDRX configurations are to be tested

- Background

  - For RAN eDRX > 10.24s, we have 2 cases for PRS measurement

    - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity. In this case, UE starts positioning measurement within PTW.

    - Case 2: eDRX cycle is longer than configured PRS measurement reporting periodicity. In this case, positioning measurement start is not limited to PTW.

- Proposals

  - Option 1 (CATT, CMCC):

    - Both RAN eDRX ≤ 10.24s and RAN eDRX ＞ 10.24s

    - [CMCC] For RAN eDRX <= 10.24s, both CN eDRX <= 10.24 s and CN eDRX >10.24 s

  - Option 2 (HW):

    - Case 2 for RAN eDRX ＞ 10.24

  - Option 3 (E///):

    - Both Case 1 and Case 2 for RAN eDRX ＞ 10.24

- Recommended WF

  - Discuss the options

  - The issue needs to be discussed for both RRC_INACTIVE and RRC_IDLE

Discussion:

Discuss with other TC parameters?

##### Question 4: what PRS measurement features are to be tested

- Proposals

  - Option 1 (CMCC):

    - Reduced number of PRS samples

    - parallelPRS-MeasRRC-Inactive-r17 capability

    - Lower Rx beam sweeping in FR2

  - Option 2 (HW):

    - 4 sample baseline

- Recommended WF

  - Discuss the options

Possible WF:

4 sample baseline

…

Discussion:

Issue 1-2-3: RRM testing for other requirements

##### REQ 1: RRM measurement (cell reselection)

- Background

  - For RAN eDRX > 10.24s, we have we have 2 cases for RRM measurement

    - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity (if PRS measurement is configured) and configured SRS transmission periodicity. In this case, RRM measurements for positioning needs are performed within PTW, and requirements are reused from eRedCap WI.

    - Case 2: eDRX cycle is larger than configured PRS measurement reporting periodicity (if PRS measurement is configured) or configured SRS transmission periodicity (if SRS transmission for positioning is configured). In this case, RRM measurements for positioning needs are not limited to PTW, and new requirements are defined.

- Proposals

  - Option 1 (CATT, HW, E///):

    - Yes

  - Option 1a (HW):

    - Define test cases only for Case 2

- Recommended WF

  - Suggest to agree on option 1a.

Discussion:

##### REQ 2: Transmit timing

- Proposals

  - Option 1 (CATT):

    - Test UE transmit timing for positioning measurements

  - Option 2 (HW):

    - Test UE autonomous TA adjustment

- Recommended WF

  - Discuss the options (the 2 options may be same)

Discussion:

What is the difference between Option 1/2?

##### REQ 3: TA validation

- Proposals

  - Option 1 (HW):

    - Yes

- Recommended WF

  - Discuss option 1.

Discussion:

Issue 1-2-4: UE types to be tested

- Proposals

  - Option 1 (CMCC, HW):

    - Define separate sets of TCs for normal (non-RedCap) and RedCap type of UEs

- Recommended WF

  - Suggest to agree on option 1

Discussion:

Option 1 agreeable?

Issue 1-2-5: Test case list

- Proposals

  - Option 1 (CATT):

| Set | Test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| PRS measurement delay tests in RRC_INACTIVE |  |  |
| 1-1 | UE Rx-Tx reporting delay test case with eDRX configured in FR1 (eDRX ≤ 10.24s) |  |
| 1-2 | UE Rx-Tx reporting delay test case with eDRX configured in FR2 (eDRX ＞ 10.24s) |  |
| PRS measurement delay tests in RRC_IDLE |  |  |
| 2-1 | RSTD reporting delay test case with eDRX configured in FR1 (eDRX ≤ 10.24s) |  |
| 2-2 | PRS RSRP reporting delay test case with eDRX configured in FR1 (eDRX ＞ 10.24s) |  |
| 2-3 | PRS RSRPP reporting delay test case with eDRX configured in FR1(eDRX ＞ 10.24s) |  |
| 2-4 | RSTD reporting delay test case with eDRX configured in FR2 (eDRX ＞ 10.24s) |  |

- Recommended WF

  - Option 1 can be taken as a starting point, further update the list based on outcomes from	Issue 1-2-2, 1-2-3 and 1-2-4.

Discussion:

- Topic #7: Core draft CRs for all threads

## General (AI 8.14.2.1)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2402676 | DraftCR to 38.133 to implement measurement gap patterns for RedCap positioning | Ericsson | 9.1A.2 | Can be endorsed |

## SL Positioning (AI 8.14.2.2)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2400117 | (NR_pos_enh2-Core) CR on core requirements for sidelink positioning | CATT | CR (not draft CR) Focus on sections: 12A.1, 12A.2, 12A.3, 12A.4, 12A.5, 12A.6, 12A.7c | Revise |
| R4-2401232 | Corrections to core requirements for Sidelink positioning | Qualcomm Incorporated | Merge into R4-2400117, R4-2401614, R4-2402800 | Merged |
| R4-2401614 | (NR_pos_enh2-Core) Draft CR on core requirement for SL AoA and SL RTOA measurement requirements | Vivo | Is not based on the specification. Focus on sections: 12A.1, 12A.2, 12A.3, 12A.4, 12A.5, 12A.6, 12A.7 | Revise |
| R4-2402177 | draftCR on RRM requirements for SL positioning | Huawei, HiSilicon | Merge into R4-2400117, R4-2401614, R4-2402800 | Merged |
| R4-2402800 | Draft CR to 38.133: corrections for SL positioning | Ericsson | Focus on sections: 12A.1, 12A.2, 12A.3, 12A.4, 12A.5, 12A.6, 12A.7 | Revise |
| R4-2402801 | Big CR remaining issues for SL positioning | Ericsson |  | Return to |

## LPHAP use case	(AI 8.14.2.3)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2401053 | (NR_pos_enh2-Core) Draft CR on UE transmit timing for positioning measurements | CMCC | 7.1.2.4 | Merged |
| R4-2401230 | Correction to UE autonomous TA adjustment for positioning SRS transmission within the SRS validity area in RRC_INACTIVE | Qualcomm Incorporated | 5.6.6.3, 7.1.2.4 | Return to |
| R4-2402179 | draftCR on RRM requirements for LPHAP | Huawei, HiSilicon | 5.6.1A, 5.6A.2 | Return to |
| R4-2402678 | DraftCR to 38.133 Corrections to core requirements for LPHAP | Ericsson | 4.5, 5.6.1, 5.6.2, 5.6.3 | Return to |

## RedCap Positioning	(AI 8.14.2.4)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2400082 | (NR_pos_enh2-Core) CR on correction of measurement period requirements for RedCap UE with FH | CATT | 4.6.2 (consider R4-2402680), 4.6.3 (consider R4-2402680), 4.6.4 (consider R4-2402680), 5.6A.4.6, 5.6A.5.6, 5.6A.6.6, 9.9A.2.6, 9.9A.3.6, 9.9A.4.8, 9.9A.5.5, 9.9A.5.6, 9.9A.5.7, 9.9A.5.8. | Revise |
| R4-2401201 | Draft CR # 16:PRS measurement requirements for RedCap positioning in RRC INACTIVE state (PRS RSRP measurement requirements) | Xiaomi | Do not use track changes on top of other changes! 5.6A.7 (consider R4-2402680) | Revise |
| R4-2401231 | Correction to CSSF for SSB when PRS measurements are configured for RedCap UE | Qualcomm Incorporated | 9.1A.5.2.1, 9.1A.5.2.2 | Return to |
| R4-2402181 | draftCR on RRM requirements for RedCap positioning | Huawei, HiSilicon | 9.9A.4 (consider R4-2400082), 9.9A.5 (consider R4-2400082) | Revise |
| R4-2402680 | DraftCR to 38.133 Corrections to core requirements for RedCap positioning | Ericsson | 4.6.1, 4.6.2, 4.6.3, 4.6.4, 5.6A.4, 5.6A.5, 5.6A.6, 5.6A.7, 9.9A.2, 9.9A.3 | Revise |
| R4-2402903 | Draft CR on correction for Rel-18 RSTD and PRS-RSRP requirements for RedCap in RRC connected state | MediaTek inc. | 9.9A.2 (consider R4-2402680, R4-2400082), 9.9A.3 (consider R4-2402680, R4-2400082) | Revise |

## PRS/SRS bandwidth aggregation (AI 8.14.2.5)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2400084 | (NR_pos_enh2-Core) CR on correction of measurement period requirements with BWA | CATT | 5.6.2.2, 5.6.2.6, 5.6.4.2, 5.6.4.6, 9.9.2.2, 9.9.2.10, 9.9.4.2, 9.9.4.9, 13.2.2.2. | Revise |
| R4-2401872 | CR 38.133 Corrections to measurement period requirements for PRS BW aggregation | Nokia, Nokia Shanghai Bell | 9.9.2.10, 9.9.4.9; Include from R4-2400084: 9.9.2.2, 9.9.4.2 | Revise |
| R4-2402183 | draftCR on RRM requirements for PRS CA | Huawei, HiSilicon | Should be based on specification. 5.6.2.6, 5.6.4.6; Include from R4-2400084: 5.6.2.2, 4.6.4.2 | Revise |
| R4-2402682 | DraftCR to 38.133 Corrections to core requirements for bandwidth aggregation based positioning measurements | Ericsson | 4.5.2.6 (new), 5.6.2.6, 5.6.4.6, 9.9.2.10, 9.9.4.9 | Revise |

## Carrier Phase Positioning (AI 8.14.2.6)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2400119 | (NR_pos_enh2-Core) CR on core requirements for carrier phase positioning | CATT | 4.5.5.5, 5.6.7 (consider R4-2402684), 5.6.8, 9.9.7, 9.9.8 | Revise |
| R4-2401007 | CR on carrier phase positioning | OPPO | 9.9.7, 9.9.8 (consider R4-2402684, R4-2400119) | Revise |
| R4-2402186 | draftCR on RRM requirements for CPP | Huawei, HiSilicon | Should be based on specification. 4.5.5.5 (consider R4-2402684, R4-2400119) | Revise |
| R4-2402684 | DraftCR to 38.133 Corrections to core requirements for carrier phase measurement for positioning | Ericsson | 4.5.5.5, 5.6.7, 5.6.8 (consider R4-2400119), 9.9.7, 9.9.8 | Revise |

- Topic #8: Perf draft CRs for all threads

## SL Positioning (AI 8.14.3.1)

| TDoc | Title | Source | Comments | Decision |
| --- | --- | --- | --- | --- |
| R4-2402803 | Draft CR to 38.133: SL positioning performance requirements structure | Ericsson | Draft CR. The structure is Ok, but revise to resolve the mismatch between parameter k and the tables to be included | Revise |

## LPHAP use case	(AI 8.14.3.2)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2400118 | (NR_pos_enh2-Perf) CR on performance requirements for LPHAP | CATT | CR (not draft CR) | Can be endorsed |

## RedCap Positioning	(AI 8.14.3.3)

## PRS/SRS bandwidth aggregation (AI 8.14.3.4)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2402692 | DraftCR to 38.133 Update to measurement report mapping for positioning measurements | Ericsson |  | Can be endorsed |

## Carrier Phase Positioning (AI 8.14.3.5)

- Topic #9: LS out

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2402190 | On performance requirements for SRS/SRS CA | Huawei, HiSilicon | LS out. | R4-2402190 can be noted. The LS content is agreeable, request a new tdoc for the LS |

Proposal (R4-2402190):

# Differential RSTD measurement report mapping

For k = -3

Minimum reported quantity value: DIFFRSTD_00000

Maximum reported quantity value: DIFFRSTD_65528

Total number of reported quantity values: 65529

For k = -4

Minimum reported quantity value: DIFFRSTD_000000

Maximum reported quantity value: DIFFRSTD_131056

Total number of reported quantity values: 131057

For k = -5

Minimum reported quantity value: DIFFRSTD _000000

Maximum reported quantity value: DIFFRSTD _262112

Total number of reported quantity values: 262113

For k = -6

Minimum reported quantity value: DIFFRSTD _000000000

Maximum reported quantity value: DIFFRSTD _524224

Total number of reported quantity values: 524225

# Differential UE Rx-Tx measurement report mapping

For k = -3

Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_00000

Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_65528

Total number of reported quantity values: 65529

For k = -4

Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000

Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_131056

Total number of reported quantity values: 131057

For k = -5

Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000

Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_262112

Total number of reported quantity values: 262113

For k = -6

Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000

Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_524224

Total number of reported quantity values: 524225

Discussion:

Send LS to RAN2 and agree on the related CR. Remove also unnecessary zeros in:

For k = -6 (for RSTD)

Minimum reported quantity value: DIFFRSTD _000000000