3GPP TSG-RAN4 Meeting #110	R4-2403344

# Athens, Greece,  Feb. 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for selection/reselection of sync referecne source for SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreement in RAN4#109, the requirements for SyncRefUE identification with allowing additional dropping of its SLSS transmission (fast sync detection) needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sentense for fast sync detection are revised below: - When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear for the requirements for SyncRefUE identification with allowing additional dropping of its SLSS transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in Clause 10 are fulfilled for a corresponding Band,

-	S-SSB_RP and S-SSB Ês/Iot according to Annex B.4.3 for a corresponding Band are fulfilled.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to GNSS directly,

-	UE shall not drop any sidelink SLSS and data transmission for the purpose of selection/reselection to the SyncRef UE.

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	UE shall not drop any sidelink data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

For other cases

-	When UE is in non-SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop its sidelink data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE. Only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the sidelink UE is allowed to drop a maximum of 6 % of its sidelink data and SLSS transmissions for the purpose of selection/reselection to the SyncRef UE. Only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X_CCA to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an EUTRA cell is used as synchronization reference source, or Tevaluate,SLSS_CCA in clause 12.3A.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	When UE is in non-SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the sidelink UE is allowed to drop a maximum of 6 % of its sidelink data and SLSS transmissions for the purpose of selection/reselection to the SyncRef UE. When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop its sidelink data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE. When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X_CCA to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an EUTRA cell is used as synchronization reference source, or Tevaluate,SLSS_CCA in clause 12.3A.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UE with the measurement period of Tmeasure,PSBCH-RSRP_CCA in Table 12.4A-1. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

Table 12.4A-1: PSBCH-RSRP measurement period for intra-frequency SyncRef UE

| SL-DRX cycleNote 1 [ms] | Tmeasure,PSBCH-RSRP_CCANote 3 [ms] |
| --- | --- |
| No SL-DRX | (2 + LPSBCH)*160 |
| SL-DRX cycle ≤ 160 | (2 + LPSBCH)*160 |
| SL-DRX cycle > 160 | (2 + LPSBCH)*SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured, the SL-DRX cycle is the shortest one. Note 2: LPSBCH is the number of S-SSB periods in which the SLSS is not available due to CCA failures during Tmeasure,PSBCH-RSRP_CCA, where LPSBCH ≤ LPSBCH,max and LPSBCH,max = 2. Note 3: Upon LPSBCH exceeding LPSBCH,max, the UE shall not reselect to the measured candidate SyncRef UE based on the measurement result. |  |

When UE is synchronized to GNSS directly, before selection/reselection of the new synchronization reference source UE shall evaluate the GNSS synchronization source reliability for at least 20 seconds before changing the synchronization reference from GNSS to another synchronization reference source. UE shall be always synchronized to GNSS directly during the evaluation of GNSS synchronization source reliability.

-------------- End of Change <1> --------------