3GPP TSG-RAN4 Meeting #110	R4-2403391

Athens, Greece, Feb. 26th  –  Mar. 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on LTM TCI state switch activation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Ran4#109, RAN4 agreed to introdcue TCI state actviation requirements for LTM candidate cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TCI state actviation requirements for LTM candidate cell are introdcued |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (8.X) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2402826 |  |  |  |  |  |  |  |  |

Start of Change 1

### 8.X TCI state activation for LTM candidate cell

The requirements in this clause apply for a UE configured with ltm-DL-OrJointTCI-StateToAddModList-r18 configurations on a candidate cell for LTM. The LTM TCI state can be activated by a MAC CE indicating LTM TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321

When the target DL TCI state refers to a candidate cell and the activation of TCI is performed before or during cell switch, the requirements in this clause are applicable provided that following conditions are met:

-	For a UE supports [LTM RTD > CP capability], the requirements apply when Rx timing difference between active TCI states on one frequency layer is up to MRTD as specified in clause [7.6.4].

-	For a UE which do not support [LTM RTD > CP capability], the requirements apply when Rx timing difference between active TCI states on any frequency layer is less than CP.

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received.

-	The target downlink TCI state remains detectable during the downlink TCI state activation period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period

-	SNR of the downlink TCI state ≥ -3dB

Otherwise, the downlink TCI state is unknown.

The requirements in this clause shall apply if the candidate cell satisfies the LTM known cell condition defined in 6.3.1.2 or 8.20.2.

If all the TCI states are known, upon receiving PDSCH carrying the MAC-CE to activate the LTM candiate cell’s TCI states at slot n, UE shall be able to acquire T/F synchronization with the SSBs associated with target TCI states within the first slot that is after n+ [THARQ +3Nslotsubframe,µ + TOk * Tfirst-SSB + M*TSSBp + TSSB-proc) / NR slot length.

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tfirst-SSB is time to first SSB transmission (i.e., SSB associated to TCI state) after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state. In case more than one TCIs are activated, Tfirst-SSB is longest time to first SSB transmssion among all the associated SSBs

-	TSSB-proc = 2 ms.

-	TSSB is SSB transmission periodicity of an inter-frequency cell (i.e., SSB associated to TCI state) after MAC CE command is decoded by the UE. If more than one TCIs are activated, TSSB is the longest SSB periodcity among all the associated SSBs.

-	M = 1 if the TCI state corresponds to inter-frequency cell; 0 otherwise.

-	TOk = 1 if target TCI state is not in the active TCI state list for candidate cell, 0 otherwise.

If UE has reported L1-RSRP for the cell whose TCI state was indicated to be activated within last [X1]ms before the slot n, TCI state is considered known.

If UE has reported the L3-RSRP but not the L1-RSRP for the cell whose TCI state was indicated to be activated within last [X2]ms, TCI state is considered unknown.]

End of Change 1