3GPP TSG-RAN WG4 Meeting # 110										R4-2403468

Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	WF on RRM requirements for NR ATG

Agenda Item:	8.9.8

Source: 	CMCC

Document for:	Approval

- Core Maintenance

Issue 1-1: ATG UE feature

For Location-based measurement report trigger

Agreement:

    - No need to introduce the UE feature ‘Location-based measurement report trigger’

For TA reporting during initial access

Agreement:

    - UE shall support UE feature ‘TA reporting during initial access’ if the UE indicates support of the capability ‘UE reporting of TA infomration’.

For Location based CHO

Agreement:

    - Follow the agreement from RAN2, per band type.

Issue 1-2: TCI state requirements for ATG UE with the antenna array

Agreement:

    - TCI state requirements apply to both UE types: UE with antenna array and UE with omni-direction antenna. (HW)

- Topic #2: ATG RRM performance requirements

Issue 2-1: Test method for UE with antenna array

Agreement:

    - Endorse the CRs without scaling factor (scaling factor = [1]), and close the WI performance part in this meeting.

    - For the draft CRs, add an Editor’s note that: “the scaling factor may be revisited if needed”.

Issue 2-2: UE mobility/location assumption for location-based CHO

Agreement:

    - [UE location, speed (1200 km/h), heading direction] shall be provide to UE via AT command.

    - RAN4 is responsible to design the initial UE location, UE speed, reference location, and distance threshold. The UE location change should be set with the consideration of two distance threshold istanceThreshFromReference1 and distanceThreshFromReference2.

      - In the intra-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled at T2 (earlier than TEvent_DU + Tidentify_intra).

      - In the inter-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled later than T2 (later than TEvent_DU + Tidentify_inter)

Issue 2-3: UE mobility/location assumption for UL transmit timing test

Agreement:

    - Changed UE location with the mobility assumption of 1200km/h, the specific UE location should be emulated by test system and provided to UE by AT command (if the UE position can be updated based on target rate by AT command) or GNSS simulator.

Issue 2-4: Test configuration: BS location

Agreement:

    - The BS location should be emulated by test system.

Issue 2-5: Test configuration: Channel model for changed UE location/ UE speed larger than 0km/h

Agreement:

    - For the test cases that UE speed larger than 0km/h:

      - RAN4 provide the maximum doppler consider the UE speed and maximum carrier frequency, the specific doppler shift trajectory is up to test system’s design considering of BS location and UE location emulation

Issue 2-6: Test configuration: cell-specific koffset

Agreement:

    - The cell specific koffset can be configured as a common parameter. The upper bound 3ms can be considered when RAN4 define the specific value.

Issue 2-7: Test configuration: network assistance on ATG cells reference location

Agreement:

    - For the CRs in this meeting, no network assistance on ATG cells reference location in the configuration.

    - Further discuss the issue if the scaling factor will be agreed in the next meeting.

Issue 2-8: Test configuration: number of neighbour cells for test for ATG UE with the antenna array

Agreement:

    - Configure one neighbour cell in the test for UE with antenna array.

Issue 2-9: TDD pattern ‘30D4S6U’

Agreement:

    - Do not introduce the new TDD pattern ‘30D4S6U’ in ATG test cases.

Issue 2-10: Test case list

Agreement:

  - Define one test case (ATG test 5-5) for DRX. Define all other test cases in non-DRX mode in CONNECTED mode.

  - The location based cell re-selection test cases should be removed.

  - For ATG UE, no need to design the inter-frequency cell re-selection test which neighbour cell configured with high priority frequency.

  - For inter-frequency handover, only consider the unknown case.

  - For intra-frequency handover, only consider the known case.

  - Intra-frequency RRC Re-establishment with known target cell

  - Inter-frequency RRC Re-establishment with unknown target cell without [serving] cell timing.

- The updated test case list:

| Test scope RRC_IDLE state mobility | Test number | Company |
| --- | --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 | CMCC |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 |  |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 |  |

| Test scope RRC_CONNECTED state mobility | Test number | Company |
| --- | --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 | CMCC |
| Inter-frequency HO with unknown target cell | ATG 2-4 |  |
| Intra-frequency location based CHO | ATG 2-5 |  |
| Inter-frequency location based CHO | ATG 2-6 |  |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 | Ericsson |
| Inter-frequency RRC Re-establishment with unknown target cell without serving cell timing | ATG 2-10 |  |
| 4-step RA type contention based random access test | ATG 2-12 | CATT |
| 4-step RA type Non-Contention based random access test | ATG 2-13 |  |
| 2-step RA type contention based random access test | ATG 2-14 |  |
| 2-step RA type Non-Contention based random access test | ATG 2-15 |  |
| RRC Connection Release with Redirection | ATG 2-16 |  |

| Test scope Timing | Test number | Company |
| --- | --- | --- |
| UE transmit timing test | ATG 3-1 | Apple |
| Timing advance adjustment delay and accuracy | ATG 3-2 |  |

| Test scope Signalling characteristics | Test number | Company |
| --- | --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 | Huawei |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 |  |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-3 |  |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-4 |  |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-5 |  |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-6 |  |
| DCI-based and Timer-based Active BWP Switch | ATG 4-7 |  |
| RRC-based Active BWP Switch | ATG 4-8 |  |
| UE specific CBW change | ATG 4-9 |  |
| Pathloss reference signal switching delay | ATG 4-10 |  |

| Test scope Measurement procedure | Test number | Company |
| --- | --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 | ZTE |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-2 |  |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-3 |  |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-4 |  |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used | ATG 5-5 |  |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-6 |  |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-7 |  |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-8 | LGE |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-9 |  |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used | ATG 5-10 |  |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-11 |  |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-12 |  |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-13 |  |

| Test scope Measurement Performance requirements | Test number | Company |
| --- | --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 | CATT |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 |  |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 |  |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 |  |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 |  |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 |  |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 |  |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 |  |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 |  |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 |  |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 |  |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 |  |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 |  |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 |  |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 |  |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 |  |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 |  |

Issue 2-12: Test configuration

Agreement:

    - Introduce the test configuration 15kHz/10MHz TDD to the ATG test cases

      - If legacy TN test case don’t have this test configuration, then no need to introduce it in the corresponding ATG test case.