3GPP TSG-RAN WG4 Meeting # 110															R4-2403544

Athens, Greece, 26 February – 01 March, 2024

Title: 	WF on core maintenance and performance part for BWP without restriction

Agenda Item:	8.3.5

Source: 	vivo, Vodafone

Document for:	Approval

- Agreements in the meeting

<Core Maintenance>

#### Issue 1-2: Requirements/UE behaviour for UE supporting both option C and A

- For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, no additional requirements/ UE behaviour will be defined.

- Both NCD-SSB and CSI-RS within active BWP can be configured by network for L1 measurement.

#### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1

- LS RAN2 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

<Performance part>

#### Issue 2-1: Test cases list for RLM/BFD/BM requirements for option B-1-1

Take the following proposal as baseline, and further discuss the CR wording and applicability rule.

- Add the following NOTE in a selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

  - NOTE: For testing the UE supporting FG 53-1, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP.

#### Issue 2-2: Test cases list for intra-frequency measurements without gaps for option B-1-1

- Add the following NOTE in a selected set of test cases from the existing test cases for intra-frequency measurements without gaps measurements

  - NOTE: For testing the UE supporting FG 53-1, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP.

#### Issue 2-3: Test cases list for RLM/BFD/BM requirements for option C

- Define new RLM/BFD/L1-RSRP/L1-SINR test cases for option C.

#### Issue 2-4: Test cases list for intra-frequency measurements and handover for option C

Define new intra-frequency measurements and handover test cases for option C.

As baseline, agree to introduce Test #2, 3, 5, 6, 7, 8. Further discuss whether Test #1 and 4 are needed.

| Test No. | Test cases | Test configuration | Section |
| --- | --- | --- | --- |
| [1 | Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB | Baseline: A.16.3.1.2 (FR1) | A.6.3.1.Z1] |
| 2 | Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB | Baseline: A.16.3.1.6 (FR1, Measured SSB: CD-SSB, Target SSB: NCD-SSB) | A.6.3.1.Z2 |
| 3 | Inter-frequency handover from FR1 to FR1; unknown target cell configured with NCD-SSB | Baseline: A.16.3.1.6 (FR1, Measured SSB: CD-SSB, Target SSB: NCD-SSB) | A.6.3.1.Z3 |
| [4 | Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB | Baseline: A.16.3.1.2 (FR2) | A.7.3.1.Z1] |
| 5 | Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB | Baseline: A.17.3.1.2 (FR2, Measured SSB: CD-SSB, Target SSB: NCD-SSB) | A.7.3.1.Z2 |
| 6 | Inter-frequency handover from FR2 to FR2; unknown target cell configured with NCD-SSB | Baseline: A.17.3.1.2 (FR2, Measured SSB: CD-SSB Target SSB: NCD-SSB) | A.7.3.1.Z3 |
| 7 | SA event triggered reporting tests without gap configured with NCD-SSB | Baseline: A.6.6.1.1 (FR1, No DRX) | A.6.6.1.Z1 |
| 8 | SA event triggered reporting tests without gap configured with NCD-SSB | Baseline: A.7.6.1.1 (FR2, No DRX) | A.7.6.1.Z1 |
| Note1: In all tests, CD-SSB is configured outside active BWP, but within UE CBW, and NCD-SSB is configured within active BWP. |  |  |  |

#### Issue 2-6: Test cases list for RLM/BFD/BM requirements for option A

- Add the following NOTE in a selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

  - NOTE: For testing the UE supporting FG 53-4, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP.

- Open issues for core maintenance

- Sub-topic 1-1: Requirements for joint options

#### Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- FFS following options

  - Option 1a:

    - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

  - Option 1b:

    - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

  - Option 2:

    - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

    - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1

- FFS following options

  - Option 1:

    - For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario.

  - Option 2:

    - For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

- Sub-topic 1-2: Others

#### Issue 1-4: Applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements

- FFS: NCD-SSB related requirements also apply for PSCell when SCG deactivation is not enabled for the UE.

  - FFS requirements are specified for EN-DC only or for both EN-DC and NR-DC.

#### Issue 1-6: Requirements for L3 intra-f measurement without gaps for option C

- FFS following option

  - Option 1:

    - Add one bullet in section 9.2.1 that the target measurement SSB is completely contained in the active downlink BWP of the UE provided the UE supports [FG53-3] and servingCellMO is present in the corresponding BWP-DownlinkDedicated.

- Open issues for performance part

#### Issue 2-1: Test cases list for RLM/BFD/BM requirements for option B-1-1

- FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

#### Issue 2-2: Test cases list for intra-frequency measurements without gaps for option B-1-1

- FFS: selected set of test cases from the existing test cases for intra-frequency measurements without gaps measurements

#### Issue 2-4: Test cases list for intra-frequency measurements and handover for option C

- Further discuss whether Test #1 and 4 are needed.

#### Issue 2-6: Test cases list for RLM/BFD/BM requirements for option A

- FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

- Work split on test cases

| TC sets | Description | Company |
| --- | --- | --- |
| TC set 1 | set of test cases for RLM/BFD/L1-RSRP/L1-SINR measurements for option B-1-1 | CATT |
| TC set 2 | set of test cases for intra-frequency measurements without gaps for option B-1-1 | CATT |
| TC set 3 | set of test cases for RLM/BFD/L1-RSRP/L1-SINR measurements option C | MTK, Ericsson |
| TC set 4 | selected set of test cases for handover requirements for option C | Huawei |
| TC set 5 | selected set of test cases for intra-frequency measurements without gaps option C | vivo |
| TC set 6 | set of test cases for RLM/BFD/L1-RSRP/L1-SINR measurements for option A | vivo |

----EoD---