3GPP TSG-RAN WG4 Meeting #110	R4-2403606

Athens, Greece, February 26 – March 1, 2024

Title: 	WF on simultaneous Rx-Tx issues

Agenda Item:	7.23.2

Source: 	Huawei

Document for:	Approval

RF front-end and antenna structure assumptions for simultaneous Rx-Tx requirements for higher order band combinations of CA_40A-41A and CA_n39A-n41A

Background:

In Murata paper R4-2400583, two kinds of antenna architectures are provided as assumptions to analyse the higher order combinations of CA_40A-41A and CA_n39A-n41A, including shared and separate antenna structures, as follows.

Observation: For a band combination, higher MSD value is calculated based on the assumption of separate antenna structure than that of shared antenna structure.

(a)

(b)

Figure 1. Reference architecture for (a) LB + MHB (b) MHB+UHB using shared antenna

(a)

(b)

Figure 2. Reference architecture for (a) LB + MHB (b) MHB+UHB using separate antenna

Option 1: Take the shared antenna assumption as baseline for the discussion going forward.

Option 2a: Specify one minimum requirement for one MSD mechanism in the band combination to accommodate all antenna structures.

Option 2b: The MSD requirements can be derived by averaging the MSD from various assumptions.

<Way forward >: Specify RF front-end and antenna structure agnostic MSD for simultaneous Rx-Tx requirements for higher order band combinations of CA_40A-41A and CA_n39A-n41A

- The requirements apply to any RF front-end and antenna structures.

Simplification of Note for simultaneous Rx-Tx requirements in spec

Background:

Background: in the approved WF R4-2321992

The WF R4-2317576 agreed that RAN4 shall review the use of notes related to simultaneous Rx/Tx currently used in the specifications and whether all potential needed requirements are captured as expected. Companies are asked to provide proposals for simplification and increasing consistency.

Agreement:

-	Work on merging Note 13 and Note 15, and check if Note 9 can be used in the table.

(Excerpt)Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n34-n399 | n34, n39 |  |
| CA_n46-n9615,16,17,18 | n46, n96 |  |
| CA_n46-n10215,16,18,20 | n46, n102 |  |
| CA_n48-n539 | n48, n53 |  |
| CA_n48-n7713,14,18 | n48, n77 |  |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. NOTE 13: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n48 with an n77 implementation. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting CA_n46-n96 or CA_n46-n102. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations NOTE 16: The minimum requirements for intra-band non-contiguous CA/DC apply for CA_n46-n96 or CA_n46-n102 and related higher order CA/DC configurations. NOTE 17: The combination is not used alone as fall back mode of other band combinations in which UL in Band 48 is not used. NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. NOTE 20: The combination is not used alone as fall back mode of other band combinations in which UL in Band n78 is not used. |  |  |

- Proposals:

    - Proposal 1 (Nokia): Void Note 13 and Note 15 in Table 5.2A.2.1-1 and apply Note 9 were used in the Table.

    - Proposal 2 (Nokia): RAN4 shall remove notes related to simultaneous Rx/Tx in all band combination tables except for the two band CA and DC combinations.

<Way forward>:

-	Void Note 13 and Note 15 in Table 5.2A.2.1-1 and apply Note 9 to the corresponding band combinations.