3GPP TSG-RAN WG4 Meeting # 110											R4-2403629

Athens, Greece, Feb. 26th – Mar. 1st , 2024

Title: 	WF on HPUE for FDD bands

Agenda Item:	7.20

Source: 	China Unicom, Skyworks

Document for:	Approval

<Topic 1 A-MPR>

<Sub-topic 1-1 HPUE with dual Tx>

The handling of HPUE with dual Tx requires special attention. When NR-U PC3 was specified, a separate table was introduced for dual Tx. While this might offer the clearest solution in terms of explicitly defined values it creates considerable work and more cluttering.

Companies are therefore asked to provide their views and proposals to efficiently solve the challenges with specifying A-MPR for dual Tx.

<Way forward>:

- Option 1: Limit the dual-Tx UE A-MPR to be at most 3 dB higher than PC3, while allowing the extra [2] dB relaxation within this boundary.

- Option 2: Introduce dedicated A-MPR for PC2 with dual Tx (as has been done for other NS flags such as NS_60).

- Option 3:

  - Void Note 7 from Table 6.2.3.1-1,

  - Limit the single-Tx and dual-Tx PC2 FDD A-MPR to be at most 3dB higher than PC3 A-MPR,

  - Capture the dual-Tx A-MPR additional relaxation for each affected NS flag.

  - We invite companies to study how to best capture the dual-Tx additional relaxation:

    - separate table from PC2 single Tx?

    - footnote to single-Tx PC2 A-MPR for each regions?

    - any other approach not precluded.

- Other Options are not precluded.

<Sub-topic 1-2 NS_07 (A-MPR for n13 PC2)>

RB region and A-MPR tables are the result of merging company contributions:

Table 1: A-MPR regions for NS_07 (PC2)

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | ≥0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
|  |  | >1.26, ≤2.16 | ≥1.26, <1.8 | A5 |
|  |  | >3.06 | >0.36 | A5 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |
|  |  | >3.24, ≤6.48 | ≥1.8, <3.24 | A5 |
|  |  | >6.48 | >0.36 | A5 |

Table 2: A-MPR for NS_07 (PC2 with single Tx)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 13.5 | 10.5 | 8.0 | 6.0 | 3.0 |
| DFT-s-OFDM QPSK | 13.5 | 10.5 | 8.0 | 6.0 | 3.0 |
| DFT-s-OFDM 16 QAM | 13.5 | 10.5 | 8.0 | 6.0 | 3.0 |
| DFT-s-OFDM 64 QAM | 13.5 | 10.5 | 8.0 | 6.0 | 3.0 |
| DFT-s-OFDM 256 QAM | 13.5 | 10.5 | 8.0 | 6.0 | 3.0 |
| CP-OFDM QPSK | 15.5 | 11.5 | 9.0 | 6.0 | 3.0 |
| CP-OFDM 16 QAM | 15.5 | 11.5 | 9.0 | 6.0 | 3.0 |
| CP-OFDM 64 QAM | 15.5 | 11.5 | 9.0 | 6.0 | 3.0 |
| CP-OFDM 256 QAM | 15.5 | 11.5 | 9.0 | 6.0 | 3.0 |

Table 3: A-MPR for NS_07 (PC2 with dual Tx) [in case of Sub-topic 1-1, Option 2]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 15.0 | 12.0 | 9.0 | 6.0 | 3.0 |
| DFT-s-OFDM QPSK | 15.0 | 12.0 | 9.0 | 6.0 | 3.0 |
| DFT-s-OFDM 16 QAM | 15.0 | 12.0 | 9.0 | 6.0 | 3.0 |
| DFT-s-OFDM 64 QAM | 15.0 | 12.0 | 9.0 | 6.0 | 3.0 |
| DFT-s-OFDM 256 QAM | 15.0 | 12.0 | 9.0 | 6.0 | 3.0 |
| CP-OFDM QPSK | 17.0 | 13.0 | 10.0 | 6.0 | 4.0 |
| CP-OFDM 16 QAM | 17.0 | 13.0 | 10.0 | 6.0 | 4.0 |
| CP-OFDM 64 QAM | 17.0 | 13.0 | 10.0 | 6.0 | 4.0 |
| CP-OFDM 256 QAM | 17.0 | 13.0 | 10.0 | 6.0 | 4.0 |

Table 3-bis: dual-Tx PC2 additional A-MPR relaxation from single-Tx PC2 A-MPR for NS_07

[in case of Sub-topic 1-1, Option 3]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| All modulations and all waveforms | 1.5 | 1.5 | 1.0 | 0.0 | 0.0 |

<Way forward>:

- Further check whether NS_07 A-MPR for PC2 with single-Tx can be specified by introducing Tables 1 and 2 to 38.101-1.

- Table 3 is a showcase how A-MPR could end up in case Option 2 is selected in Sub-topic 1-1

- Table 3-bis as one example (to be further studied) of how the dual-TX A-MPR relaxation could be captured for sub-topic 1-1 option3.

<Sub-topic 1-3 A-MPR and related issues for n28 PC2>

<Agreement>:

- For a UE implementing the dual-duplexer on band n28, no A-MPR for NS_17 is needed for PC2 as well as PC3, for BW of 5MHz and 10MHz.

<Way forward>:

- Continue work on NS_17 A-MPR requirements.

<Sub-topic 1-4 NS_46 (A-MPR for n7 PC2)>

<Way forward>:

- Try to finalize the results for NS_46 by merging the A-MPR regions and values in R4-2400171, R4-2402213, R4-2402742.

- Further check the following candidate solution from R4-2400171:

Table 7: A-MPR regions for NS_46 for PC2 (Red colour indicates differences to [R4-2320669])

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2555 ≤ FC ≤ 2565 | ≥ 7.92 | ≥ 7.2 | A5 |
|  |  | ≥ 7.92 | < 2.16 | A5 |
| 15 MHz | 2547.5 ≤ FC ≤ 2562.5 | ≥ 11.16 | ≥ 7.2 | A6 |
|  |  | ≥ 11.16 | < 7.2 | A10 |
| 20 MHz | 2540 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A6 |
|  |  | ≥ 15.3 | ≥ 8.64 | A6 |
|  |  | ≥ 14.4, < 15.3 | < 8.64 | A10 |

Table 8: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 12.24 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 0 | > 16.02 | A9 |
|  |  | ≥ 0 | ≥ 12.24, ≤ 16.02 | A6 |
|  |  | ≥ 17.64 | < 12.24 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <5.04 | >max (0, 12*SCS*RBend - 4.5) | A11 |
|  |  | ≥5.04, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 9: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 7.5 | 11.0 | 6.5 | 3.0 | 5.0 |
|  | QPSK | 6.5 | 2.5 | 4.0 | 7.5 | 11.0 | 6.5 | 3.0 | 5.0 |
|  | 16 QAM | 6.5 | 2.5 | 4.0 | 7.5 | 11.0 | 6.5 | 3.5 | 5.0 |
|  | 64 QAM | 6.5 | 2.5 | 4.0 | 7.5 | 11.0 | 6.5 | 3.5 | 5.0 |
|  | 256 QAM | 6.5 |  |  | 7.5 | 11.0 | 6.5 |  | 5.0 |
| CP-OFDM | QPSK | 6.5 | 3.5 | 6.0 | 8.5 | 12.0 | 8 | 4.5 | 6.5 |
|  | 16 QAM | 6.5 | 3.5 | 6.0 | 8.5 | 12.0 | 8 | 4.5 | 6.5 |
|  | 64 QAM | 6.5 | 3.5 | 6.0 | 8.5 | 12.0 | 8 | 4.5 | 6.5 |
|  | 256 QAM | 6.5 |  |  | 8.5 | 12.0 | 8 |  |  |

<Sub-topic 1-5 NS_12, NS_13, NS_14, NS_15 (A-MPR for n26 PC2)>

<Way forward>:

- Set a starting point for NS_12, NS_13, NS_14, NS_15 by merging the A-MPR regions and values in R4-2402214 and R4-2315374. New inputs can be still taken into consideration.

<Sub-topic 1-6 NS_06 (A-MPR for n13, n14, n85 PC2)>

<Agreement>:

Introduce A-MPR for outer allocations by specifying Table 2 for NS_06.

Table 2: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

Sub-topic 1-6 has been completed at this meeting with the approval of the TP in R4-2403625.

Reference

[1] R4-2401072 Topic summary for [110][113] HPUE_Basket_FDD	Moderator (China Unicom)	RAN4-110