3GPP TSG-RAN4 Meeting #110	R4-2043640

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2165 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC] CR for 38.101-1 Add the missing CA_n41A-n77A configuration for ULCA and TxD scenario with 3Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_n41A-n77A was selected as PC2 an example band combination for the 4Rx/3Tx WI. This combo has been included in Table 6.2H.3.1-1 of TS38.101-1 v18.4.0 which is for the ULCA+UL MIMO scenario, while missing in clause 6.2L.3.1 for the ULCA+TxD scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missing combination CA_n41A-n77A to enable ULCA+TxD scenario with 3Tx. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ULCA+TxD 3Tx scenario is not introduced for CA_n41A-n77A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2L.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 corrects the superscript note in the table in 6.2L.3.1 from 4 to 2. |  |  |  |  |  |  |  |  |

### < First Change >

### 6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity

#### 6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2L.3.1-1. The period of measurement shall be at least one sub frame (1 ms).

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n41A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n26A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n77A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: FWA form factor is targeted unless otherwise stated. |  |  |  |  |  |  |

### < End of Changes >