3GPP TSG-RAN4 Meeting #110	R4-2403641

# Greece, Athens, February 16th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draft-CR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5: sub-clause 9.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-1-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce content in chapter 9.2.1 in TS 38.101-5 based on the agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce maximum output power requirements in chapter 9.2.1 in TS 38.101-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The maximum output power requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

9.2	Transmitter power

9.2.1	UE maximum output power

9.2.1.0	General

The UE classes are specified based on the assumptions of certain UE types with specific device architectures including antenna beam steering types. The requirements are specified for different UE types. And for the the hybrid beam steering capable UE, which can adjust its antenna(s) or beam(s) in both electronic steering and mechanical steering ways, the applicable requirements should follow either electronic or mechanical beam steering requirements depending on the UE type it declared. The UE types can be found in Table 9.2.1.0-1 below.

Table 9.2.1.0-1: Assumptions of UE Types

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| Note 1: The UE types are assuming UE has only one antenna beam towards one satellite at a given time in this release. Note 2: The Mobile VSAT communicating with non-GSO is not considered in this release. |  |  |

9.2.1.1	Minimum requirements for Fixed VSAT

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 9.2.1.1-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.2.1.1-1: UE minimum peak EIRP for Fixed VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n510 | 1 | 70 |
|  | 2 | 70 |
|  | 3 | 61 |
| Note: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for EIRP are found in Table 9.2.1.1-2 below.

Table 9.2.1.1-2: UE maximum output power limits for Fixed VSAT

| Operating band | Max EIRP (dBm) |
| --- | --- |
| n512, n511, n510 | 76.2 |

The maximum output power values for TRP are TBD, FFS how to specify them.

Note: The maximum TRP limit for UE should also follow the regulatory requirements, including both ECC and FCC requirements.

9.2.1.2	Minimum requirements for Mobile VSAT

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 9.2.1.2-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.2.1.2-1: UE minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| Note: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for EIRP are found in Table 9.2.1.2-2 below.

Table 9.2.1.2-2: UE maximum output power limits for Mobile VSAT

| Operating band | Max EIRP (dBm) |
| --- | --- |
| n512, n511 | 76.2 |

The maximum output power values for TRP are TBD, FFS how to specify them.

Note: The maximum TRP limit for UE should also follow the regulatory requirements, including both ECC and FCC requirements.

<End of Change>