3GPP TSG-RAN WG4 Meeting #110    	R4-2403824

Athens, Greece, 26 February – 1 March 2023

Title:	WF on CA_NC_NS_100

Agenda item:	6.1.5

Source:	Skyworks Solutions Inc., Meta Ireland.

Document for:	Approval

# 1	Introduction

The UTRA ACLR requirements for intra-band non-contiguous CA (CA_NC_NS_100) have been recently introduced. This WF captures initial proposals in [1] on refining these requirements.

- On UTRA ACLR requirements for intra-band non-contiguous CA

# 2.1 	Background

Figure 1 reproduces the UTRA ACLR core requirements for intra-band non-contiguous CA.

Figure 1: UTRA ACLR requirements for intra-band non-contiguous CA.

- The following observation is reproduced from [1]:

- for Wgap = 10MHz, the UTRAACLR1 of each CC can be verified, but not the UTRAACLR2,

- for Wgap = 20MHz, both the UTRAACLR1 and the UTRAACLR2 requirements can be verified in the gap.

These constraints are not currently reflected in the core requirements. This WF proposes to clarify the “in-gap” UTRA ACLR requirements vs Wgap.

## 2.2	Way Forward on CA_NC_NS_100

WF on CA_NC_NS_100:

In Clause 6.5A.2.4.2.4, introduce the following clarification of UTRA ACLR requirements in the gap:

  - For 10<=Wgap < 20MHz: only the UTRA ACLR1 requirement applies is the gap;

  - For Wgap >= 20MHz: both the UTRA ACLR1 and the UTRA ACLR2 requirements apply is the gap.

Interested companies are invited to provide further improvements to core requirement specifications.

- References

- [1]		R4-2400372, On CA_NC_NS_100, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.