3GPP TSG-RAN WG4 Meeting #110	R4-2403834

# Athens, Greece,  February 26th - March 1st 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.849 | CR | 0008 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support for the VLP mode in US |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Charter Communications Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | On September 2023, FCC decided to permit very low-power (VLP) operation in the following sub-bands: U-NII-5 and U-NII-7. And the corresponding changes in the US regulatory domain have been already captured during the 3GPP RAN#102 meeting by updating TR 37.890. To support the VLP mode in US a new NS flag is needed because the US regulations limit the VLP mode to U-NII-5 and U-NII-7 sub-bands within the 6GHz unlicensed band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new NS_71 flag is added to support the VLP mode in US. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VLP mode in the unlicensed 6GHz band will not be supported in US. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Updating NS flag from NS_70 to NS_71 as the value ‘70’ is already used for UAV flag. |  |  |  |  |  |  |  |  |

<< Start of changes >>

# 6	RF requirements

## 6.1	UE specific

### 6.1.1	Transmitter characteristics

This section details specific transmitter characteristics for a UE operating in the 6 GHz NR band.

Table 6.1.1-1: Summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_64 |
| UK | N/A | NS_01 | NS_65 |
| Morocco | N/A | NS_01 | NS_65 |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | NS_68 |
| Qatar | N/A | NS_01 | NS_65 |
| Jordan | N/A | NS_01 | NS_65 |
| Russian Federation | N/A | NS_01 | NS_68 |
| South Africa | N/A | NS_58 | NS_64 |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | NS_71 |
| Canada | NS_54 | NS_59 | NS_66 |
| Brazil | N/A | NS_53 | NS_67 |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | NS_65 |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | NS_60 | NS_66 |
| Argentina | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_64 |
| Australia | N/A | NS_01 | NS_68 |
| New Zealand | N/A | NS_01 | NS_68 |
| Malaysia | N/A | NS_01 | NS_65 |
| Japan | N/A | NS_63 | NS_69 |
| Singapore | N/A | NS_01 | NS_68 |

<< End of changes >>