3GPP TSG-RAN WG4 Meeting #110	R4-2403862

Athens, Greece, 26 February ‒ 1 March, 2024

Title: 	WF on coverage enhancement for part 1

Agenda Item:	8.19.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Issue 1: Whether to introduce UE capability for dpc-Reporting-FR1

Agreement:

- Introduce UE capability for dpc-Reporting-FR1.

Issue 2: Whether to extend the value range of PowerClassPerBandPerBC in order to cover PC5 for better support high power limit for PC3+PC5 CA/DC

Agreement:

- Inform RAN2 that per-band per BC power class capability should include power class 5 since Rel-18.

  - Refer to the LS in R4-2403659.

Issue 3: UE capability for dpc-Reporting-FR1

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-1 | Support of ΔPPowerClass reporting mechanism | Support of ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting which is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 and TS 38.101-3. | No | Yes | N/A | UE does not support of report on the ΔPPowerClass | Per UE | No | FR1 only | N/A | Component 1 candidate values: - Type 1: The UE can only report ∆PPowerClass for non-CA operation - Type 2: The UE can report ∆PPowerClass for non-CA operation, and the UE can also report ∆PPowerClass/ ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC for CA operation. | Optional with capability signalling |