3GPP TSG-RAN WG4 Meeting #110bis		R4-2404657

Changsha, China, 15th – 19th April, 2024

Agenda item:	 9.14.3

Source:	 vivo

Title:	Views on LP-WUS BS RF requirements

Document for:	 Approval

- Introduction

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

  - Current NR BS requirements is baseline

RAN plenary approved the Rel-19 LP-WUS WID with the following RAN4 BS RF scope [1]:

This contribution shares initial views on LP-WUS BS RF requirements.

- LP-WUS BS RF requirements

  - Dynamic range for LP-WUS BS RF

#### 7.1.2.5	gNodeB RF impacts

The power boosting of 3 dB or 6 dB of a WUS signal relative to the average power of a configured NR carrier can improve the WUS signal coverage. It is beneficial to enable the 3dB or 6 dB power boosting at the existing network. Therefore, RAN4 assumes to reuse the legacy RF specification as a starting point. Some company observed that 3 dB or 6 dB power boosting for a 24 RB WUS signal may not be feasible for some of deployed gNB when average power is kept the same before and after power boosting. It is feasible to configure different PSD carriers in a multiple carrier configuration where WUS signal can be placed in a higher PSD carrier among all configured carriers. The amount of boosting is up to manufacturer declaration following the legacy manufacturer declaration.

BS manufacturer could declare if power boosting for WUS signal is supported and the boosting level in the range of [0] to [6] dB is considered. Final power boosting level and the condition of power boosting will be decided based on further analysis in WI phase.

In the conclusion part [9.4 RAN4 RF study]:

-	It is beneficial to enable power boosting for LP-WUS to improve the WUS signal coverage. The supporting of LP-WUS power boosting and boosting level can be declared by manufacturer.

In SI phase, initial outcome and observations for BS LP-WUS output power in TR 38.869:

As stated in the WID, current BS requirements is the baseline, which means at least the NR dynamic range of should be followed.

In RAN4 BS requirements, TS 38.104 Table 6.3.2.2-1, the following RE power control dynamic range is defined:

RE power control dynamic range:

Table 6.3.2.2-1: RE power control dynamic range (in TS 38.104)

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

Given LP-WUS waveform is generated by NR OFDM, so the dynamic range of LP-WUS can follow the original Modulation scheme, e.g., for QPSK-based LP-WUS waveform generation, the dynamic range could be +4dB up. Then the expected [6]dB LP-WUS power boosting identified in SI phase means further increase upper limit of 2dB for LP-WUS/LP-SS beyond current dynamic range.

Therefore, the task in RAN4 for BS RF dynamic range is to study whether additional 2dB dynamic range can be achieved and how many WUS RBs can be boosted. Meanwhile, RAN4 should also identify a generic framework/approach on how to evaluate the feasibility of this value, for making final decision.

Proposal 1: Based on SI outcome and WID scope, RAN4 should study whether 2dB could be increased for LP-WUS RBs beyond current dynamic range level.

Proposal 2: RAN4 should discuss and decide a general framework to evaluate the LP-WUS/LP-SS dynamic range.

  - Other BS RF requirements

For other BS transmission requirements, RAN4 still need to check whether there is any impacts on other BS RF requirements to support LP-WUS transmission. If necessary, potential BS RF requirements should also be discussed and specified.

Proposal 3: RAN4 further check whether other BS RF requirements should be re-evaluated and specified to support LP-WUS transmission.

- Conclusion

In this contribution, we share our views on LP-WUR BS RF and have the following proposals:

Proposal 1: Based on SI outcome and WID scope, RAN4 should study whether 2dB could be increased for LP-WUS RBs beyond current dynamic range level.

Proposal 2: RAN4 should discuss and decide a general framework to evaluate the LP-WUS/LP-SS dynamic range.

Proposal 3: RAN4 further check whether other BS RF requirements should be re-evaluated and specified to support LP-WUS transmission.

# References

- RP-240801, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, RAN meeting #103, Mar 2024.

- TR 38.869 LP-WUS RAN1 TR