3GPP TSG-RAN4 Meeting #110bis 	R4-2406005

# Changsha, China, 15 - 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH requirements with CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New Requirements for HST FR2 PDSCH with CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new Section for HST FR2 PDSCH with CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for PDSCH HST FR2 with CA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a revision of R4-2321188 already endorsed in RAN4#109. The revision is to add a margin of 0.8dB instead of 0.5dB for 64QAM as in the later agreements for HST FR2 with multi-Rx in RAN4#110, 0.8 dB is added for 64QAM. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 7.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 7.2-1 for PDSCH single carrier tests are reused for PDSCH CA test unless otherwise stated.

### 7.2A.1	1RX requirements

(Void)

### 7.2A.2	2RX requirements

#### 7.2A.2.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 7.2A.2.1-3 based on the single carrier requirements for different bandwidth specified in Table 7.2A.2.1-2, with the parameters in Table 7.2A.2.1-1 and the downlink physical channel setup according to Annex C.5.1. The performance requirements specified in this sub-cluase do not apply for UE single carrier test.

Table 7.2A.2.1-1: Test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | 120kHz SCS: FR2.120-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 7.2A.2.1-2: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.4 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.2 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |

Table 7.2A.2.1-3: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 120 kHz + TDD 120 kHz | As defined in Table 7.2A.2.1-2 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 7.1.1.5. |  |  |

#### 7.2A.2.2 Minimum Requirements for HST-FR2-DPS with CA

For HST-FR2-DPS with CA, the requirements are defined in Table 7.2A.2.2-5 based on the single carrier requirements for 120KHz SCS and different bandwidth specified in Table 7.2A.2.2-3 and Table 7.2A.2.2-4, with the additional parameters in Table 7.2A.2.2-2 and the downlink physical channel setup according to Annex C.5.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 7.2A.2.2-1.

Table 7.2A.2.2-1: Test Purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-FR2-DPS scenario defined in B.3.4.2 with CA with 1 active PDSCH TCI states | 1 |
| Verify UE performance in the HST-FR2-DPS scenario defined in B.3.4.1 with CA with 2 active PDSCH TCI states | 2 |

Table 7.2A.2.2-2: Test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 1 and 2 |
|  |  |  |  | 6 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 8 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 5 and 6 |
|  |  |  |  | 6 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 9 and 10 |
|  |  |  |  | 6 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 15 |
|  |  |  |  | l0 = 8 for CSI-RS resource 14 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 13 and 14 |
|  |  |  |  | 6 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 17 and 18 |
|  |  |  |  | 6 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 23 |
|  |  |  |  | l0 = 8 for CSI-RS resource 22 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 21 and 22 |
|  |  |  |  | 6 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 25 and 26 |
|  |  |  |  | 6 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 31 |
|  |  |  |  | l0 = 8 for CSI-RS resource 30 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 29 and 30 |
|  |  |  |  | 6 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in TS38.101-4 Annex A.1.3 |
| Note 1: For Test 1 in Table 7.2A.2.2-5, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0 (i≠0). PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=12k-2n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=2,3,4… to slot# 2k-1n+THARQ+TMAC proc,k=1,2,3…, PDCCH and PDSCH associated with TCI # ((k mod 4)+8) is transmitted by kth RRH from slot# 2k+1n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=0,1,2… to slot# [2k+1n+THARQ+TMAC proc],k=0,1,2…, where k is the RRH number, n = 28800 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing, TfirstSSB = 132 is the number of slots to first SSB transmission occasion after MAC CE command is decoded by the UE, TSSB proc= 16 is the number of slots for SSB processing, TfirstTRSafterSSB = 66 is the number of slots to first TRS transmission occasion after first SSB is processed by the UE, TTRSproc= 16 is the number of slots for TRS processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 2 in Table 7.2A.2.2-5, SSB # (k mod 4) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0. PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=1k-1n+2+THARQ+TMAC proc,k=2,3,4… to slot# [k * n+THARQ+TMAC proc], k=1,2,3…, where k is the RRH number, n = 57600 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Note 2: Only configured for Test 1 in Table 7.2A.2.2-5. |  |  |  |  |

Table 7.2A.2.2-3: Single carrier performance for HST-FR2-DPS CA Configurations with 1 Active TCI State

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.5-16.1 TDD | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2, ULA Low | 70 | [13.6] |
| R.PDSCH.5-16.2 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2, ULA Low | 70 | [13.9] |
| R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2, ULA Low | 70 | [14.0] |
| R.PDSCH.5-16.3 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2, ULA Low | 70 | [13.8] |

Table 7.2A.2.2-4: Single carrier performance for HST-FR2-DPS CA Configurations with 2 Active TCI States

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.5-17.1 TDD | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2, ULA Low | 70 | [13.7] |
| R.PDSCH.5-17.2 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2, ULA Low | 70 | [13.9] |
| R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2, ULA Low | 70 | [14.0] |
| R.PDSCH.5-17.3 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2, ULA Low | 70 | [13.8] |

Table 7.2A.2.2-5: Minimum performance for HST-FR2-DPS multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 120 kHz + TDD 120 kHz | As defined in Table 7.2A.2.2-3 |
| 2 | TDD 120 kHz + TDD 120 kHz | As defined in Table 7.2A.2.2-4 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 7.1.1.5. |  |  |

<End of Change 1>