# 3GPP TSG-RAN WG4 Meeting # 110bis	 R4-2306590

# Changsha, China, 15th – 19th April, 2024

Title: 	WF on 4Tx SRS issues

Agenda Item:	6.1.4

Source: 	vivo

Document for:	Approval

Background

An issue has been raised for ΔPPowerClass for 4Tx for SRS antenna switching in [1]. A set of tentative solutions were raised in [2] with an accompanying draft CR in [3]. However, no agreements have been reached so far and it is decided that May meeting will be last meeting to treat this issue in Rel-18 as documented in Chair’s notes:

Chair: The deadline to address this issue is May meeting 2024 (RAN4#111).

Way Forward

Proposal: Companies are encouraged to consider the following options for ΔPPowerClass for SRS antenna switching for 4Tx.

Option 1: Combine the ΔPPowerClass and ΔTRxSRS to achieve the needed backoff with some flexibility for 4Tx.

- Details and related issues according to [1] and [2]

- Pros: Leaving the flexibility for UE implementation, while do not restrict the UL power;

- Cons: Complicated; Network not aware of power class used for sounding SRS; It allows unnecessary relaxation for certain architectures.

Option 2: Using fixed ΔPPowerClass for 4Tx based on the minimum capability architecture, i.e. 4*23 dBm.

- Define 3dB for t2ry, 6dB for t1ry, for all cases without considering other architectures.

- Pros: Simple and power achievable irrespective of architectures. Network aware of power class used for sounding SRS.

- Cons: Unnecessarily restrict output power for architectures with at least a 26dBm RF chain.

Option 3: Keep it as it is and do not define new requirements for 4Tx. (Default)

- Current wording already provide a 3dB value for PC1.5 which would imply a 26dBm RF chain should be used for 4Tx SRS antenna switching, and might be problematic for 4*23dBm architecture.

- No dedicated conformance testing for this part.

Option 4: Other solutions or variants of previous solutions not precluded.

References

R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

R4-2404658, 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis

R4-2404659, Draft CR for 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis