3GPP TSG-RAN4 Meeting #111	R4-2407083

# Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2207 | rev |  | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 on removing MSD requirements with intra-band contiguous UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MSD test configurations consisting of intra-band contiguous UL CA have been specified with relatively small RB allocations non-contiguously placed between the two contiguous UL carriers. As UL CA is configured primarily to increase the UL throughout, it does not seem to be very practical to schedule small RB allocations non-contiguously between the two contiguous UL carriers where the total resource allocation is even less than a single carrier can already provide. Non-contiguous UL allocations may also be subject to higher MPR/A-MPR in order to fulfill the emission requirements and may result in relatively high MSD due to clustered inter-modulation product falling onto victim DL carrier. Therefore, MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations are considered not necessitated due to that not only the UL configurations are impractical from network scheduling point of view but also the requirements do not really provide additional test coverage beyond what can already be verified in the same band or band combination without contiguous UL CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Void Table 7.3A.2.1-1 and remove the entire table below. - Revise text on intra-band contiguous UL CA reference sensitivity requirement for FDD bands in sub-clause 7.3A.2.1 to clarify that the reference sensitivity is not specified as it can be verified in one uplink carrier configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unnecessary MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations would remain in the technical specifications which will not only continue adding RAN4 workload in future but also create unnecessary UE test burden. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2,  A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is not specified as it can be verified in one uplink carrier configuration.

Table 7.3A.2.1-1: Void

<<< End of changed sections >>>