3GPP TSG-RAN4 Meeting #111	R4-2407086

# Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1191 | rev |  | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-3 on removing MSD requirements with intra-band contiguous UL CA in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R17_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MSD test configurations consisting of intra-band contiguous UL CA have been specified with relatively small RB allocations non-contiguously placed between the two contiguous UL carriers. As UL CA is configured primarily to increase the UL throughout, it does not seem to be very practical to schedule small RB allocations non-contiguously between the two contiguous UL carriers where the total resource allocation is even less than a single carrier can already provide. Non-contiguous UL allocations may also be subject to higher MPR/A-MPR in order to fulfill the emission requirements and may result in relatively high MSD due to clustered inter-modulation product falling onto victim DL carrier. Therefore, MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations are considered not necessitated due to that not only the UL configurations are impractical from network scheduling point of view but also the requirements do not really provide additional test coverage beyond what can already be verified in the same band or band combination without contiguous UL CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove MSD test configuration with intra-band contiguous UL CA as aggressor in Table 7.3B.2.3.5.1-1, i.e., DC_1C_n3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unnecessary MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations would remain in the technical specifications which will not only continue adding RAN4 workload in future but also create unnecessary UE test burden. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

###### 7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |  |
| DC_1_n3 | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |  |
|  | n3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |  |
| DC_1A_n8A | 1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6.0 |  | IMD4 |  |
|  | n8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |  |
| DC_1A_n71A DC_1A_n71B | 1 |  | 1958 |  | 5 |  | 25 |  | 2148 |  | N/A |  | N/A |  |
|  | n71 |  | 668 |  | 5 |  | 25 |  | 622 |  | 15.1 |  | IMD3 |  |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 29.8 |  | IMD23 |  |
|  | n77 |  | 4090 |  | 10 |  | 50 |  | 4090 |  | N/A |  | N/A |  |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n77(3A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |  |
|  | n77, n78 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |  |
| DC_2A_n46A | 2 |  | 1880 |  | 5 |  | 25 |  | 1960 |  | 12.0 |  | IMD3 |  |
|  | n46 |  | 5720 |  | 20 |  | 100 |  | 5720 |  | N/A |  | N/A |  |
| DC_2A_n48A | 2 |  | 1852.5 |  | 5 |  | 25 |  | 1932.5 |  | 12 |  | IMD4 |  |
|  | n48 |  | 3625 |  | 20 |  | 100 |  | 3625 |  | N/A |  | N/A |  |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |  |
|  | n66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |
|  | n66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |  |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |  |
|  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8.0 |  | IMD4 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |  |
|  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |  |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD23 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 8.0 |  | IMD43 |  |
|  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |  |
| DC_3_n1 | 3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |  |
|  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |  |
| DC_3_n5 | 3 |  | 1771 |  | 10 |  | 50 |  | 1866 |  | 4 |  | IMD4 |  |
|  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | N/A |  | N/A |  |
|  | 3 |  | 1721 |  | 10 |  | 50 |  | 1816 |  | N/A |  | N/A |  |
|  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 24 |  | IMD23 |  |
| DC_3A_n7A DC_3C_n7A | 3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |
|  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 10.2 |  | IMD4 |  |
| DC_3_n8 | n8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |  |
|  | 3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |  |
|  | n8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |
|  | 3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |  |
| DC_3A_n20A | 3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |  |
| DC_3C_n20A | n20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |
|  | 3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |
|  | n20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |  |
| DC_3A_n38A | 3 |  | 1712.8 |  | 5 |  | 25 |  | 1807.8 |  | 8.2 |  | IMD4 |  |
|  | n38 |  | 2616.7 |  | 10 |  | 50 |  | 2616.7 |  | N/A |  | N/A |  |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |  |
|  | n41 |  | 2657.5 |  | 10 |  | 50 |  | 2657.5 |  | N/A |  | N/A |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_n77(3A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 26 |  | IMD23 |  |
|  | n77, n78 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 8.0 |  | IMD43 |  |
|  | n77, n78 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |  |
| DC_4A_n2A | 2 |  | 1860 |  | 20 |  | 502 |  | 1940 |  | 5 |  | IMD3 |  |
|  | 4 |  | 1752.5 |  | 5 |  | 25 |  | 2152.5 |  | N/A |  | N/A |  |
|  | 2 |  | 1868.3 |  | 5 |  | 25 |  | 1948.3 |  | N/A |  | N/A |  |
|  | 4 |  | 1735 |  | 5 |  | 25 |  | 2135 |  | 5 |  | IMD5 |  |
| DC_4A_n5A | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |  |
|  | 4 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |  |
| DC_4A_n7A | 4 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |
|  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | IMD4 |  |
| DC_5A_n3A | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | N/A |  | N/A |  |
|  | n3 |  | 1771 |  | 10 |  | 50 |  | 1866 |  | 4 |  | IMD4 |  |
|  | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 24 |  | IMD23 |  |
|  | n3 |  | 1721 |  | 10 |  | 50 |  | 1816 |  | N/A |  | N/A |  |
| DC_5_n7 | n7 |  | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |  |
|  | 5 |  | 834 |  | 5 |  | 25 |  | 879 |  | 12 |  | IMD33 |  |
| DC_5_n38 | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 12 |  | IMD33 |  |
|  | n38 |  | 2577 |  | 10 |  | 50 |  | 2577 |  | N/A |  | N/A |  |
| DC_5A_n66A | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |  |
|  | n66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |  |
| DC_5A_n77A8 DC_5A_n77(2A)8 | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |  |
|  | n77 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |  |
|  | 5 |  | 826.5 |  | 5 |  | 25 |  | 871.5 |  | 5.5 |  | IMD5 |  |
|  | n77 |  | 4177.5 |  | 10 |  | 50 |  | 4177.5 |  | N/A |  | N/A |  |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78C | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |  |
|  | n78 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |  |
| DC_7_n3 | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 13 |  | IMD4 |  |
|  | n3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |
| DC_7_n5 | 7 |  | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |  |
|  | n5 |  | 834 |  | 5 |  | 25 |  | 879 |  | 12 |  | IMD33 |  |
| DC_7A_n20A | 7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
|  | n20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
| DC_7_n40 | 7 |  | 2510 |  | 5 |  | 25 |  | 2630 |  | 23 |  | IMD3 |  |
|  | n40 |  | 2390 |  | 5 |  | 25 |  | 2390 |  | N/A |  | N/A |  |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | IMD4 |  |
|  | n66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A_n77(2A) DC_7C_n77A DC_7C_n77(2A) | 7 |  | 2540 |  | 5 |  | 25 |  | 2660 |  | 7.1 |  | IMD4 |  |
|  | n77 |  | 3870 |  | 10 |  | 50 |  | 3870 |  | N/A |  | N/A |  |
| DC_7_n79 | 7 |  | 2510 |  | 5 |  | 25 |  | 2630 |  | [8] |  | IMD4 |  |
|  | n79 |  | 4900 |  | 40 |  | 216 |  | 4900 |  | N/A |  | N/A |  |
| DC_8A_n1A | 8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |  |
|  | n1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6 |  | IMD4 |  |
| DC_8A_n3A | 8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |  |
|  | n3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |  |
|  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |
|  | n3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |  |
| DC_8A_n20A | n20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 25 |  | IMD33 |  |
|  | 8 |  | 890.5 |  | 5 |  | 25 |  | 935.5 |  | N/A |  | N/A |  |
|  | n20 |  | 847.5 |  | 5 |  | 25 |  | 806.5 |  | N/A |  | N/A |  |
|  | 8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD33 |  |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |  |
|  | n41 |  | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |
| DC_8A_n77A, DC_8A_n78A, DC_8A_n78(2A), DC_8A_n77(3A), DC_8A_SUL_n78A-n81A | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 8.3 |  | IMD4 |  |
|  | n77, n78 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | N/A |  |
| DC_8A_n79A, DC_8A_n79C, DC_8A_SUL_n79A-n81A | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 4.8 |  | IMD5 |  |
|  | n79 |  | 4532.5 |  | 40 |  | 216 |  | 4532.5 |  | N/A |  | N/A |  |
| DC_11A_n28A | 11 |  | 1430.5 |  | 5 |  | 25 |  | 1478.5 |  | N/A |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  | 25 |  | 798 |  | 10.4 |  | IMD4 |  |
| DC_12A_n77A DC_12A_n77(2A) | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 5.5 |  | IMD5 |  |
|  | n77 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |
| DC_12A_n78A | 12 |  | 710 |  | 5 |  | 25 |  | 740 |  | 5.5 |  | IMD5 |  |
|  | n78 |  | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |  |
| DC_13A_n5A | 13 |  | 783 |  | 5 |  | 25 |  | 752 |  | N/A |  | N/A |  |
|  | n5 |  | 828 |  | 5 |  | 25 |  | 873 |  | 25 |  | IMD3 |  |
| DC_13A_n7A DC_13A_n7(2A) | 13 |  | 784.5 |  | 5 |  | 25 |  | 753.5 |  | N/A |  | N/A |  |
|  | n7 |  | 2520 |  | 40 |  | 216 |  | 2640 |  | 2.5 |  | IMD5 |  |
| DC_13A_n77A | 13 |  | 784.5 |  | 5 |  | 20 |  | 753.5 |  | 5.5 |  | IMD5 |  |
|  | n77 |  | 3891.5 |  | 10 |  | 50 |  | 3891.5 |  | N/A |  | N/A |  |
| DC_14A_n5A | 14 |  | 791 |  | 5 |  | 25 |  | 761 |  | N/A |  | N/A |  |
|  | n5 |  | 836 |  | 5 |  | 25 |  | 881 |  | 25 |  | IMD3 |  |
|  | 14 |  | 795.5 |  | 5 |  | 25 |  | 765.5 |  | 25 |  | IMD3 |  |
|  | n5 |  | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |
| DC_14A_n77A DC_14A_n77(2A) | 14 |  | 795.5 |  | 5 |  | 15 |  | 765.5 |  | 5.5 |  | IMD5 |  |
|  | n77 |  | 3947.5 |  | 10 |  | 50 |  | 3947.5 |  | N/A |  | N/A |  |
| DC_18A_n3A | 18 |  | 823 |  | 5 |  | 25 |  | 868 |  | N/A |  | N/A |  |
|  | n3 |  | 1721 |  | 5 |  | 25 |  | 1816 |  | 4 |  | IMD4 |  |
| DC_18A_n77A DC_18A_n78A | 18 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |  |
|  | n77, n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_19A_n77A | 19 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 13.6 |  | IMD43 |  |
|  | n77 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_19A_n78A | 19 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 13.6 |  | IMD4 |  |
|  | n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_20A_n3A | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |
|  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |  |
|  | 20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |  |
|  | n3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |
| DC_20A_n38A | 20 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
|  | n38 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_20_n7 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
|  | n7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
| DC_20A_n8A | 20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 25 |  | IMD3 |  |
|  | n8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD3 |  |
| DC_20_n41 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12.1 |  | IMD3 |  |
|  | n41 |  | 2512 |  | 10 |  | 50 |  | 2512 |  | N/A |  | N/A |  |
| DC_20_n41 | 20 |  | 841 |  | 5 |  | 25 |  | 800 |  | 8.1 |  | IMD5 |  |
|  | n41 |  | 2564 |  | 10 |  | 50 |  | 2564 |  | N/A |  | N/A |  |
| DC_20A_n77A, DC_20A_n78A DC_20A_n78C7, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 |  | 850 |  | 5 |  | 25 |  | 809 |  | 11 |  | IMD4 |  |
|  | n77, n78 |  | 3359 |  | 10 |  | 50 |  | 3359 |  | N/A |  | N/A |  |
| DC_20A_n77A | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | 6.5 |  | IMD5 |  |
|  | n77 |  | 4159 |  | 10 |  | 50 |  | 4159 |  | N/A |  | N/A |  |
| DC_21A_n28A7 | 21 |  | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | 2.5 |  | IMD5 |  |
|  | n28 |  | 735.5 |  | 5 |  | 25 |  | 790.5 |  | N/A |  | N/A |  |
| DC_21A_n79A | 21 |  | 1457.5 |  | 5 |  | 25 |  | 1505.5 |  | 18.4 |  | IMD3 |  |
|  | n79 |  | 4420.5 |  | 40 |  | 216 |  | 4420.5 |  | N/A |  | N/A |  |
| DC_25A_n77A DC_25A-25A_n77A | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |  |
|  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 25 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8 |  | IMD4 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
|  | 25 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |  |
|  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |  |
| DC_25A_n78A DC_25A-25A_n78A | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  | 25 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 8 |  | IMD4 |  |
|  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |  |
|  | 25 |  | 1875 |  | 5 |  | 25 |  | 1955 |  | 5 |  | IMD5 |  |
|  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
| DC_26A_n41A | 26 |  | 839 |  | 5 |  | 25 |  | 884 |  | 15.6 |  | IMD33 |  |
|  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |  |
| DC_28_n50 | 28 |  | 730 |  | 10 |  | 50 |  | 775 |  | 15.3 |  | IMD 2 |  |
|  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |  |
|  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 6 |  | IMD 4 |  |
|  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |  |
|  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 0.5 |  | IMD 5 |  |
|  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |  |
| DC_28A_n51A | 28 |  | 742.3 |  | 5 |  | 25 |  | 797.3 |  | 5 |  | IMD4 |  |
|  | n51 |  | 1429.5 |  | 5 |  | 25 |  | 1429.5 |  | N/A |  | N/A |  |
| DC_26A_n77A, DC_26A_n78A | 26 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 11.1 |  | IMD4 |  |
|  | n77, n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 5.5 |  | IMD5 |  |
|  | n77, n78 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |  |
| DC_30A_n77A DC_30A_n77(2A) | 30 |  | 2310 |  | 5 |  | 25 |  | 2355 |  | 8.0 |  | IMD4 |  |
|  | n77 |  | 3487.5 |  | 10 |  | 50 |  | 3487.5 |  | N/A |  | N/A |  |
| DC_38A_n3A | n3 |  | 1713 |  | 5 |  | 25 |  | 1808 |  | 8.2 |  | IMD4 |  |
|  | 38 |  | 2617 |  | 5 |  | 25 |  | 2617 |  | N/A |  | N/A |  |
| DC_38A_n8A | 38 |  | 2617.5 |  | 5 |  | 25 |  | 2617.5 |  | N/A |  | N/A |  |
|  | n8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | 8.1 |  | IMD5 |  |
| DC_41A_n3A DC_41C_n3A | n3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |  |
|  | 41 |  | 2657.5 |  | 5 |  | 25 |  | 2657.5 |  | N/A |  | N/A |  |
| DC_42_n3 | 42 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |  |
|  | n3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 26 |  | 2nd3 |  |
|  | 42 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |  |
|  | n3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 8.0 |  | IMD4 |  |
| DC_42_n28 | 42 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |  |
|  | n28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 5.5 |  | IMD5 |  |
| DC_48A_n2A | 48 |  | 3625 |  | 20 |  | 100 |  | 3625 |  | N/A |  | N/A |  |
|  | n2 |  | 1852.5 |  | 5 |  | 25 |  | 1932.5 |  | 12 |  | IMD4 |  |
| DC_48A_n12A | 48 |  | 3557.5 |  | 10 |  | 50 |  | 3557.5 |  | N/A |  | N/A |  |
|  | n12 |  | 705.5 |  | 5 |  | 25 |  | 735.5 |  | 5.5 |  | IMD5 |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 |  | 3625 |  | 20 |  | 100 |  | 3625 |  | N/A |  | N/A |  |
|  | n25 |  | 1852.5 |  | 5 |  | 25 |  | 1932.5 |  | 12 |  | IMD4 |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 |  | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |  |
|  | n66 |  | 1715 |  | 5 |  | 25 |  | 2115 |  | 4 |  | IMD5 |  |
| DC_66A_n2A, DC_66A-66A_n2A | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |
|  | n2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |  |
|  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |  |
|  | n2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |
| DC_66A_n5A | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |  |
|  | 66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |  |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |
|  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | IMD4 |  |
| DC_66A_n25A | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |
|  | n25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |  |
|  | 66 |  | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | 23 |  | IMD3 |  |
|  | n25 |  | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |  |
|  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |  |
|  | n25 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |
| DC_66A_n46A | 66 |  | 1735 |  | 5 |  | 25 |  | 2135 |  | 12.0 |  | IMD3 |  |
|  | n46 |  | 5605 |  | 20 |  | 100 |  | 5605 |  | N/A |  | N/A |  |
| DC_66A_n48A | 66 |  | 1715 |  | 5 |  | 25 |  | 2115 |  | 4 |  | IMD5 |  |
|  | n48 |  | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |  |
| DC_66A_n71A | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 5 |  | IMD4 |  |
|  | n71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |  |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | 31.0 |  | IMD2 |  |
|  | n77 |  | 3950 |  | 10 |  | 50 |  | 3950 |  | N/A |  | N/A |  |
|  | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 5.0 |  | IMD5 |  |
|  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |
| DC_66A_n78A | 66 |  | 1730 |  | 5 |  | 25 |  | 2150 |  | 5.0 |  | IMD5 |  |
|  | n78 |  | 3660 |  | 10 |  | 50 |  | 3660 |  | N/A |  | N/A |  |
| DC_71A_n38A | 71 |  | 665 |  | 5 |  | 25 |  | 619 |  | 11 |  | IMD4 |  |
|  | n38 |  | 2614 |  | 10 |  | 50 |  | 2614 |  | N/A |  | N/A |  |
| DC_71A_n41A | 71 |  | 666 |  | 5 |  | 25 |  | 620 |  | 11 |  | IMD4 |  |
|  | n41 |  | 2618 |  | 5 |  | 25 |  | 2618 |  | N/A |  | N/A |  |
| DC_71A_n66A | 71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |  |
|  | n66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 5 |  | IMD4 |  |
| DC_71A_n77A8 | 71 |  | 671 |  | 5 |  | 25 |  | 625 |  | 5.5 |  | IMD5 |  |
|  | n77 |  | 3309 |  | 10 |  | 50 |  | 3309 |  | N/A |  | N/A |  |
| DC_71A_n78A | 71 |  | 681.5 |  | 5 |  | 25 |  | 635.5 |  | 5.5 |  | IMD5 |  |
| DC_71A_n78(2A) | n78 |  | 3361.5 |  | 10 |  | 50 |  | 3361.5 |  | N/A |  | N/A |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

<<< End of changed sections >>>