3GPP TSG-RAN WG4 Meeting #111	R4-2407143

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	Supporting Simulations for NR NTN UE Demodulation

Agenda item:	7.16.8.2

Document for:	Information

Introduction

In this contribution we present the results of our simulation campaign based on the parameters agreed during RAN4 discussions.

Discussion

PDSCH Results

Below are the results for PDSCH, based on the simulation parameters discussed during RAN4#110-bis.

| MCS | HARQ Process | Channel | Antenna Configuration | Ideal Result SINR [dB] @70% TP | Impairment |
| --- | --- | --- | --- | --- | --- |
| MCS4 | 16 HARQ process | NTN-TDLC-5-600 | 1x1 | 0.7 | 3.4 |
|  |  | NTN-TDLC-5-1200 |  | 0.8 | 3.5 |
|  |  | NTN-TDLC-5-2000 |  | 0.9 | 3.6 |
|  |  | NTN-TDLC-5-600 | 1x2 | -2.4 | 0.4 |
|  |  | NTN-TDLC-5-1200 |  | -2.3 | 0.4 |
|  |  | NTN-TDLC-5-2000 |  | -2.1 | 0.6 |
| MCS13 |  | NTN-TDLC-5-600 | 1x1 | 8.6 | 11.3 |
|  |  | NTN-TDLC-5-1200 |  | 8.7 | 11.4 |
|  |  | NTN-TDLC-5-2000 |  | 8.3 | 11.0 |
|  |  | NTN-TDLC-5-600 | 1x2 | 5.1 | 7.8 |
|  |  | NTN-TDLC-5-1200 |  | 5.3 | 8.0 |
|  |  | NTN-TDLC-5-2000 |  | 5.4 | 8.1 |
| MCS4 | 32 HARQ process | NTN-TDLC-5-600 | 1x1 | -2.4 | 0.4 |
|  |  | NTN-TDLC-5-1200 |  | 0.9 | 3.6 |
|  |  | NTN-TDLC-5-2000 |  | -2.1 | 0.6 |
|  |  | NTN-TDLC-5-600 | 1x2 | -2.4 | 0.4 |
|  |  | NTN-TDLC-5-1200 |  | -2.3 | 0.4 |
|  |  | NTN-TDLC-5-2000 |  | -2.1 | 0.6 |
|  | 16 HARQ process (4 w/ feedback disabled) | NTN-TDLC-5-600 | 1x1 | 0.7 | 3.4 |
|  |  | NTN-TDLC-5-1200 |  | 0.8 | 3.5 |
|  |  | NTN-TDLC-5-2000 |  | 0.9 | 3.6 |
|  |  | NTN-TDLC-5-600 | 1x2 | -2.4 | 0.4 |
|  |  | NTN-TDLC-5-1200 |  | -2.2 | 0.5 |
|  |  | NTN-TDLC-5-2000 |  | -2.1 | 0.6 |

Conclusion

In this contribution we have presented Nokia's results from our simulation campaign for NR NTN UE demodulation.

No observations or proposals were made.