# 3GPP TSG-RAN WG4 Meeting # 111                                                             R4-2407172

Fukuoka City, Fukuoka, Japan, 20th - 24th May, 2024

# Source:	Mediatek Inc., ZTE Corporation

# Title:	Discussion on IMD4 MSD for CA_n41A-n79C and CA_n41C-n79A

# Agenda Item:	6.1.1.1

# Document for:	Approval

- Introduction

The BCS4 and 5 for CA_n41A-n79C and CA_n41C-n79A were requested and included in the basket WID, this contribution provides evaluation on MSD in order to introduce CA_n41A-n79C and CA_n41C-n79A in TR38.718-02-01.

The TP for CA_n41A-n79C and CA_n41C-n79A were provided in previous meeting. However, the technical analysis to check the MSD issues was missing considering the n41 and n79 support simultaneous Rx/Tx operation. There’s MSD due to CA_n41C UL IMD4 fall in n79 DL range. The contribution provide evaluation on the MSD due to UL aggressor IM4 with the assumption of separate antenna architecture. The PC3 n41C UL IMD4 were measured at HB PAM output port and the measured tone level IMD4 is -73.7dBm. Meanwhile there’s IMD4 generated due to n79 LNA IIP4 linearity. The n79 LNA IIP4 linearity may dominate the whole MSD calculation.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n79 filter rejection on n41 range | 40 | dB |
| n41 PA module UL IMD4 at n79 DL range | -73.7 | dBm |
| Front-end loss | 4 | dB |
| n79 Rx chain cascaded IIP4 | -14 | dBm |
| IMD4 generated by n79 LNA due to n41 aggressor at n79 LNA input | -78 | dBm |
| PA RXBN noise | -130 | dBm/Hz |
| Thermal noise at n79 RX ANT port | -165.6 | dBm/Hz |
| Transceiver effective phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

With above parameters, we calculate IMD4 MSD due to uplink n41C aggressor as below:

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| UL_CA_n41C TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n41PA output noise power at n79 LNA input port | -90.1 | -90.1 | dBm/40MHz |
| n79 LNA IM4 due to n41 uplink | -95 | -95 | dBm/40MHz |
| Thermal noise at RX ant port | -165.6 |  | dBm/Hz |
| IMD4 at n79 LNA input due to n41 PA IMD4 | -87.7 | -87.7 | dBm |
| IMD4 at n79 LNA input due to n79 LNA IIP4 caused by n41 aggressor CCs | -78 | -78 | dBm |
| Total noise level refer to n79 LNA input port | -77 | -77 | dBm |
| MRC Combined |  |  |  |
| MSD (40MHz BW) | 12.6 |  | dB |

Table 2 Link analysis for Band n79 40MHz REFSENS calculation

Proposal 1: IMD4 MSD due to UL_CA_n41C in n79 DL as the value below,

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 12.615 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

- Conclusion

Proposal 1: IMD4 MSD due to UL_CA_n41C in n79 DL as the value below,

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 12.615 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

- Reference

[1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

[2] R4-2401274 TP for TR38.718-02-01_CA_n41A-n79C and CA_n41C-n79A