# 3GPP TSG-RAN WG4 Meeting #111	R4-2407251

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.16.8.2

Source:	Apple

Title:	Simulation results for FR2 NTN

Release:	Rel-18

Document for:	Discussion

# Introduction

In RAN4#110bis UE demodulation performance requirements for eNTN were discussed and way forward [1] was agreed.  Simulation assumptions were agreed in [2]. In this contribution we present simulation results for demodulation requirements for eNTN.

# Simulation Results

For the simulation assumptions agreed in [1] we provide initial simulation results for eNTN in the table below.

Table 1: Simulation results for eNTN-PDSCH

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | HARQ Config | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% of Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| 1-1 | 200 / 120 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLC5-1200 | 1x1 Low | 0.5 | 2.5 |
| 1-2 | 200 / 120 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-1200 | 1x1 Low | 8.2 | 10.2 |
| 1-3 | 200 / 120 | QPSK, 0.30 | 32 HARQ Proc | NTN-TDLC5-1200 | 1x1 Low | 0.5 | 2.5 |
| 1-4 | 200 / 120 | QPSK, 0.30 | Disabled HARQ | NTN-TDLC5-1200 | 1x1 Low | 1.3 | 3.3 |

Table 2: Simulation results for eNTN-PDCCH

| Bandwidth (MHz)/ SCS (KHz) | CORESET RB | CORESET duration | Aggregation level | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | SNR @ 1% of Pm-dsg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Alignment | Impairment |
| 200/120 | 132 | 1 | 8 | 1_0 | 42 | NTN-TDLC5-1200 | 1x1 Low | 2.5 | 4.5 |
| 200/120 | 132 | 2 | 16 | 1_1 | 56 | NTN-TDLC5-1200 | 1x1 Low | 0.4 | 2.4 |

# Conclusion

In this contribution we present simulation results for demodulation requirements for eNTN.

# Reference

- R4-2406024, “Way Forward for [110bis][325] NR_NTN_enh_SAN_UE_demod”, Huawei.

- R4-2405146, “Simulation assumption on demodulation requirements for NR NTN enhancements”, Huawei