3GPP TSG-RAN WG4 Meeting #111                                    	                          R4-2407267

# Fukuoka, Japan, May 20 – May 24, 2024

Agenda item:	7.21.2.1

Source:	Apple Inc.

Title:	Simulation Results for Enhanced Support of RedCap

Release:	Rel-18

Document for:	Discussion

# Ideal Simulation Results

PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 1.78 | -0.82 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 10.62 | 6.54 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 15.6 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 12 | 9480 | TDLA30-10 | - | 18 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 1.03 | -0.93 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 10.92 | 6.75 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 15.4 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 6 | 4736 | TDLA30-10 | - | 17.7 |

PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate |  | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 |  | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 1.57 | -0.52 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 |  | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 10.98 | 6.91 |
| 64QAM | 1Rx | MCS19 in Table 1 |  | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 15.6 | - |
| 64QAM | 2Rx | MCS19 in Table 1 |  | 0.5 | Rank 2 | 12 | 9480 | TDLA30-10 | - | 18 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 0.73 | -1.0 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 10.7 | 6.64 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 15.4 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 6 | 4736 | TDLA30-10 | - | 17.7 |

# Impairment Simulation Results

PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 3.78 | 1.18 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 12.62 | 8.54 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 17.6 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 12 | 9480 | TDLA30-10 | - | 20 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 3.03 | -0.93 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 12.92 | 6.75 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 15.4 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 6 | 4736 | TDLA30-10 | - | 17.7 |

PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate |  | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 |  | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 3.57 | -0.52 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 |  | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 12.98 | 8.91 |
| 64QAM | 1Rx | MCS19 in Table 1 |  | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 17.6 | - |
| 64QAM | 2Rx | MCS19 in Table 1 |  | 0.5 | Rank 2 | 12 | 9480 | TDLA30-10 | - | 20 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR 1Rx | SNR 2Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 2.73 | 1.0 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 12.7 | 8.64 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 17.4 | - |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 2 | 6 | 4736 | TDLA30-10 | - | 19.7 |

# References

- RP-222675, “New WID on enhanced support of reduced capability NR devices”, Ericsson, RAN#97e.