3GPP TSG-RAN WG4 Meeting # 111                               							 R4-2407279

Fukuoka, Japan, 20th – 24th, April 2024

Source: 	Apple

Title: 	On intra-band CA for ATG UE

Agenda Item:	10.8.2.1

Document for:	Discussion

# Introduction

Intra-band CA requirements for ATG have been discussed in the last RAN4 meeting and the agreements were captured in the WF [1]. The following open issues are for further study.

Issue 3-2-2: BCS for DL CA_n79C

Issue 3-2-3: initial analysis for ATG Rx requirements for intra-band CA

This contribution will give further analysis on these open issues and presents our proposals.

# Discussion

## BCS

The following 2 options for BCS are open for further study. Option 1 includes all channel bandwidths in Band n79 and option 2 only includes larger channel bandwidth (>50MHz) in Band n79. We think option 1 is not cost effective considering that the ATG deployment purpose is to address in-flight MBB demand and Band n79 has abundant spectrum. Hence, we prefer to adopt option 2. While if operator prefer to include all CBW in Band n79 we are also ok to compromise to option 1.

Proposal 1: it is proposed to adopt option 2 for ATG intra-band CA operation in Band n79.

  - Option 1:

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | - | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 4: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |  |  |  |  |

  - Option 2:

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA config | Uplink CA config | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | BCS |
| CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |

## Rx RF requirement

Table 2.2-1 Rx requirements for ATG UE

| Sub-clause | Requirement | Discussion |
| --- | --- | --- |
| 7 | Receiver characteristic |  |
| 7.1 | General | No impact due to introduction of CA. |
| 7.2 | Diversity characteristics | For TN UE, two Rx, four Rx and eight Rx requirements are specified both for single carrier and for CA. For ATG UE, only two Rx and four Rx are considered. The CA requirement should also be based on two Rx and four Rx (optional) requirements. |
| 7.3 | Reference sensitivity | TN UE REFSENS recruitment is reused for ATG UE single carrier operation. For CA, the same logic is applicable. E.g. the requirement specified in 7.4A.1 for intra-band contiguous CA can be reused for ATG UE. |
| 7.4 | Maximum input level | Specific requirement on maximum input level is specified based on ATG scenario link budget calculation. For intra-band contiguous CA, the Power in largest transmission bandwidth configuration CC and other CCs need to be scaled for ATG UE CA based on the single carrier level. Specific CA requirement need to be specified for ATG UE. |
| 7.5 | Adjacent channel selectivity | TN UE Case 1 ACS is completely reused for ATG UE for single carrier operation. ATG Case 2 ACS is specified based on TN case 2 ACS but scaled the interference power level based on ATG maximum input level. For ATG UE CA, specific case 2 ACS requirement need to be defined. |
| 7.6 | Blocking characteristic | In-band blocking requirement and out-of-band blocking requirement are reused from TN UE single carrier requirement for ATG UE For CA, the requirement specified in 7.6A.2 for intra-band contiguous CA can be applied for ATG UE. |
| 7.7 | Spurious response | Spurious response requirement is reused from TN UE single carrier requirements for ATG UE. For CA, the requirement specified in 7.7A.2 for intra-band contiguous CA can be applied for ATG UE. |
| 7.8 | Intermodulation characteristics | Intermodulation requirement is reused from TN UE single carrier requirements for ATG UE. For CA, the requirement specified in 7.8A.2 for intra-band contiguous CA can be applied for ATG UE. |
| 7.9 | Spurious emission | Spurious emission requirement is reused from TN UE single carrier requirements for ATG UE. No requirement is needed for intra-band contiguous CA following the TN UE methodology. |

Proposal 2: it is proposed to use the analysis in Table 2.2-1 as the baseline for ATG UE Rx RF requirements development.

# Summary

This contribution presented our consideration on the open issues for ATG UE supporting intra-band CA. The following proposals are concluded.

Proposal 1: it is proposed to adopt option 2 for ATG intra-band CA operation in Band n79.

Proposal 2: it is proposed to use the analysis in Table 2.2-1 as the baseline for ATG UE Rx RF requirements development.

# References

[1] RP-2406594, WF on Rel-19 ATG UE requirements, RAN4#110bis