3GPP TSG-RAN WG4 Meeting #111              	R4-2407341

Fukuoka City, Fukuoka, Japan, 20th-24th May, 2024

Agenda item:	7.8.7.1

Source:	Samsung

Title:	Discussion and simulation results for dedicated spectrum less than 5MHz

Document for:	Discussion

- Introduction

In this contribution, we provide our simulation results regarding PDCCH and PBCH for dedicated spectrum less than 5MHz are provided.

# 2	Discussion

## 2.1	PDCCH

In RAN4#110bis meeting, the WF[1] was approved with the following agreements as:

Issue 1-1-1: Number of Tx in PDCCH requirements Agreement: - Define two test cases for punctured PDCCH in less than 5 MHz CBW with 1x2 and 2x4 antenna configurations: Number of Tx; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLA30-10; 1x2 Low; 1; [TBA]. 2; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLC300-100; 2x4 Low; 1; [TBA]. Way forward: - Companies are encouraged to clarify further the requirement value selection and simulation results depending on how punctured CCE is treated. Issue 1-1-2: PDCCH requirements in HST conditions Agreement: - Not to introduce HST scenario for PDCCH requirements.

In RAN4#110bis meeting, the simulation results for alignment were observed that it was divided into two camps. To understand the results some companies raised questions regarding how punctured CCE was treated in the simulation. In this regards, we would like to clarify how punctured CCE is applied in our results.

In RAN4#110 meeting, RAN4 achieves agreement on simulation assumption for punctured PDCCH as follows:

Agreement: - Define punctured PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW with the following parameters: - 15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs), 2Rx/4Rx; - Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0

As highlighted above, we have used CCE#4, #5, #6, and partial CCE#7, which consist of one transmitted PRB and one punctured PRB, as illustrated in Figure 1, where RB14 is transmitted, while RB15 is punctured in CCE#7. Taking into account partial CCE, the results of non-punctured and punctured PDCCH are provided in Figure 1. The difference of SNR at 1% BLER is approximately 0.7 to 0.8 dB, which are reasonable results.

Figure 1 Illustration of partial CCE and PCCCH BLER

Observation 1: CCE#4, #5, #6, and partial CCE#7, which consist of one transmitted PRB and one punctured PRB are used for PDCCH simulation results

The simulation results less than 5MHz PDCCH with impairment result are provided as follows:

Proposal 1: Consider following results for PDCCH requirements:

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 5 | 7 |
| 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -3.3 | -1.3 |

## 2.2	PBCH

The simulation result less than 5MHz PBCH with impairment results are provided as follows:

Proposal 2: Consider following results for PBCH requirements:

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  | Impaired SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -6 | -4 |
|  |  |  |  | 1x4 Low | 1 | -8.7 | -6.7 |

# 3	Conclusion

In this contribution, our view on UE demodulation requirement is provided.

Observation 1: CCE#4, #5, #6, and partial CCE#7, which consists of one transmitted PRB and one punctured PRB are used for PDCCH results

Proposal 1: Consider following results for PDCCH requirements:

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 5 | 7 |
| 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -3.3 | -1.3 |

Proposal 2: Consider following results for PBCH requirements:

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  | Impaired SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -6 | -4 |
|  |  |  |  | 1x4 Low | 1 | -8.7 | -6.7 |

# References

R4- 2406044, “Way Forward for [110bis][323] NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell