3GPP TSG-RAN WG4 Meeting # 111								            R4-2407415

Fukuoka, Japan, 20 – 24 May 2024

Title: 	On UE IMT parameters for 7125-8400MHz for LS and WI phase

Source: 	Skyworks Solutions Inc.

Agenda Item:	10.3.3 Study the IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range [FS_NR_IMT_4400_7125_14800MHz]

Document for:	Approval

# 1	Background

In RAN4#110b, a way forward [1] captured the consensus and open issues for the IMT parameter for the frequency ranges other than 4400-4800MHz. For the 7125-8400MHz range, we provided our view in terms of mandatory features and optional features. In this contribution, we simplify our proposals from [2] by clearly stating what should be part of the response LS versus aspects that may need to be studied in the WI phase.

# Discussion

## 2.1 Way forward input

The way forward [1] aspects relevant to the UE are copied here and our position stated.

Way forward:

-	n104 as starting point

o	Identify which parameters need further discussion and could differ from n104

Issue 2-1 Duplex Method

- Option 1: Align with 4GHz outcome, i.e., TDD as baseline, check SBFD

- Option 2: Follow n104

Proposal on duplex method:

- TDD as baseline and used for the LS response.

- SBFD may be further studied in the WI phase.

Issue 2-2 Typical Channel Bandwidth (100MHz is maximum bandwidth)

- n104 (100MHz) baseline

  - FFS higher bandwidths (e.g. 200MHz)

Proposal on duplex method:

- 100MHz as baseline and used for the LS response.

- 200MHz may be further studied in the WI phase according to SCS and actual spectrum availability.

Issue 2-3 Transmit bandwidth configuration (Signal bandwidth)

- n104 spectrum utilization assumed

  - FFS check for higher bandwidth

Proposal on signal BW:

- 100MHz RB BW of 273RB/30kHz SCS as baseline and can be used for the LS response.

- 200MHz may be further studied in the WI phase according to SCS and actual spectrum availability and use an equivalent signal BW of 2x273RB/30kHz SCS.

Issue 2-5 Spectrum mask

- UE: n104 as basis for emissions levels

Observation: If there is an ACLR relaxation without SEM relaxation, the benefits will be limited and probably non-existent for higher power classes than PC3.

Proposal on UE spectrum mask:

- n104 SEM as baseline and can be used for the LS response.

- SEM mask relaxation may be further studied in the WI phase together with ACLR relaxation.

Issue 2-6 ACLR

- UE

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

Proposal on UE ACLR:

- Since this is consistent with our earlier response to ITU for the 6-24GHz range 26dB or 27dB PC3 ACLR is baseline and can be used for the LS response.

- ACLR relaxation for PC2 and PC1.5 may be further studied in the WI phase together with SEM relaxation.

Issue 2-8 Maximum output power (UE)

- Option 1: 23dBm only.

- Option 2: 20dBm

- Option 3: Use n104 (23 and 26dBm)

- Option 4: 29dBm

Proposal on UE maximum power:

- 23dBm as baseline and can be used for the LS response.

- PC2 and PC1.5 w/wo multiple Tx may be further studied in the WI phase including related SEM and ACLR relaxations.

Issue 2-9 Noise Figure

- UE

  - Option 1: Follow n104 noise figure (12dB)

  - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB

  - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz,

Proposal on UE noise figure:

- Option 3 (NF was 9-13dB) is consistent with former LS covering this frequency range and compatible with n104 and can be used for the LS response.

- PC2 and PC1.5 w/wo multiple Tx may be further studied in the WI phase.

# Conclusion

In this contribution we have discussed the baseline and optional performance parameters for the 7125-8400MHz frequency range with the following proposals in terms of LS response and potential studies for the WI phase.

Proposal on duplex method:

- TDD as baseline and used for the LS response.

- SBFD may be further studied in the WI phase.

Proposal on duplex method:

- 100MHz as baseline and used for the LS response.

- 200MHz may be further studied in the WI phase according to SCS and actual spectrum availability.

Proposal on signal BW:

- 100MHz RB BW of 273RB/30kHz SCS as baseline and can be used for the LS response.

- 200MHz may be further studied in the WI phase according to SCS and actual spectrum availability and use an equivalent signal BW of 2x273RB/30kHz SCS.

Proposal on UE spectrum mask:

- n104 SEM as baseline and can be used for the LS response.

- SEM mask relaxation may be further studied in the WI phase together with ACLR relaxation.

Proposal on UE ACLR:

- Since this is consistent with our earlier response to ITU for the 6-24GHz range 26dB or 27dB PC3 ACLR is baseline and can be used for the LS response.

- ACLR relaxation for PC2 and PC1.5 may be further studied in the WI phase together with SEM relaxation.

Proposal on UE maximum power:

- 23dBm as baseline and can be used for the LS response.

- PC2 and PC1.5 w/wo multiple Tx may be further studied in the WI phase including related SEM and ACLR relaxations.

Proposal on UE noise figure:

- Option 3 (NF was 9-13dB) is consistent with former LS covering this frequency range and compatible with n104 and can be used for the LS response.

- PC2 and PC1.5 w/wo multiple Tx may be further studied in the WI phase.

# References

[1] R4-2406615 WF on IMT parameters for other frequency ranges, CATT, RAN4#110b

[2] R4-2404246 On UE IMT parameters for 7125-8400MHz based on n104 extrapolation, Skyworks Solutions Inc., RAN4#110b