3GPP TSG-RAN WG4 Meeting # 111								            R4-2407416

Fukuoka, Japan, 20 – 24 May 2024

Title: 	On UE IMT parameters for 14800-15350MHz for LS and WI phase

Source: 	Skyworks Solutions Inc.

Agenda Item:	10.3.4 Study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range [FS_NR_IMT_4400_7125_14800MHz]

Document for:	Approval

# 1	Background

In RAN4#110b, a way forward [1] captured the consensus and open issues for the IMT parameter for the frequency ranges other than 4400-4800MHz. For the 14800-15350MHz range we provided our view in terms of mandatory features, and optional features but mostly in terms of studies in [2]. In this contribution, we simplify our proposals by clearly stating what should be part of the response LS versus aspects that may need to be studied in the WI/SI phase.

# Discussion

## 2.1 Way forward input

The way forward [1] aspects relevant to the UE are copied here and our position stated.

Way forward

-	Further discussing the following issues

Issue 3-1 Common understanding on UE antenna array options

- Option 1: “UE beamforming” (FR2 like)

  - Assumed to be FR2 like UE, with two panels similar to Rel-14 NR study 38.803

  - Analogue like beamforming

  - Option 1a: 2x2 antennas per panel

- Option 2: FR1 like

  - Assumed to be FR1 like UE with more than one TX/RX antenna

  - Isotropic antennas

  - Option 2a: 2 TX

  - Option 2b: 4 TX

  - Option 2c: 4 RX

  - Option 2d: 6 RX

  - Option 2e: 8 RX

Observations:

- The majority of UEs may still be small form factors UE in this frequency range including some FWA UEs

- Within the small form factors UEs, there may be still some UEs that implement a single Tx

- We recognize that other larger form factors UEs could enable an antenna array for beamforming in a similar way as FR2.

- The number of Tx and Rx are independent options

- Based on this, the IMT studies should be based on the minimum implementation set which is FR1 like 1Tx and 4Rx UE.

- Still, we are convinced there are significant cell range benefit and potential ACLR/SEM relaxations to consider digital single stream beam forming with multiple Tx.

Proposal for 14800-15350MHz:

- LS response to WP5D should be based on minimum set implementation: FR1 like, 1Tx PC3 and 4Rx

- In the SI phase coexistence studies assume the same UE characteristics than in the LS to derive ACLR, ACS requirements. Applicable relaxations to SEM are also studied.

- In the WI phase:

  - Single stream digital beam forming based on 2Tx and 4Tx may be studied together with further ACLR/ACS relaxations.

  - 6/8Rx may also be studied.

  - Phase array beam forming may be studied for large UE form factors.

# Conclusion

In this contribution we have discussed the baseline and optional performance parameters for the 14800-15350MHz frequency range with the following proposals in terms of LS response and potential studies for the SI/WI phase.

Proposal for 14800-15350MHz:

- LS response to WP5D should be based on minimum set implementation: FR1 like, 1Tx PC3 and 4Rx

- In the SI phase coexistence studies assume the same UE characteristics to derive ACLR, ACS requirements. Applicable relaxations to SEM are also studied.

- In the WI phase:

  - Single stream digital beam forming based on 2Tx and 4Tx may be studied together with further ACLR/ACS relaxations.

  - 6/8Rx may also be studied.

  - Phase array beam forming may be studied for large UE form factors.

# References

[1] R4-2406615 WF on IMT parameters for other frequency ranges, CATT, RAN4#110b

[2] R4-2404245 On UE IMT parameters for 14800-15350MHz, Skyworks Solutions Inc., RAN4#110b