3GPP TSG-RAN WG4 Meeting # 111                                                                R4-2407525

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title: 	                 Discussion on deployment scenarios and spectrum usage for A-IoT

Source: 	CATT

Agenda item:	10.13.2.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for AIoT deployment scenarios and spectrum usage according to the WF [2].

- Spectrum usage

  - Possible spectrum usage cases

In this paper, we provide some analysis of spectrum usage for the A-IoT operation from interference perspectives. We analyze the D1T1 and D2T2 cases which use device 1 and device 2a, both D1T1 and D2T2 include two scenarios, i.e. CW transmitting from inside or outside the topology, each scenario has four cases according to R2D spectrum and CW/D2R spectrum, D2R backscattering is transmitted in the same carrier as CW.

Table 1: Possible spectrum usage cases

| Spectrum usage | Case in analysis | R2D spectrum | CW/D2R spectrum |
| --- | --- | --- | --- |
| Case 1 | Case 1-1, Case1-2, Case 1-3, Case 1-4 | DL | DL |
| Case 2 | Case 2-1, Case2-2, Case 2-3, Case 2-4 | UL | DL |
| Case 3 | Case 3-1, Case 3-2, Case 3-3, Case 3-4 | UL | UL |
| Case 4 | Case 4-1, Case 4-2, Case 4-3, Case 4-4 | DL | UL |

  - Interference analysis for different cases

The interference links for different cases are provided in Table 2 and Table 3. For D1T1, it’s assumed that NR gNB and AIoT reader are co-site.

Table 2: Interference of each case, D1T1

| CW |  | R2D spectrum | CW/D2R spectrum | Victim link |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Reader D2R | Device CW | Device R2D | Legacy gNB UL | Legacy UE DL |
| CW is transmitted from inside the topology | Case 1-1 | DL | DL | SI: gNB→D2R CW→D2R R2D→D2R | gNB→CW | gNB→R2D | No interference signal | R2D→UE CW→UE D2R→UE |
|  | Case 2-1 | UL | DL | SI: gNB→D2R CW→D2R | gNB→CW | UE→R2D | SI: R2D→gNB | CW→UE |
|  | Case 3-1 | UL | UL | UE→D2R CW→D2R R2D→D2R | UE→CW | UE→R2D | SI: CW→gNB SI: R2D→gNB D2R→gNB | No interference signal |
|  | Case 4-1 | DL | UL | UE→D2R CW→D2R | UE→CW | gNB→R2D | SI: CW→gNB D2R→gNB | R2D→UE |
| CW is transmitted from outside the topology | Case 1-2 | DL | DL | SI: gNB→D2R CW→D2R R2D→D2R | gNB→CW | gNB→R2D | No interference signal | R2D→UE CW→UE D2R→UE |
|  | Case 2-2 | UL | DL | SI: gNB→D2R CW→D2R | gNB→CW | UE→R2D | SI: R2D→gNB | CW→UE |
|  | Case 3-2 | UL | UL | UE→D2R CW→D2R R2D→D2R | UE→CW | UE→R2D | CW→gNB SI: R2D→gNB D2R→gNB | No interference signal |
|  | Case 4-2 | DL | UL | UE→D2R CW→D2R | UE→CW | gNB→R2D | CW→gNB D2R→gNB | R2D→UE |

Table 3: Interference of each case, D2T2

| CW | Case | R2D spectrum | CW/D2R spectrum | Victim |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Reader D2R | Device CW | Device R2D | Legacy gNB UL | Legacy UE DL |
| CW is transmitted from inside the topology | Case 1-3 | DL | DL | gNB→D2R CW→D2R R2D→D2R | gNB→CW | gNB→R2D | No interference signal | R2D→UE CW→UE D2R→UE |
|  | Case 2-3 | UL | DL | gNB→D2R CW→D2R | gNB→CW | UE→R2D | R2D→gNB | CW→UE |
|  | Case 3-3 | UL | UL | UE→D2R CW→D2R R2D→D2R | UE→CW | UE→R2D | CW→gNB R2D→gNB D2R→gNB | No interference signal |
|  | Case 4-3 | DL | UL | UE→D2R CW→D2R | UE→CW | gNB→R2D | CW→gNB D2R→gNB | R2D→UE |
| CW is transmitted from outside the topology | Case 1-4 | DL | DL | gNB→D2R CW→D2R R2D→D2R | gNB→CW | gNB→R2D | No interference signal | R2D→UE CW→UE D2R→UE |
|  | Case 2-4 | UL | DL | gNB→D2R CW→D2R | gNB→CW | UE→R2D | R2D→gNB | CW→UE |
|  | Case 3-4 | UL | UL | UE→D2R CW→D2R R2D→D2R | UE→CW | UE→R2D | CW→gNB R2D→gNB D2R→gNB | No interference signal |
|  | Case 4-4 | DL | UL | UE→D2R CW→D2R | UE→CW | gNB→R2D | CW→gNB D2R→gNB | R2D→UE |

  - Observations and proposal

For the interferences to reader D2R reception, gNB output power is larger than UE, so gNB has greater interference to reader D2R. In addition, both case1 and case2 have SI problem, similar to SBFD, which should be considered in the RF analysis or co-existence study. From this aspect Case4 and Case3 brings less problem, than case2 and case1.

For the interferences to device CW and device R2D, the interference level depends on the deployment scenario of the legacy system and the A-IoT system. For D1T1, if the interference signal power of gNB to device is greater than that of UE to device, Case 3 is best.

For the interferences to legacy gNB UL reception, since both R2D and CW/D2R use  DL spectrum, there’s no interference for case 1. The distance from the Device to gNB is relatively large, so the interference signal power from the device to gNB is relatively small. The interference signal power levels from low to high are case1, case2, case4, case3.

For interferences to legacy UE DL reception, since both R2D and CW/D2R use  UL spectrum, Case3 has no interference signal. The distance from the transmitting source of CW and R2D to UE is longer than that from the Device to UE, so the corresponding interference level is lower,  the interference signal power level from low to high are case3, case4, case2, case1.

Observation 1: For interferences to reader D2R reception, the interference signal power level from low to high are case4, case3, case2, case1.

Observation 2: For interferences to device CW reception and device R2D reception, the interference situation depends on the deployment scenario of the legacy system and the A-IOT system.

Observation 3: For interferences to device CW reception and device R2D reception, it’s difficult to say which case is better depending on the deployment scenarios.

Observation 4: For interferences to legacy gNB UL reception, the interference signal power level from low to high are case1, case2, case4, case3.

Observation 5: For interferences to legacy UE DL reception, the interference signal power level from low to high are case3, case4, case2, case1.

From above observations, all of the cases in the Table 1 has advantages and drawbacks. However, considering legacy UE DL should be protected in high priority, both R2D and CW/D2R are transmitted in UL spectrum is a relatively better choice. So we have the following proposal,

Proposal 1: Both R2D and CW/D2R are transmitted in UL spectrum if minimum interference to legacy UE reception is intended.

- Deployment scenarios

For the deployment scenarios of AIoT, there’re many possibilities. Some of them are defined in the LS from RAN1 [3]. However, there’s no clarification how legacy NR gNB is located with AIoT BS. In the last RAN4 meeting, there were some agreements that the following

Usually the worst case of the deployment scenarios is simulated in the study for the new feature, but in this AIoT study, the scenarios in high priority may not be worst case. The following is one of the agreements,

- Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation

  - A-IoT is located within a NR transmission bandwidth configuration

  - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS

In the WID, following objective is captured,

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

“co-site” is captured in the WID, but there is a contribution intended to clarify it but it was not discussed in last meeting. It seems the priorities for the co-existence simulation in last RAN4 meeting are not the worst scenarios.

Observation 6: The “co-site” in the WID needs clarification.

Proposal 2: If “co-site” scenario needs further study including co-existence and guard RB should be decided.

- Summary

This contribution provides our analysis for deployment scenarios and spectrum usage of A-IoT. We have the following observations and proposals.

For spectrum usage,

Observation 1: For interferences to reader D2R reception, the interference signal power level from low to high are case4, case3, case2, case1.

Observation 2: For interferences to device CW reception and device R2D reception, the interference situation depends on the deployment scenario of the legacy system and the A-IOT system.

Observation 3: For interferences to device CW reception and device R2D reception, it’s difficult to say which case is better depending on the deployment scenarios.

Observation 4: For interferences to legacy gNB UL reception, the interference signal power level from low to high are case1, case2, case4, case3.

Observation 5: For interferences to legacy UE DL reception, the interference signal power level from low to high are case3, case4, case2, case1.

Proposal 1: Both R2D and CW/D2R are transmitted in UL spectrum if minimum interference to legacy UE reception is intended.

For deployment scenarios,

Observation 6: The “co-site” in the WID needs clarification.

Proposal 2: If “co-site” scenario needs further study including co-existence and guard RB should be decided.

Reference

[1] RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, RAN#103

[2] R4-2406714, “WF on Ambient IoT in NR”, RAN4#110bis

[3] R1-2403782, “LS on Ambient-IoT evaluation scenarios and assumptions”, RAN1#116bis

Annex A: Detail interference analysis for Cases of D1T1 and D2T2

A.1 Interference analysis for D1T1, CW is transmitted from inside the topology

A.1.1 R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

Figure 1: R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

A.1.2 R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

Figure 2: R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

A.1.3 R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

Figure 3: R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

A.1.4 R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum

Figure 4: R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum

A.2 Interference analysis for D1T1, CW is transmitted from outside the topology

A.2.1 R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

Figure 5: R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

A.2.2 R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

Figure 6: R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

A.2.3 R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

Figure 7: R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

A.2.4 R2D is FDD DL spectrum, CW/D2R are FDD UL spectrum

Figure 8 R2D is FDD DL spectrum, CW/D2R are FDD UL spectrum

A.3 Interference analysis for D2T2, CW is transmitted from inside the topology

Figure 8: R2D is FDD DL spectrum, CW/D2R are FDD UL spectrum

A.3.1 R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

Figure 9: R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

A.3.2 R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

Figure 10: R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

A.3.3 R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

Figure 11: R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

A.3.4 R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum

Figure 12: R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum

A.4 Interference analysis for D2T2, CW is transmitted from outside the topology

A.4.1 R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

Figure 13: R2D is in FDD DL spectrum, CW/D2R are in FDD DL spectrum

A.4.2 R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

Figure 14: R2D is in FDD UL spectrum, CW/D2R are in FDD DL spectrum

A.4.3 R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

Figure 15: R2D is in FDD UL spectrum, CW/D2R are in FDD UL spectrum

A.4.4 R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum

Figure 16: R2D is in FDD DL spectrum, CW/D2R are in FDD UL spectrum