3GPP TSG-RAN WG4 Meeting #111	R4-2407542

Fukuoka, Japan, May 20th ‒ 24th, 2024

Title: 	Table modifications for UE RF specs improvement

Agenda Item:	12.1.1.1

Source: 	CATT

Document for:	Approval

# 1 Introduction

RAN#103 assigns RAN4 with the task of exploring potential methods to enhance the quality of RAN4 specifications [1]. Subsequently, RAN4#110bis has commenced discussions in alignment with this directive, leading to an agreed way forward [2], where our proposal in our paper R4-2404448 on table modification is captured [3].

In this contribution, we further discuss this proposal by consolidating the comments received during the previous meeting.

# 2 Discussion

The proposal is to introduce a template-based approach to simplify tables in TS 38.101-1, and the template for the example on NR CA operating bands is [3]:

- Template (CA_nA-nB [DL interruption allowed]) means:

  - an NR CA band combination consisting of band nA and nB,

  - and if the optional part [DL interruption allowed] is present, the DL interruption is not allowed for this band combination, otherwise allowed.

Under such a template, the NR CA operating bands can be simplified as below:

NR CA bands

CA_n1-n3, CA_n1-n5, CA_n1-n7,CA_n1-n8, CA_n1-n18, CA_n1-n20, CA_n1-n26, CA_n1-n28 CA_n1-n38,

CA_n1-n40, CA_n1-n411, CA_n1-n46, CA_n1-n67, CA_n1-n74, CA_n1-n75, CA_n1-n771 No, CA_n1-n781 No

However, during RAN4#110bis, a concern was voiced regarding its potential lack of user-friendliness. Actually, the template approach may have better readability by introducing one rule:

- Each row in the new NR CA band table has an “index” band and includes all band combinations consisting of the index band in a lexicographical order,

- The index bands among rows are also in the lexicographical order

The below illustrates the example of the rule for part of two bands inter-band CA.

The original part of table is:

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n5 | n1, n5 |  |
| CA_n1-n7 | n1, n7 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n18 | n1, n18 |  |
| CA_n1-n20 | n1, n20 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n38 | n1, n38 |  |
| CA_n1-n40 | n1, n40 |  |
| CA_n1-n411 | n1, n41 |  |
| CA_n1-n67 | n1, n67 |  |
| CA_n1-n74 | n1, n74 |  |
| CA_n1-n771 | n1, n77 | No |
| CA_n1-n781 | n1, n78 | No |
| CA_n1-n791 | n1, n79 | No |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n7 | n2, n7 |  |
| CA_n2-n12 | n2, n12 |  |
| CA_n2-n14 | n2, n14 |  |
| CA_n2-n29 | n2, n29 |  |
| CA_n2-n30 | n2, n30 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n2-n78 | n2, n78 |  |
| CA_n3-n5 | n3, n5 |  |
| CA_n3-n7 | n3, n7 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n18 | n3, n18 |  |
| CA_n3-n20 | n3, n20 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n341 | n3, n34 |  |
| CA_n3-n38 | n3, n38 |  |
| CA_n3-n401 | n3, n40 | No |
| CA_n3-n411 | n3, n41 | No |
| CA_n3-n67 | n3, n67 |  |
| CA_n3-n74 | n3, n74 |  |
| CA_n3-n75 | n3, n75 |  |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n3-n791 | n3, n79 | No |

It can be simplified into a table with 3 rows (NR band in red is the index band in each row):

- The first row corresponds to Index band n1, where all band combinations including band n1 are captured in this row.

- The second row corresponds to Index band n2, where all band combinations include band n2 are captured in this row.

- The third row corresponds to Index band n3, where all band combinations include band n3 are captured in this row.

NR CA bands for inter-band CA (two bands)

CA_n1-n3, CA_n1-n5, CA_n1-n7, CA_n1-n8, CA_n1-n18, CA_n1-n20, CA_n1-n26, CA_n1-n28, CA_n1-n38, CA_n1-n40, CA_n1-n411, CA_n1-n46, CA_n1-n67, CA_n1-n74, CA_n1-n75, CA_n1-n771 No, CA_n1-n781 No, CA_n1-n781 No

CA_n2-n5, CA_n2-n7, CA_n2-n12, CA_n2-n14, CA_n2-n29, CA_n2-n30, CA_n2-n48, CA_n2-n66, CA_n2-n77, CA_n2-n78,

CA_n3-n5, CA_n3-n7, CA_n3-n8, CA_n3-n18, CA_n3-n20, CA_n3-n28, CA_n3-n341, CA_n3-n38, CA_n3-n401 No, CA_n3-n411 No, CA_n3-n67, CA_n3-n74, CA_n3-n75, CA_n3-n771 No, CA_n3-n781 No, CA_n3-n791 No

Proposal 1: RAN4 to introduce a template-based approach to improve UE RF specs quality.

Proposal 2: For the NR CA band combination tables, RAN4 to introduce the template “CA_nA-nB [DL interruption allowed]” and simplify the tables with a rule that each row in the new NR CA band table has an “index” band and includes all band combinations consisting of the index band in a lexicographical order, and the index bands among rows are also in the lexicographical order for the sake of readability.

# 3 Conclusion

In this contribution, we have the following proposals on table modifications for UE RF specs improvement:

Proposal 1: RAN4 to introduce a template-based approach to improve UE RF specs quality.

Proposal 2: For the NR CA band combination tables, RAN4 to introduce the template “CA_nA-nB [DL interruption allowed]” and simplify the tables with a rule that each row in the new NR CA band table has an “index” band and includes all band combinations consisting of the index band in a lexicographical order, and the index bands among rows are also in the lexicographical order for the sake of readability.

# Reference

- RP-240782, Moderator's summary on RAN4 spec quality, RAN4 Chair (Huawei)

- R4-2406709, WF on UE RF Spec Improvement, Qualcomm.

- R4-2404448, On UE RF specifications improvement, CATT