3GPP TSG-RAN4 Meeting #111	R4-2407598

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Title:	TP to TR 38.898 Addition of PC2 for DC_11A_n77A

Source:	Samsung, KDDI Corporation

Agenda item:	6.15.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.898 to add PC2 for DC_11A_n77A. DC_11A_n77A PC3 is completed as a fallback.

************************************* Start of TP*****************************************

## 5.x	DC_11_n77

### 5.x.1	Configurations for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

### 5.x.2	Maximum output power for DC

Table 5.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_11A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

- The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of band 11.

- The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 11.

- The 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 11.

- No new cross band isolation interference.

No need to define new MSDs.

### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

************************************* End of TP*****************************************

# Reference

[1] RP-240241 Revised WID: Core part: High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)