3GPP TSG-RAN WG4 Meeting #111                                          R4-2307614

Fukuoka, Japan, May 20 – 24, 2024

Source: 	Huawei, HiSilicon

Title: 	On Rx requirements for sidelink intra-band non-contiguous CA in PC3 and PC2 in Rel-19

Agenda Item:	10.9.2.3

Document for:	Approval

Introduction

The New WID revision [1] was approved in RAN#103 with the scope for sidelink contiguous and non-contiguous CA operation in ITS band.

- Specify the RF requirements for NR sidelink CA in n47: - Intra-band non-contiguous CA with power class 2 and power class 3, with Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz - Intra-band contiguous CA with power class 2 - Introduce necessary changes for release independence specification, if identified Note: Work for power class 3 starts first in RAN4. Whether there is any RRM core requirement to be specified will be confirmed in Q3 2024.

In this contribution, we provide our consideration and initial analysis on RAN4 work on Rx requirements for NR sidelink intra-band non-contiguous CA with power class 3 and power class 2.

Discussion

In Rel-15 LTE V2X, the requirements of intra-band contiguous multi-carrier operation bandwidth class B and class C were defined in addition to the single carrier V2X case. It is noted that it is a full set of Tx and Rx requirements in order to support the multi-carrier operation.

Similarly, in Rel-18 the NR sidelink intra-band contiguous CA operation with PC3 was specified in RAN4, another set of requirements was defined in addition to that of Rel-16/17 V2X/Sidelink.

In the WID for sidelink CA, intra-band non-contiguous CA with PC3 and PC2 is to be considered. The work on Rx requirements in Rel-19 would include at least

- Rx requirements mainly on

  - REFSENS

  - Maximum input level,

  - ACS locking

  - In-band/ out-of-band blocking

  - Spurious response

  - Wide band Intermodulation

The description in the specification should be updated from that only for contiguous intra-band SL CA to also accommodate the requirements for non-contiguous intra-band SL CA.

Proposal 1: The description of the following Rx requirements in the specification should be updated from that only for contiguous intra-band SL CA to also accommodate the requirements for non-contiguous intra-band SL CA

- REFSENS

- Maximum input level,

- ACS locking

- In-band/ out-of-band blocking

- Spurious response

- Wide band Intermodulation

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work on Rx requirements for NR sidelink intra-band non-contiguous CA with power class 3 and power class 2 with the following and proposal:

Proposal 1: The description of the following Rx requirements in the specification should be updated from that only for contiguous intra-band SL CA to also accommodate the requirements for non-contiguous intra-band SL CA

- REFSENS

- Maximum input level,

- ACS locking

- In-band/ out-of-band blocking

- Spurious response

- Wide band Intermodulation

- References

[1] RP-240840 New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band