3GPP TSG-RAN WG4 Meeting # 111	R4-2407692

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item: 4.4

Source: MediaTek Inc.

Title: (NR_RRM_enh-Core) Remaining issues for Rel-16 NeedforGap

Document for: Discussion

# Introduction

In RAN4#110 meeting, the issue on Rel-16 NeedforGap was discussed. An WF [1] was agreed with options listed as below and FFS points highlighted.

Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - [FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE] - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18 Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18

In this paper, we provide our views on the remaining issues.

# Discussion

The first issue is on whether to mandate a Rel-18 UE to support Rel-18 interRAT-NeedForIntrNR-r18 if the UE reports Rel-16 interRAT-NeedForGaps-r16=FALSE. Our preference is No.

The main reason comes from the fact that some legacy (Rel-16 or Rel-17) UEs are already implemented to support interRAT-NeedForGaps-r16=FALSE but with the interruption length and ratio which may not meet the new Rel-18 requirements. To fulfill Rel-18 requirements, some modification efforts at UE side are likely needed. However, UE vendors may decided to upgrade to Rel-18 for other reasons rather than NeedforGap, e.g., for LTM, NES, NCSG, NTN … etc. Mandating the report of interRAT-NeedForIntrNR-r18 will lead to unnecessary burden to UE implementation. Therefore, we prefer to keep interRAT-NeedForIntrNR-r18 optional for Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE. This also aligns with the discussions and conclusions achieved in Issue 2-3-2 in [2].

Proposal 1: Keep interRAT-NeedForIntrNR-r18 optional for Rel-18 UEs if the UE reports interRAT-NeedForGaps-r16=FALSE

The other issue is about what to do if Rel-18 UE does not support interRAT-NeedForIntrNR-r18 in issue 1-1-1 or NeedForInterruptionNR-r18 in issue 1-1-2. In our view, the UE behavior is the same as Rel-16, i.e., whether UE needs interruption or not is undefined.

Proposal 2: For UEs not supporting interRAT-NeedForIntrNR-r18 or NeedForInterruptionNR-r18, whether UE needs interruption or not is undefined

# Summary

In this paper, we provide our views about the remaining issues for Rel-16 NeedforGap. We have the following proposals.

Proposal 1: Keep interRAT-NeedForIntrNR-r18 optional for Rel-18 UEs if the UE reports interRAT-NeedForGaps-r16=FALSE

Proposal 2: For UEs not supporting interRAT-NeedForIntrNR-r18 or NeedForInterruptionNR-r18, whether UE needs interruption or not is undefined

# Reference

[1] R4-2403559, WF on Interruption requirements for Rel-16 measurements without gaps, Nokia, Nokia Shanghai Bell

[2] R4-2406490, WF on RRM requirements for NR_MG_enh2_part2, Intel