3GPP TSG-RAN WG4 Meeting #111	R4-2407743

Fukuoka, Japan, May 20-24, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance aspects on other NES features

Agenda item:	7.24.2.2

Document for:	Discussion

Introduction

At RAN4#110bis meeting, the principle for test cases design were further discussed and the draft BigCR was agreed in [1]. In this paper, we are discussing the test cases on NES-based handover delay.

Discussion

At RAN4#110 meeting, the following test cases were agreed for NES-based CHO. In particular, separate cases were defined based on if DCI 2-9 command is decoded before or after the RSRP condition of CHO is met.

| No. | Feature | Test case | Subclause | Note | Responsibility |
| --- | --- | --- | --- | --- | --- |
| 2-1 | NES-based CHO | NES triggering intra-frequency target CHO delay From FR1 to FR1 | A.6.3.3.X | DCI 2-9 command is decoded before the RSRP condition of CHO is met | CMCC |
| 2-2 |  | NES triggering inter-frequency target CHO delay From FR1 to FR1 | A.6.3.3.X | DCI 2-9 command is decoded after the RSRP condition of CHO is met | Apple |
| 2-3 |  | NES triggering intra-frequency target CHO delay From FR2 to FR2 | A.7.3.3.X | DCI 2-9 command is decoded before the RSRP condition of CHO is met | Vivo |
| 2-4 |  | NES triggering inter-frequency target CHO delay From FR2 to FR1 | A.7.3.3.X | DCI 2-9 command is decoded after the RSRP condition of CHO is met | Intel |

In last meeting, we raised the possibility that CHO condition may not be met any more when receiving DCI 2-9 command. We believe this needs to be considered when defining the NES-based CHO delay and some tentative proposals are made in [2]. Pending on the conclusion, the test cases where DCI 2-9 command is decoded after the RSRP condition of CHO is met needs to be updated considering if CHO condition is met or not when receiving DCI 2-9 command.

Proposal 1:   The test cases where DCI 2-9 command is decoded “after” the RSRP condition of CHO is met needs to be updated considering if CHO condition is met or not when receiving DCI 2-9 command.

Conclusion

In this paper we have made the following proposals and observations related to the RRM performance aspects:

Proposal 1:   The test cases where DCI 2-9 command is decoded “after” the RSRP condition of CHO is met needs to be updated considering if CHO condition is met or not when receiving DCI 2-9 command.

References

[1] R4-2406520, Draft Big CR to TS 38.133 on performance requirements for NR Network energy, Huawei

[2] R4-2407740, core maintenance on network energy saving, Nokia, Nokia Shanghai Bell