3GPP TSG-RAN WG4 Meeting #111	R4-2407748

Fukuoka City, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	On Advanced Receivers - Test parameters - simulations

Agenda item:	7.11.2.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment as well as with impairments on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#110-bis [1] for facilitating companies to provide simulation results.

Simulation Assumptions

Table 1 provides an overview of the already agreed simulation assumptions taken from the WF [1] and proposed test parameters:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for phase II simulation | R-ML, MMSE-IRC |
| --- | --- |
| Sub-carrier spacing, Bandwidth | FDD:15KHz, 10 MHz TDD:30 KHz, 40 MHz(TDD) |
| UL-DL Pattern | FDD: Not applicable, TDD: FR1.30-1 |
| Rank allocation for the target and co-scheduled UE, with 1 co-scheduled UE | 2Tx2Rx UE: 1+1 2Tx4Rx UE: 1+1 4Tx4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for - rank 1 (DMRS port 0) + 1 (DMR port 1) |
| Antenna configuration | For rank 1+1: 2T2R, 2T4R For rank 2+2: 4Tx 4Rx |
| Channel model | TDLC300-100, TDLA30-10 when the rank of the target UE is 1 TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium and ULA low (for 2T2R,2T4R) Rank 2+2: ULA Low, XP medium(4T4R) |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Orthogonal PMI selection For rank 2+2: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | Assume the scrambling ID for DMRS sequence is the same for the target UE and the co-scheduled UE(s) |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration.(10 MHz FDD, 40 MHz TDD) Full OFDM symbol allocation. |
| TDRA of co-scheduled UE | Assume TDRA of target and co-scheduled UE is same |
| FDRA of co-scheduled UE | Same as target UE |

Simulation Results

Alignment Results

Table 2 Alignment results for FDD and TDD cases showing gain of R-ML receiver over MMSE-IRC receiver.

| Case Number (New) | Case Number (110bis) | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | Target UE MCS (Table 1) | MO for the co- UE | DCI Index | R-ML | MMSE-IRC |  | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | FDD 15KHz SCS | 10 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 14.4 |  | 21.4 | 7 |
| 2 | 4 |  |  |  | 2T4R |  |  |  |  |  |  | 12.1 |  | 21.5 | 9.4 |
| 3 | 7 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 17 |  | 18 | 1 |
| 4 | 11 | TDD 30KHz | 40 MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 15.4 |  | 22.6 | 7.2 |
| 5 | 13 |  |  |  | 2T4R |  |  |  |  |  |  | 12.9 |  | 24.3 | 11.3 |
| 6 | 16 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 17.1 |  | 18 | 0.9 |
| 7 | 20 | FDD 15 KHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | Index 6 | 14.6 |  | 21.4 | 6.8 |
| 8 | 23 |  |  |  | 2T4R |  |  |  |  |  |  | 13.6 |  | 21.5 | 7.9 |
| 9 | 26 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 21.9 |  | 26.7 | 4.8 |
| 10 | 29 |  |  |  | 2T4R |  |  |  |  |  |  | 19.8 |  | 25.9 | 6.1 |
| 11 | 20a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 11.6 |  | 14.5 | 2.9 |
| 12 | 23a |  |  |  | 2T4R |  |  |  |  |  |  | 7.2 |  | 8.3 | 1.1 |
| 13 | 31 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  |  |  |  |  |
| 14 | 32 |  |  |  |  |  |  | XP medium |  |  |  |  |  |  |  |
| 15 | 34 | TDD 30KHz SCS | 40 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK |  | 15.5 |  | 22.6 | 7.1 |
| 16 | 37 |  |  |  | 2T4R |  |  |  |  |  |  | 13.3 |  | 24.2 | 10.9 |
| 17 | 40 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 24.1 |  | NA | Inf |
| 18 | 43 |  |  |  | 2T4R |  |  |  |  |  |  | 22.6 |  | 30.5 | 7.9 |
| 19 | 34a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 12.3 |  | 15.6 | 3.2 |
| 20 | 37a |  |  |  | 2T4R |  |  |  |  |  |  | 7.5 |  | 8.9 | 1.4 |
| 21 | 45 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  |  |  |  |  |
| 22 | 46 |  |  |  |  |  |  | XP medium |  |  |  |  |  |  |  |

Cases 9,10 and Cases 17,18with MCS17 and 16QAM scheduled for target and co-UE respectively have high SNR requirement above 20 dB and are seen not feasible for requirement definition.

Cases 11,12 and Cases 19,20 with ULA low antenna correlation and orthogonal precoders for co-UE show less gain over MMSE-IRC receiver as compared to ULA medium correlation.

Impairment Results

Table 3 Results with impairments.

| Case Number (New) | Case Number (110bis) | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | Target UE MCS (Table 1) | MO for the co- UE | DCI Index | R-ML |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | FDD 15KHz SCS | 10 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 16.9 |  |
| 2 | 4 |  |  |  | 2T4R |  |  |  |  |  |  | 14.6 |  |
| 3 | 7 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 19.5 |  |
| 4 | 11 | TDD 30KHz | 40 MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 17.9 |  |
| 5 | 13 |  |  |  | 2T4R |  |  |  |  |  |  | 15.4 |  |
| 6 | 16 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 19.6 |  |
| 7 | 20 | FDD 15 KHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | Index 6 | 17.1 |  |
| 8 | 23 |  |  |  | 2T4R |  |  |  |  |  |  | 16.1 |  |
| 9 | 26 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 24.4 |  |
| 10 | 29 |  |  |  | 2T4R |  |  |  |  |  |  | 22.3 |  |
| 11 | 20a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 14.1 |  |
| 12 | 23a |  |  |  | 2T4R |  |  |  |  |  |  | 9.7 |  |
| 13 | 31 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  |  |  |
| 14 | 32 |  |  |  |  |  |  | XP medium |  |  |  |  |  |
| 15 | 34 | TDD 30KHz SCS | 40 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK |  | 18 |  |
| 16 | 37 |  |  |  | 2T4R |  |  |  |  |  |  | 15.8 |  |
| 17 | 40 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 26.6 |  |
| 18 | 43 |  |  |  | 2T4R |  |  |  |  |  |  | 25.1 |  |
| 19 | 34a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 14.8 |  |
| 20 | 37a |  |  |  | 2T4R |  |  |  |  |  |  | 10 |  |
| 21 | 45 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  |  |  |
| 22 | 46 |  |  |  |  |  |  | XP medium |  |  |  |  |  |

Conclusion

In this contribution we have provided Nokia's simulation results for Advanced receivers. The following Observations were made:

- Cases 9,10 and Cases 17,18with MCS17 and 16QAM scheduled for target and co-UE respectively have high SNR requirement above 20 dB and are seen not feasible for requirement definition.

Cases 11,12 and Cases 19,20 with ULA low antenna correlation and orthogonal precoders for co-UE show less gain over MMSE-IRC receiver as compared to ULA medium correlation.

- References

- R4-2406114	Way Forward for [110bis][324] NR_demod_enh3_Part1