3GPP TSG-RAN WG4 Meeting #111	R4-2407750

Fukuoka City, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	On Lessthan5MHz UE demod perf and CSI requirements

Agenda item:	7.8.7.1

Document for:	Discussion

Introduction

For the topic of Lessthan5Mhz UE demodulation performance and CSI requirements, the main remaining topic is final definition of requirements based on aligned simulation results and clarification on how punctured CCE is treated for punctured PDCCH requirements.

In the following we will provide Nokia’s view on the remaining open issues as well as make observations and proposals where needed.

Discussion

Number of Tx in PDCCH requirements

In RAN4#110bis it was agreed to introduce two testcases for punctured PDCCH. There were however concerns that the provided simulation results were grouped in two distinctive groups. Due to this, it was encouraged for companies to clarify if simulation results provided were using the punctured CCE or not for demodulation (see [1]).

Issue 1-1-1: Number of Tx in PDCCH requirements Agreement: - Define two test cases for punctured PDCCH in less than 5 MHz CBW with 1x2 and 2x4 antenna configurations: Number of Tx; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLA30-10; 1x2 Low; 1; [TBA]. 2; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLC300-100; 2x4 Low; 1; [TBA]. Way forward: - Companies are encouraged to clarify further the requirement value selection and simulation results depending on how punctured CCE is treated.

We see three ways the UE can process the punctured PDCCH for the agreed testcase:

- Demodulation done on 4 CCE without knowledge that half of one CCE does not contain valid data.

- Demodulation done on 4 CCE with knowledge that half of on CCE does not contain valid data.

- Demodulation of only the 3CCE, which are not punctured.

From Nokia’s perspective, we assume that the UE will be utilizing all available information when demodulating the PDCCH. This includes also using the available CCE where only half of the data is valid. Also, we do not expect UEs to be implemented so the demodulation specifically knows that half of the CCE is punctured as we see it as a complex implementation with smaller gain if compared to simply do the demodulation on all 4 CCEs similar as if no puncturing was done.
Hence, Nokia’s results provided in RAN4#110bis has included 4CCE for demodulation where 1 CCE of the 4 is punctured (i.e. half of the CCE does not contain valid data).

We have for reference in this meeting included the case where it is assumed the UE only demodulates the 3 of 4 CCEs, which are not punctured showing a SNR increase @70% TP 0.5dB (see [2]).

From a network perspective, we expect the UE to be able to utilize all provided information bits, hence Nokia’s simulation results provided in RAN4#110bis when demodulating the punctured PDCCH uses all 4 CCE defined in the agreed testcase. We do not expect the UE is aware of which bits are punctured, hence the UE will treat the 4 CCE similar as if none of the CCEs are punctured.

Conclusion

The paper has provided Nokia’s view on the remaining issues for Lessthan 5MHz topic.

The following Observation was made:

- From a network perspective, we expect the UE to be able to utilize all provided information bits, hence Nokia’s simulation results provided in RAN4#110bis when demodulating the punctured PDCCH uses all 4 CCE defined in the agreed testcase. We do not expect the UE is aware of which bits are punctured, hence the UE will treat the 4 CCE similar as if none of the CCEs are punctured.

- References

- R4-2406044	Way Forward for [110bis][323] NR_FR1_lessthan_5MHz_BW_demod

- R4-2407751	On Lessthan5MHz UE demod perf and CSI  req - Sim