# 3GPP TSG-RAN WG4 Meeting #111	R4-2407844

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	10.14.4

Document for:	Discussion

# 1Introduction

In RAN#103 meeting, the latest revised WID on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved [1]. The related RRM work was initially discussed in last meeting. A WF was achieved and approved [2]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

### Sub-topic 2-1 General aspects Issue 2-1-1: Core requirements to be specified for LP-WUR measurement Agreement: - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS FFS on the following topics Issue 2-1-2: Core requirements to be specified for MR RRM relaxation Issue 2-1-3: Criteria (entry/exit conditions) for LP-WUR measurement Issue 2-1-4: Criteria (entry/exit conditions) for MR RRM measurement relaxation Issue 2-1-5: States need be considered Issue 2-1-6: Impact on specification ### Sub-topic 2-2 LP-WUR requirements at RRC_IDLE/INACTIVE state Issue 2-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode Agreement: Agreement on the target SNR for RAN4 simulation: - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study. - Aspects not considered in RAN1 (if any) can be further discussed in RAN4. Issue 2-2-3: Accuracy Agreement: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. For information only: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded. FFS on the following topics Issue 2-2-2: Noise figure and others impact(RF impairment) on SNR target X Issue 2-2-4: Measurement metrics Issue 2-2-5: LP-SS with periodicity Issue 2-2-6: Time/frequency sync Issue 2-2-7: Timeline on RRM requirement evaluation work Issue 2-2-8: Other considerations Issue 2-3-1: MR RRM relaxation factor for serving cell/neighbour cell FFS Issue 2-4-1: Entry/exit condition for LP-WUS monitoring FFS Issue 2-5-1: LP-WUR at CONNECTED mode FFS Issue 2-6-1: eDRX related FFS

- Discussion

Aiming at UE energy efficiency enhancement, a separate receiver besides the main receiver which can monitor wake-up signal with ultra-low power consumption is introduced for LP-WUS/WUR UE. Based on the R18 SI outcome, a new sync signal LP-SS was agreed to be introduced to the LP-WUR besides the wake-up signal, i.e., LP-WUS. Currently, the fundamental design for LP-WUS and LP-SS, e.g., waveform and periodicity, are still on-going in RAN1.

During last meeting discussion, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 1: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

In our understanding, the measurement delay requirements are defined on top of the desired accuracy as well as SINR. Based on measurement accuracy and SINR, the number of the reference signal samples could be decided. Then, the measurement period requirements are derived from the number of reference signal samples within a given time. According to the outcome of last RAN4 meeting, it was agreed to introduce two sets of measurement requirements for LP-WUR serving cell measurement, based on LP-SS and PSS/SSS respectively. From our side, it is preferred to consider the same set of measurement accuracy no matter what RS used for measurement.

Proposal 2: RAN4 to consider a common target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR.

In last meeting, several companies gave opinions on the entry/exist conditions for LP-WUR operation, including LP-WUR measurement and LP-WUS monitoring. As we understand the LP-WUR measurement behavior is introduced to guarantee the performance LP-WUS monitoring, the measurement is not expected to start later than the LP-WUS monitoring. Furthermore, the early launch of LP-WUR measurement could lead to unnecessary power consumption. So, we tend to agree that the entry/exit of LP-WUR measurement should be consistent with LP-WUS monitoring.

Proposal 3: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring.

Similar as the enable/disable condition for UE RSRP measurements in RRC_IDLE mode on intra-frequency cells, we think threshold on serving cell measurement result could be introduced to control the entry/exit for LP-WUR operation.

Proposal 4: Threshold on serving cell measurement result could be introduced to control the entry/exit of LP-WUR operation.

Based on the conclusion reached in study phrase, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbor cells are applied and UE MR enters in ultra-deep sleep during LP-WUS monitoring. To obtain enough power saving gain, one of the objectives in the WI is to “specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR”. In Rel-16 power saving WI, relaxed RRM measurement requirements for neighbor cell in RRC_IDLE/INACTIVE mode are introduced. UE meeting the RRM relaxation criteria is allowed to conduct measurements with longer intervals. Generally, we agree that the Rel-16 relaxation method could be taken as baseline when considering the MR RRM relaxation in LP-WUS.

Currently, RAN2 is working on the related relaxation criteria/condition. So, we prefer that the detailed relaxed RRM relaxation requirements could be discussed when there is progress in RAN2.

Proposal 5: RAN4 to define the relaxed RRM measurement requirements for MR after RAN2 making progress on the relaxation criteria.

With the feature of LP-WUS/WUR, the UE MR could enter ultra-deep sleep state in RRC_IDLE/INACTIVE mode. UE MR is supposed to wake-up based on LP-WUS triggering. In this case, the MR may need to do re-synchronization on time/frequency domain in the first place and then paging reception and RRM measurements. We agree that RAN4 should consider the corresponding requirements for MR to guarantee the paging reception and RRM measurements.

Proposal 6: RAN4 should consider the requirements for UE MR to guarantee the paging reception and RRM measurements.

- Conclusion

Proposal 1: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 2: RAN4 to consider a common target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR.

Proposal 3: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring.

Proposal 4: Threshold on serving cell measurement result could be introduced to control the entry/exit of LP-WUR operation.

Proposal 5: RAN4 to define the relaxed RRM measurement requirements for MR after RAN2 making progress on the relaxation criteria.

Proposal 6: RAN4 should consider the requirements for UE MR to guarantee the paging reception and RRM measurements.

- Reference

- RP-240801	Revised WID: Low-power wake-up signal and receiver for NR (LP-	WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung

- R4-2406367, WF for RRM requirements for LP-WUS/WUR, Vivo