3GPP TSG-RAN WG4 Meeting # 111	R4-2407915

Fukuoka, Japan, May 20 – May 24, 2024

Source:	China Unicom

Title:	TP for TR 38.896 to add PC2 for n5

Agenda item:			6.19.2

Document for:	Approval

1. Introduction

A text proposal for TR 38.896 to add PC2 for n5 including RSD requirements based on  the WF in [1].

2. Text Proposal

---Start of changes---

5.x	NR band n5

5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n5 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.x.2	A-MPR requirements

Further check public safety protection requirements.

5.x.3	Reference sensitivity requirements

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | Skyworks(R4-2300652) | 0.4 | 0.5 | 0.9 | 1.9 | 2.1 |  |  |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.2 | 0.9 | 2.1 | 2.6 | N/A | N/A | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 0.9 | 0.9 | 1.0 | 3.1 | 3.3 | - | - | - | - | - |
|  | Murata(R4-2305393) | 0.7 | 0.6 | 0.4 | 2.9 | 3.0 | - | - | - | - | - |
| Average |  | 0.5 | 0.8 | 0.8 | 2.5 | 2.8 |  |  |  |  |  |

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | Skyworks(R4-2300652) | 1.0 | 1.3 | 1.8 | 5.1 | 5.5 |  |  |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 | 0.5 | 6.2 | 6.8 | N/A | N/A | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 2.0 | 2.0 | 2.0 | 5.9 | 6.2 | - | - | - | - | - |
|  | Murata(R4-2305393) | 1.7 | 1.9 | 2.1 | 5.8 | 6.3 | - | - | - | - | - |
|  | MediaTek(R4-2302353) | 0 | 0 | 0.5 | 5.1 | 5.7 |  |  |  |  |  |
|  | Spreadtrum(R4-2304084) | 1.6 | 1.8 | 2.1 | 6.2 | 6.5 |  |  |  |  |  |
| Average |  | 1.1 | 1.3 | 1.5 | 5.7 | 6.2 |  |  |  |  |  |

---End of changes---

References

- R4-2306544: WF on FDD HPUE Basket WIs; China Unicom 		RAN4#106bis-e